| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G809 |
| --- | --- |

| Title: | Crosscheck of JCTVC-G135 on rectangular (2NxN and Nx2N) intra prediction from MediaTek |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jun Xu 1730 N. First Street San Jose, CA 95112, USA | Tel: Email: | 1-408-352-4086 jun.xu@am.sony.com |
|  | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This report presents the cross verification results of JCTVC-G135. Studying of software shows that a fast mode selection algorithm is not described in JCTVC-G135. Simulation results match with results from proponents of JCTVC-G135. Simulation time from cross-checking results is close to that from proponents’ results.

# Notes

In the software of JCTVC-G135 [1], there is a fast mode decision algorithm under macro MTK_RECT_INTRA_FAST, which is not mentioned in the description of JCTVC-G135. The results provided by JCTVC-G135 are with MTK_RECT_INTRA_FAST on.

# Results

The software from JCTVC-G135 was complied with macro settings as follows.

#define MTK_RECT_INTRA                     1

#define MTK_RECT_INTRA_FAST                1  //default setting

#define MTK_RECT_INTRA_FASTER              0  //faster version

The cross-check results match with results from JCTVC-G135 in BD-rate numbers. The time information is close.

A summary of results is listed in Table 1, where the anchor is HM4.0. Complete results are available in “JCTVC-G809.xls”.

Table 1 Results of cross-check of JCTVC-G135

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -1.6% | -1.9% | -1.4% | -2.3% | -2.6% |
| Class B | -1.5% | -2.4% | -2.5% | -2.6% | -2.9% | -3.3% |
| Class C | -1.8% | -1.5% | -1.7% | -2.6% | -2.8% | -3.1% |
| Class D | -1.8% | -1.2% | -1.2% | -2.4% | -2.4% | -2.5% |
| Class E | -2.5% | -5.0% | -4.5% | -4.1% | -7.0% | -5.7% |
| Overall | -1.6% | -2.2% | -2.3% | -2.5% | -3.3% | -3.3% |
|  | -1.6% | -2.2% | -2.3% | -2.5% | -3.2% | -3.3% |
| Enc Time[%] | 134% |  |  | 143% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |

# Reference

- S. Liu, X. Zhang, Z. Zhou and S. Lei, “Non-CE6: Rectangular (2NxN and Nx2N) Intra Prediction”, JCTVC-G135, Geneva, November 2011.