| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G810 |
| --- | --- |

| Title: | Crosscheck of JCTVC-G323 on sub-block scan for residual coding from Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution cross-checked the BDR performance of  the two 4x4 sub-block scan methods with CABAC in JCTVC-G323 for Class A, B,  C, D and E test sequences. The cross-checked results matched the results in JCTVC-G323. One method is for higher coding efficiency. It resulted in BDR of -0.02%, 0.03%, 0.03% for AI_HE, RA_HE, and LB_HE, respectively. The other method is for processing of 2 in parallel. It resulted in BDR of 0.02%, 0.06%, 0.05% for AI_HE, RA_HE, and LB_HE respectively. This contribution did not crosscheck the performance of the sub-block size 8x8 and 16x16, and it did not cross-check the performance of Class F test sequences as in JCTVC-G323.

# Introduction

JCTVC-G323 [1] proposed to use 4x4, 8x8, or 16x16 sub-block scan pattern for encoding transform coefficients to address the hardware complexity concern of  large scan pattern  in JCTVC-F597[2]. As presented in JCTVC-F597, 4x4 sub-block scan pattern resulted in significance reduction in complexity for the implementation in JCVC-F597.  For complexity reduction, this contribution only cross-checked the BDR performance of 4x4 sub-blocks.

For 4x4 sub-blocks, JCTVC-G323 implemented two methods to determine the context of 16x16 and 32x32 significance map with CABAC.  One method is for higher coding efficiency. It resulted in BDR of -0.02%, 0.03% , 0.03% for AI_HE, RA_HE, and LB_HE respectively.

The other method is for better parallel processing. In particular, it supports the determination of the contexts of two transform coefficients of 16x16 and 32x32 significance map in parallel with CABAC. It resulted in BDR of 0.02%, 0.06%, 0.05% for AI_HE, RA_HE, and LB_HE respectively.

# Simulation Results

The simulations were performed with the bug fixed version of the software provided by Qualcomm for JCTVC-G323 [1] in a Microsoft HPC cluster where the common test conditions and reference configurations specified by the Software AhG are followed:

- All intra simulations are performed on AMD Opteron Processor 6136 cluster @ 2.4GHz.

- All RA simulations are performed on Intel Xeon X5690 cluster @ 3.47GHz.

- All LD simulations are performed on Intel Xeon X5680 cluster @ 3.33GHz.

The BD-Rate results matched the results in JCTVC-G323[1] provided by Qualcomm.

## BDR results of 4x4 scan without parallel processing support

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.07% | -0.31% | -0.11% |
| Class B | 0.03% | 0.02% | 0.04% |
| Class C | -0.02% | 0.00% | 0.00% |
| Class D | -0.01% | -0.04% | -0.07% |
| Class E | -0.06% | -0.15% | -0.17% |
| Overall | -0.02% | -0.09% | -0.05% |
|  | -0.02% | -0.09% | -0.08% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.09% | 0.29% | 0.47% |
| Class B | 0.04% | -0.03% | 0.07% |
| Class C | -0.04% | -0.11% | 0.09% |
| Class D | 0.00% | 0.02% | -0.32% |
| Class E |  |  |  |
| Overall | 0.03% | 0.04% | 0.08% |
|  | 0.02% | 0.02% | 0.05% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.06% | -0.05% | -0.11% |
| Class C | 0.01% | -0.05% | 0.05% |
| Class D | 0.00% | 0.07% | -0.32% |
| Class E | 0.07% | 0.04% | 0.40% |
| Overall | 0.03% | 0.00% | -0.03% |
|  | 0.03% | -0.02% | -0.07% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 101% |  |  |

## BDR results of 4x4 scan with 2 parallel processing support

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.01% | -0.24% | -0.01% |
| Class B | 0.08% | 0.11% | 0.11% |
| Class C | 0.01% | 0.01% | 0.03% |
| Class D | 0.00% | 0.02% | 0.02% |
| Class E | 0.00% | -0.09% | -0.06% |
| Overall | 0.02% | -0.03% | 0.03% |
|  | 0.02% | -0.03% | 0.01% |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | 0.13% | 0.14% | 0.05% |
| Class B | 0.07% | 0.00% | 0.12% |
| Class C | -0.01% | -0.02% | -0.02% |
| Class D | 0.06% | -0.04% | -0.14% |
| Class E |  |  |  |
| Overall | 0.06% | 0.02% | 0.01% |
|  | 0.06% | 0.01% | 0.03% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.06% | 0.11% | -0.32% |
| Class C | 0.06% | 0.11% | 0.01% |
| Class D | -0.03% | -0.40% | -0.23% |
| Class E | 0.13% | -0.05% | 0.61% |
| Overall | 0.05% | -0.05% | -0.04% |
|  | 0.05% | -0.08% | -0.07% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

# References

- Joel Sole, Rajan Joshi, Marta Karczewicz, “Non-CE11: Diagonal sub-block scan for HE residual coding”, JCTVC-G323_r1, 7th Meeting: Geneva, CH, 21-30 November, 2011.

- C. Auyeung and T. Suzuki, “CE11: Hardware complexity of large zig-zag scan for level-coding of transform coefficients”, JCTVC-F597, 6th JCT-VC Meeting, Torino, IT, July 2011.