| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G811 |
| --- | --- |

| Title: | Crosscheck of JCTVC-G497 on SIMD optimization of proposed HEVC core transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Cheung Auyeung 1730 N. First Street, MD:3NW San Jose, CA 95112 USA | Tel: Email: | 1-408-352-4725 cheung.auyeung@am.sony.com |
| Source: | Sony Electronics Inc. |  |  |

_____________________________

# Abstract

The estimated cycle count of the core transforms implemented with SIMD instructions are approximately from -20% to +2%  of the cycle count provided in JCTVC-G497_r2.  The differences in cycle count are in part due to differences in the micro architecture of the INTEL CPU used in the crosscheck and JCTVC-G497_r2.  The differences are also due in part that turbo-boost was used in the crosscheck and not used in JCTVC-G497_r2.

# Introduction

This contribution cross-checked the estimate cycle count of the core transforms based on the software provided by the proponents of JCTVC-G497 [1]. The core transforms in the software are implemented with Intel SIMD instructions by means of intrinsic functions in C.

# Simulation Results

The software was compiled with Intel Compiler Ver 11 with Microsoft Visual Studio Professional 2010. The simulations were performed on a Windows 7 64 bit PC with Intel Core-i7 975 CPU at 3.33 GHz and turbo-boost.

The estimated cycle counts are approximately from -20% to +2% of the cycle counts provided in JCTVC-G497_r2 [1].  The differences in the cycle counts are in part due to differences in the Intel Nehalem micro architecture of the CPU used in the crosscheck and the Intel Clarkdale micro architecture of the CPU used in JCTVC-G497_r2.  The differences are also due in part that the turbo-boost was used in the crosscheck but not used in JCTVC-G497_r2.

Table 1: Estimated cycle count for partial 2D inverse transforms of various size, 
Intel i7 (Nehalem), Windows 7, 64-bit, with turbo-boost.

| NxN | KxK | Estimated cycle count - Column-row order | Estimated cycle count - Row-column order |
| --- | --- | --- | --- |
| 8x8 | 8x8 | 152 | 142 |
| 8x8 | 4x4 | 87 | 75 |
| 16x16 | 16x16 | 948 | 830 |
| 16x16 | 8x8 | 541 | 430 |
| 16x16 | 4x4 | 343 | 259 |
| 32x32 | 32x32 | 6181 | 5368 |
| 32x32 | 16x16 | 3097 | 2614 |
| 32x32 | 8x8 | 1925 | 1473 |
| 32x32 | 4x4 | 1396 | 1006 |

# References

- A. Fuldseth, Lars Petter Endres, Stian Selnes, Volodymyr Arbatov, Markus Püschel, and Franz Franchetti,  “SIMD optimization of proposed HEVC core transforms”, JCTVC-G497, 7th Meeting: Geneva, CH, 21-30 November, 2011.