| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 Nov, 2011 | Document: JCTVC-G812 WG11 Number: |
| --- | --- |

| Title: | AHG16: Padding process simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Dr. San Diego, CA 92121, USA | Tel: 1-858-651-5135 Email: xianglin@qualcomm.com wchien@qualcomm.com martak@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution provides a single-pass padding scheme for generating reference samples used for intra prediction. With the proposed scheme, memory access is more regular and consistent than the current one in HM4.0. Simulation results show that with constrained intra prediction enabled and 1500 bytes/slice, its impact on coding performance with HE configurations is in average 0.00% for all intra, 0.04% for random access and 0.01% for low delay. For LC configurations, it is in average 0.00% for all intra, 0.06% for random access and 0.01% for low delay.

# Introduction

Currently in HM4.0, padding is performed in a two-pass manner. As shown in Fig 1, along the shaded area the checking starts from point A and ends at point B. Every border pixel is checked whether it is available for use of prediction of a current block.

Fig. 1. Padding process in HM4.0

In case a current border pixel Pcur is not available, a next available border pixel, Pnext , is to be searched. Assume the border pixel immediately before Pcur along the checking direction is Ppre, as shown in Fig. 1. All border pixels between Ppre and Pnext , including Pcur, are assigned a value of:

Ppad =  (Ppre + Pnext) / 2

In case that Pcur is a first pixel starting from A wherein Ppre does not exist, all border pixels from Pcur to Pnext are assigned a value of:

Ppad =  Pnext

In case that Ppre exists but Pnext does not exist, i.e. no border pixels available from Pcur to the end point B in the shaded area, all border pixels from Pcur to the end point B are assigned a value of :

Ppad =  Ppre

In case that no border pixels are available, a default pixel value, is assigned to every border pixel for prediction.

Such a padding process requires a “jumpy” style memory access. Whenever a border pixel is not available, a next available border pixel needs to be searched before the current border pixel can be padded.

# Proposed Method

Based on this proposal, a border pixel is first identified to be a starting point for padding. Then padding is performed along two directions separately. If a current border pixel is not available, it simply copies the value of the border pixel that is preceding it based on padding order.

Fig. 2. Proposed padding process

This is shown in Fig. 2. Firstly pixel PL is selected as a start point if it is available. If it is not available, pixel PA is used as a starting point. In case PA is not available, it is assigned a default value e.g. (1 << ( BitDepthY -1)).

Padding is performanced from the start point towards to both A and B. Assume PA is selected as the starting point. Then padding is performed pixel by pixel from PA to the end point A and B respectively.

With the proposed scheme, if a current border pixel is not available, there is no need to look ahead searching for the next available border pixel. As a result, memory access is improved.

For comparison, a more conservative padding scheme could be searching for an available border pixel only for the start point. More specifically, as defined in current HM 4.0, padding process starts from point A shown in Fig. 2. If border pixel at A is available, padding is performed the same way as described above, i.e. padding an unavailable pixel by copying the value of the border pixel that is preceding it based on padding order. If border pixel A is unavailable, padding is done as currently defined in HM4.0, i.e. searching for a next available pixel and use its value for padding. Essentially, the only difference between this scheme and the current HM4.0 is to replace the averaging operation in padding an unavailable pixel with an operation of copying the previous pixel based on padding order. Simulation results show that such a scheme has virtually no loss in coding performance.

# Simulation Results

Simulations are performed using common test conditions[1]. Additionally, to make the padding situation more likely to occur, maximum number of bytes per slice is set to 1500, with constrained intra prediction enabled. Detailed results can be found in the associated excel data sheet. Excel data sheets JCTVC-G812_M1 and JCTVC-G812_M2 provide results respectively for the proposed first and second scheme,

With the proposed first scheme where at most two locations are checked as a starting point, there is almost no performance difference with all intra configuration. With HE configurations, the coding performance impact is in average 0.00% for all intra, 0.04% for random access and 0.01% for low delay. With LC configurations, it is in average 0.00% for all intra, 0.06% for random access and 0.01% for low delay.

With the more conservative scheme in which only the averaging operation of HM4.0 is replaced with copying the previous pixel, simulation results are also quite close to original ones. With HE configurations, the coding performance impact is in average 0.00% for all intra, 0.01% for random access and 0.02% for low delay. With LC configurations, it is in average 0.00% for all intra, 0.00% for random access and 0.02% for low delay.

# Conclusions

With no or significantly reduced search needed for an available border pixel in padding a current unavailable pixel, the proposed padding scheme improves memory access compared with the current one in HM4.0. Coding efficiency impact is very low. We suggest one of the two simplified padding schemes be adopted into HM.

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, July 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# WD text change

## WD text change based on proposed first scheme

- 8.3.3.1.1		Reference sample substitution process for intra sample prediction

Inputs to this process are the reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

Outputs of this process are the modified reference samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 for intra sample prediction.

The values of the samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are modified as follows:

- If all samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “not available for intra prediction,” the value ( 1 << ( BitDepthY - 1 ) ) is substituted for the values of all samples p[ x, y ].

- Otherwise (at least one but not all samples p[ x, y ] are marked as “not available for intra prediction”), the following steps are performed:

- If p[ -1, nS-1 ] is available, set x0 as -1 and y0 as (nS-1).

- Otherwise, set x0 as (nS-1) and y0 as -1. If p[nS-1, -1] is marked as “not available for intra prediction”, the value (1<<(BitDepthY-1)) is substituted for the value of sample p[nS-1,-1].

- If x0 is equal to -1,

  - For x=-1, y=nS…nS*2-1, if p[x,y] is marked as “not available for intra prediction”, the value of p[x,y-1] is substituted for the value of p[x,y]

  - For x=-1, y=nS-2…-1, if p[x,y] is marked as “not available for intra prediction”, the value of p[x,y+1] is substituted for the value of p[x,y]

  - For x=0…nS*2-1, y=-1, if p[x,y] is marked as “not available for intra prediction”, the value of p[x-1,y] is substituted for the value of p[x,y]

- Otherwise,

  - For x=nS-2…-1, y=-1, if p[x,y] is marked as “not available for intra prediction”, the value of p[x+1,y] is substituted for the value of p[x,y].

  - For x=-1, y=0…2*nS-1, if p[x,y] is marked as “not available for intra prediction”, the value of p[x,y-1] is substituted for the value of p[x,y].

  - For x=nS…nS*2-1, y=-1, if p[x,y] is marked as “not available for intra prediction”, the value of p[x-1,y] is substituted for the value of p[x,y]

- Otherwise (at least one but not all samples p[ x, y ] are marked as “not available for intra prediction”), the following steps are performed for each sample p[ x0, y0 ] with x0 = -1, y0 = -1..nS*2-1 and x0 = 0..nS*2-1, y0 = -1 marked as "not available for intra prediction":

- Let the variables q and r be initially set to -1.

- The variable q is modified as follows:

  - If x0 is equal to -1, a reference sample p[ x, y ] marked as “available for intra prediction” is identified by searching sequentially among p[ x, y ] starting from x = -1, y = y0+1 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q. [Ed.: (WJ) is there any elegant way rather than searching?]

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0-1, y = -1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = x0-1..-1, y = -1, the search is continued among p[ x, y ] sequentially starting from x = -1, y = 0 to x = -1, y = nS*2-1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to q. [Ed.: (WJ) is there any elegant way rather than searching?]

- The variable r is modified as follows:

  - If x0 is equal to -1, a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = -1, y = y0-1 to x = -1, y = -1. When a sample marked as “available for intra prediction” is not found among p[ x, y ] with x = -1, y = y0-1..-1, the search is continued among p[ x, y ] sequentially starting from x = 0, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r. [Ed.: (WJ) is there any elegant way rather than searching?]

  - Otherwise (x0 is not equal to -1), a sample marked as “available for intra prediction” is identified among the samples p[ x, y ] by searching sequentially starting from x = x0+1, y = -1 to x = nS*2-1, y = -1. As soon as a sample marked as “available for intra prediction” is found, the search is terminated and the value of p[ x, y ] is assigned to r. [Ed.: (WJ) is there any elegant way rather than searching?]

- The value of the sample p[ x0, y0 ] is modified as follows:

  - If q is not equal to -1 and r is not equal to -1, the value ( ( q + r + 1 ) >> 1 ) is substituted for the value of p[ x0, y0 ].

  - Otherwise, if q is not equal to -1, the value of q is substituted for the value of p[ x0, y0 ].

  - Otherwise (q is equal to -1 and r is not equal to -1), the value of r is substituted for the value of p[ x0, y0 ].

All samples p[ x, y ] with x = -1, y = -1..nS*2-1 and x = 0..nS*2-1, y = -1 are marked as “available for intra prediction.”

## WD text change based on proposed second scheme

…

- Otherwise (at least one but not all samples p[ x, y ] are marked as “not available for intra prediction”), the following steps are performed:

- If p[ -1, nS*2-1 ] is not available, searching sequentially starting from x = -1, y = nS*2-1 to x = -1, y = -1, then from x=0, y=-1 to x=nS*2-1, y=-1. As soon as a sample p[x,y] marked as “available for intra prediction” is found, the search is terminated and the value of p[x,y] is assigned to p[-1, nS*2-1 ].

- For x=-1, y=nS*2-2…-1, if p[x,y] is marked as “not available for intra prediction”, the value of p[x,y+1] is substituted for the value of p[x,y]

- For x=0…nS*2-1, y=-1, if p[x,y] is marked as “not available for intra prediction”, the value of p[x-1,y] is substituted for the value of p[x,y]

- Otherwise (at least one but not all samples p[ x, y ] are marked as “not available for intra prediction”), the following steps are performed for each sample p[ x0, y0 ] with x0 = -1, y0 = -1..nS*2-1 and x0 = 0..nS*2-1, y0 = -1 marked as "not available for intra prediction":

…