| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G813 |
| --- | --- |

| Title: | ALF with single filter type |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Madhukar Budagavi Texas Instruments Inc., USA | Tel: Email: | +1-214-480-1861 madhukar@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

In HM-4.0, two types of ALF filters are used – cross and star. This contribution presents BD-Rate results when only one of the two filters is used. It is asserted that using only cross results in a BD-Rate loss of: AI-HE: 0.08%, RA-HE: 0.32%, LB-HE: 0.40%; using only star results in a BD-Rate loss of: AI-HE: 0.16%, RA-HE: 0.41%, LB-HE: 0.58%. It is asserted that from an encoder complexity reduction perspective a single filter type that can provide most of the gains of two filter types is desirable since it reduces the number of encoder passes. A single filter type of cross filter with rectangular center was reported in JCTVC-G130 to capture most of the gains of chroma ALF with two filter types. This contribution proposes that a similar filter be evaluated for luma.

# ALF with single filter type

In HM-4.0, two types of ALF filters are used – cross and star. From an encoder complexity reduction perspective a single filter type that can provide most of the gains of two filters types is desirable since it reduces the number of encoder passes.

HM-4.0 was modified to use only one of the two HM-4.0 filter types: cross and star. Table below lists the BD-Rate results.

|  | AI-HE | RA-HE | LB-HE |
| --- | --- | --- | --- |
| No ALF | 1.94% | 3.68% | 3.11% |
| Star only | 0.16% | 0.41% | 0.58% |
| Cross only | 0.08% | 0.32% | 0.40% |

# Conclusions

It is recommended that ALF with single filter type be studied in CE.

# Patent rights declaration(s)

Texas Instruments Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).