| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G814_r1 WG11 Number: m22386 |
| --- | --- |

| Title: | CE7: Cross verification of JCTVC-G108, On secondary transforms for intra prediction residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of the 6x6 and 8x8 cases of the secondary transform described in JCTVC-G108, “On secondary transforms for intra prediction residual.” This activity is part of Core Experiment 7 on Additional Transforms. A brief analysis of the software implementation is followed by a summary of experimental results. The simulation results performed for the cross-verification match those provided by the proponents, and examination of the software confirms that it is implementing the proposed secondary transform.

# Introduction

For 4x4 intra blocks, the current Working Draft (WD4) [1] applies the 4x4 mode-dependent DCT/DST. For 8x8 and larger blocks, a core DCT-like transform is applied. In JCTVC-G108 [2], a proposed secondary transforms may be applied to the low-frequency coefficients of 8x8 and larger blocks, depending upon the intra prediction mode. This work was performed as part of Core Experiment 7 on Additional Transforms [3]. Secondary transform sizes of 2x2 through 8x8 were evaluated. For the cross-verification described here, secondary transform sizes of 6x6 and 8x8 were evaluated.

# Secondary transforms

The 8-point secondary transform is defined using the following coefficients:

{   123,   -35,    -8,    -3,    -2,    -1,    -1,    -1}

{    32,   120,   -29,   -10,    -5,    -3,    -2,    -1}

{   -14,   -24,  -123,    21,     8,     4,     3,     2}

{     7,    11,    17,   125,   -16,    -7,    -4,    -2}

{    -4,    -7,    -8,   -13,  -126,    13,     5,     3}

{     3,     4,     5,     6,    11,   127,   -10,    -5}

{    -2,    -3,    -3,    -4,    -5,    -9,  -127,     9}

{     2,     2,     3,     3,     3,     5,     8,   128}

The 6-point secondary transform is defined using these coefficients:

{   123,   -35,    -8,    -3,    -2,    -1,}

{    32,   120,   -29,   -10,    -5,    -3,}

{   -14,   -24,  -123,    21,     8,     4,}

{     7,    11,    17,   125,   -16,    -7,}

{     5,     7,     8,    14,   126,   -13,}

{     3,     5,     5,     7,    12,   127,}

In the current software implementation, the secondary transform size is selected during compile time. For an N-point secondary transform size, the transform is applied to the rows and/or columns of the NxN low-frequency transform coefficients output by the HM core transform. The application of the secondary transform is determined by Table 8-14, “Specification of horizTrType and vertTrType,” in WD4_d5. Hence, the table used for secondary transforms after 8x8 and larger blocks (through 32x32) in the proposed method is identical to that used for the 4x4 DCT/DST in WD4.

# Performance of proposed method

The proposed method from JCTVC-G108 was implemented on top of HM 4.0. For this cross verification, simulations using software provided by the proponents was and compared to a reference coded with HM 4.0, using the common conditions described in [4]. The performance is summarized in Table 1 for the 6x6 secondary transform, and in for the 8x8 secondary transform. Full results are included in the accompanying spreadsheet. For the 6x6 secondary transform, BD-Rate metrics average -0.7% for the AI-HE configurations and -0.6% for AI-LC configurations. For RA configurations, the BD-Rate change averages to -0.3%. Results for the 8x8 secondary transform are similar, except that the AI-LC configuration has a BD-Rate change of -0.7%. All the results match those provided by the proponents. Encoder run times for the proposed method are up to a few percent slower than the reference, which is consistent with the run-times from JCTVC-G108. Because the decoder takes so little time to run on modern compute platforms, the variations shown here are not unexpected.

Table 1. Performance using 6x6 secondary transform

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.5% | -0.5% | -0.7% | -0.6% | -0.9% |
| Class B | -0.8% | -0.6% | -0.5% | -0.6% | -0.8% | -0.9% |
| Class C | -0.4% | -0.5% | -0.5% | -0.5% | -0.6% | -0.6% |
| Class D | -0.3% | -0.4% | -0.4% | -0.4% | -0.3% | -0.4% |
| Class E | -1.1% | -0.6% | -0.6% | -0.9% | -1.0% | -1.1% |
| Overall | -0.7% | -0.5% | -0.5% | -0.6% | -0.7% | -0.8% |
|  | -0.7% | -0.5% | -0.5% | -0.6% | -0.7% | -0.8% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | 0.2% | -0.2% | -0.3% | 0.0% | 0.0% |
| Class B | -0.4% | -0.3% | -0.2% | -0.4% | -0.5% | -0.4% |
| Class C | -0.2% | -0.4% | -0.4% | -0.3% | -0.5% | -0.2% |
| Class D | -0.1% | -0.1% | -0.4% | -0.2% | -0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.1% | -0.3% | -0.3% | -0.3% | -0.2% |
|  | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

Table 2. Performance using 8x8 secondary transform

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -0.6% | -0.7% | -0.8% | -0.7% | -1.1% |
| Class B | -0.8% | -0.8% | -0.7% | -0.6% | -1.0% | -1.0% |
| Class C | -0.5% | -0.5% | -0.5% | -0.6% | -0.7% | -0.7% |
| Class D | -0.3% | -0.4% | -0.4% | -0.4% | -0.4% | -0.4% |
| Class E | -1.1% | -0.8% | -0.9% | -0.9% | -1.1% | -1.2% |
| Overall | -0.7% | -0.6% | -0.6% | -0.7% | -0.8% | -0.9% |
|  | -0.7% | -0.6% | -0.7% | -0.7% | -0.8% | -0.9% |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 104% |  |  | 104% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.4% | -0.2% | -0.9% | -0.4% | -0.1% | -0.3% |
| Class B | -0.4% | -0.3% | -0.3% | -0.4% | -0.5% | -0.5% |
| Class C | -0.3% | -0.6% | -0.2% | -0.3% | -0.4% | -0.2% |
| Class D | -0.2% | 0.0% | -0.6% | -0.2% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.3% | -0.5% | -0.3% | -0.3% | -0.3% |
|  | -0.3% | -0.3% | -0.5% | -0.3% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |

# Conclusions

The results obtained during this cross verification match those provided by the proponents. Examination of the software confirms that it is properly implementing the described secondary transforms. BD-Rate improvements averaging close to 0.7% were obtained with increases of only a percent or two in encoder run-time. Decoder run-time differences are negligible due to short decode times on modern compute platforms.

# References

- B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F803_d5, Torino, IT, July 2011.

- A. Saxena, Y. Shibahara, F. Fernandes, and T. Nishi, “CE 7 : On secondary transforms for intra prediction residual,”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G108, Geneva, CH, November 2011

- R. Cohen, C. Yeo, R. Joshi, F. Fernandes, “Description of Core Experiment 7: Additional Transforms”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F907, Torino, IT, July 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, Torino, IT, July 2011