| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G817_r1 WG11 Number: m22389 |
| --- | --- |

| Title: | Non-CE7: Cross verification of JCTVC-G345, Restricted mode-dependent 8x8 DST for Intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Robert Cohen Mitsubishi Electric Research Laboratories 201 Broadway Cambridge, MA 02139 U.S.A | Tel: Email: | +1-617-621-7503 cohen@merl.com |
| Source: | Mitsubishi Electric Research Laboratories |  |  |

_____________________________

# Abstract

This document reports the outcome of Mitsubishi Electric Research Laboratory’s (MERL) cross-verification of JCTVC-G345, “Non-CE7: Cross verification of JCTVC-G345, Restricted mode-dependent 8x8 DST for Intra prediction.” A brief analysis of the software implementation is followed by a summary of experimental results. The simulation results performed for the cross-verification match those provided by the proponents, and examination of the software confirms that it is implementing an 8x8 or 4x4 DCT/DST depending upon the CU, PU, and TU. Two cases were tested: 8x8 and 4x4 DCT/DST is applied only when the PU size and TU size are identical, and similarly when the CU, PU, TU sizes are identical.

# Introduction

For 4x4 intra blocks, the current Working Draft (WD4) [1] applies the 4x4 mode-dependent DCT/DST. A similar concept was used for 8x8 transforms in JCTVC-F282 [2]. The 8x8 mode-dependent DCT/DST used the same table as 4x4 transforms to determine which transform to apply based upon the intra prediction mode. This table was used for all times an 8x8 (and 4x4) transform was applied. The purpose of JCTVC-G345 [3] is to see if JCTVC-F282 can be improved upon, both in terms of performance and run-time, by limiting the conditions when the DST is applied.

# Conditions tested and examination of software

Two cases were tested. Method 1 applies the mode-dependent DCT/DST for 4x4 and 8x8 TUs only when the TU size is equal to the PU size. Method 2 uses an addition constraint in that to apply the 8x8 mode-dependent DCT/DST, the CU must also have the same size. When these conditions aren’t met, the DCT core transform is applied.

## Method 1: PU and TU sizes are identical

The default condition in the JCTVC-G345 software is to apply a mode-dependent DCT/DST for both 8x8 and 4x4 TUs. This condition is limited by the following statement prior to calling the transform or inverse transform:

UInt uiDir =

( (pcCU->getPartitionSize(uiAbsPartIdx) == SIZE_2Nx2N && uiTrDepth != 0) ||

(pcCU->getPartitionSize(uiAbsPartIdx) == SIZE_NxN   && uiTrDepth != 1)

) ? REG_DCT : pcCU->getLumaIntraDir( uiAbsPartIdx ) ;

This statement has the effect of forcing the transform type to be REG_DCT, i.e. the DCT, when the TU size is not the same as the PU size.

## Method 2: CU, PU and TU sizes are identical

Here, the condition created before calling the transform or inverse transform is:

UInt uiDir = ( uiTrDepth != 0 && uiWidth == 8 ) ?

REG_DCT : pcCU->getLumaIntraDir( uiAbsPartIdx );

Thus, for 8x8 TUs, the mode-dependent DCT/DST is used only when the transform tree has a depth of zero. 4x4 TUs operate in the same manner as described in WD4.

# Performance of proposed methods

The proposed methods from JCTVC-G345 were implemented on top of HM 4.0. For this cross verification, simulations using software provided by the proponents were compared to a reference coded with HM 4.0, using the common conditions described in [4].

## Method1: PU and TU sizes are identical

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | 0.2% | 0.2% | -0.3% | -0.4% | -0.6% |
| Class B | -0.3% | 0.3% | 0.3% | -0.1% | -0.4% | -0.4% |
| Class C | -0.2% | 0.0% | 0.1% | -0.3% | -0.3% | -0.3% |
| Class D | -0.1% | -0.1% | -0.1% | -0.3% | -0.2% | -0.2% |
| Class E | -0.4% | 1.1% | 0.8% | -0.3% | -0.2% | -0.3% |
| Overall | -0.3% | 0.3% | 0.2% | -0.2% | -0.3% | -0.4% |
|  | -0.3% | 0.2% | 0.2% | -0.3% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.3% | 0.4% | -0.1% | -0.5% | -0.6% |
| Class B | -0.1% | 0.3% | 0.2% | -0.1% | -0.2% | -0.3% |
| Class C | -0.1% | 0.0% | 0.1% | -0.2% | -0.5% | -0.1% |
| Class D | -0.1% | 0.1% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.2% | 0.1% | -0.1% | -0.3% | -0.3% |
|  | -0.1% | 0.1% | 0.1% | -0.1% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

## Method2: CU, PU and TU sizes are identical

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | 0.1% | 0.0% | -0.4% | -0.4% | -0.7% |
| Class B | -0.2% | 0.0% | 0.0% | -0.2% | -0.5% | -0.5% |
| Class C | -0.1% | -0.2% | -0.2% | -0.3% | -0.5% | -0.5% |
| Class D | -0.1% | -0.3% | -0.3% | -0.2% | -0.3% | -0.3% |
| Class E | -0.4% | 0.7% | 0.4% | -0.5% | -0.3% | -0.4% |
| Overall | -0.3% | 0.0% | 0.0% | -0.3% | -0.4% | -0.5% |
|  | -0.3% | 0.0% | -0.1% | -0.3% | -0.4% | -0.5% |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.1% | -0.5% | -0.1% | -0.2% | -0.6% |
| Class B | -0.2% | 0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class C | -0.1% | -0.3% | -0.1% | -0.2% | -0.4% | -0.3% |
| Class D | -0.1% | 0.1% | -0.4% | -0.1% | -0.3% | -0.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.0% | -0.2% | -0.1% | -0.3% | -0.3% |
|  | -0.1% | 0.0% | -0.3% | -0.1% | -0.3% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 100% |  |  |

# Conclusions

The results obtained during this cross verification match those provided by the proponents. Examination of the software confirms that it is properly implementing the described 8x8 mode-dependent transforms along with the restrictions on when they are applied. Average BD-Rate gains are about 0.2% to 0.3% for all-intra configurations. Method 2 (CU=PU=TU) produces about 0.1% more gain over Method 1 (PU=TU) for the AI-LC configuration. Run times for the proposed tool are on par with the HM 4.0 reference, allowing for expected variations due to the compute platform.

# References

- B. Bross, et al., “WD4: Working Draft 4 of High-Efficiency Video Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F803_d5, Torino, IT, July 2011.

- A. Saxena, F. Fernandes, “CE 7: Mode-Dependent 8x8 DCT/DST for Intra Prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F282, Torino, IT, July 2011

- A. Ichigaya, Y. Sugito, S. Sakaida, “Non-CE7: Restricted mode-dependent 8x8 DST for Intra prediction,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-G345, Geneva, CH, November 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, Torino, IT, July 2011