| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G818 |
| --- | --- |

| Title: | Cross-check for Offset coding in SAO from MediaTek and Qualcomm (JCTVC-609) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Elena Alshina JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.net jeonghoon@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

# Abstract

This is cross-check report for modified off-set coding method in SAO proposed by Qualcomm and MediaTek. In average across 6 test cases in common test condition: IA-HE/LC, RA-HE/LC, LD(B)-HE/LC tested modification provides 0.0% (Y), -0.2% (U), -0.2% (V) BD-rate change. Performance matches with reported by proponent.

# Introduction

To improve the offset coding, proposed design has two parts. First, proponents design a proper codeword to represent offset value according to its probability. Second, proponents use a simple prediction technique to reduce the offset magnitude.

Implementation matches with description provided in [1].

# Test results

This test was run under common test conditions [2] for : IA-HE/LC, RA-HE/LC, LD(B)-HE/LC cases and optional test for LD(P)_HE/LC also was done.

Summary of these tests is presented in Table 1. More details can be found and spread-sheets attached to this report. The target of this proposal is to reduce SAO overhead information. Proposed modification mainly improves performance in terms of Chroma BD-rate. For Luma performance improvement is noticeable for small resolution classes (C and D). We have found fluctuation of encoding or decoding time within 2% compare to anchor (HM4.0)

Table1. Test results summary.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class B | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class E | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.1% |
| Overall | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.5% | 0.0% | 0.0% | -0.2% |
| Class B | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% | -0.1% |
| Class C | -0.1% | -0.2% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class D | 0.0% | -0.1% | -0.2% | 0.0% | -0.2% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% |
|  | 0.0% | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.3% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | -0.3% | 0.0% | 0.1% | -0.4% |
| Class D | -0.1% | -0.3% | -0.4% | 0.0% | -0.4% | -0.3% |
| Class E | 0.0% | -0.1% | 0.1% | -0.1% | 0.0% | -0.6% |
| Overall | 0.0% | -0.2% | -0.2% | -0.1% | -0.1% | -0.3% |
|  | 0.0% | -0.3% | -0.2% | -0.1% | -0.1% | -0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.2% | -0.3% | -0.1% | -0.1% | -0.4% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | -0.4% | -0.2% |
| Class D | -0.1% | -0.4% | -0.6% | -0.1% | -0.1% | -0.6% |
| Class E | 0.0% | -0.5% | -0.3% | -0.1% | -1.2% | -0.9% |
| Overall | 0.0% | -0.3% | -0.3% | -0.1% | -0.4% | -0.5% |
|  | 0.0% | -0.4% | -0.3% | -0.1% | -0.4% | -0.5% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusions

We confirm performance reported by MediaTek and Qualcomm in JCTVC-G609. Modified off-set encoding in SAO provides 0.0% (Y), -0.2% (U), -0.2% (V) BD-rate change.

# References

- Chih-Ming Fu, Yu-Wen Huang, Shawmin Lei, In-Suk Chong and M. Karczewicz, “Offset coding in SAO”, JCTVC-G609, Geneva, Switzerland, November, 2011

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011