| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G819 |
| --- | --- |

| Title: | CE10: Cross check for core transform proposed from Qualcomm (JCTVC-G579) by Samsung. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Elena Alshina Wonjae Lee JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.net ellio.lee@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

# Abstract

This is cross-check report for Qualcomm’s contribution in CE10. We have made performance test for normal QP range. We also provide h/w test results for 32 pt inverse transform.

# Introduction

Implementation was modified by proponent compare to original contribution JCTVC-F352. During h/w preparation additional modification which allows reducing G/C was done. Final version of code was provided for cross-check in November. Implementation was done in HM3.0 (according to CE10 agreement). Performance of this algorithm in HM4.0 was not tested in CE10.

Core transform proposed by Qualcomm support only full-factorization form. Matrix quantization instead of flat quantization in HM core transform is used.

# Performance test

We run full-frame test at normal QP range according to [1] for IA-HE/LC, RA-HE/LC and LD(B)-HE/LC test cases. Proposed by Qualcomm core transform provides in average 0.0%(Y)/0.2%(U)/0.0%(V) BD-rate change compare to anchor which is HM3.0. More details can be found Table 1 and excel spreadsheet attached to this report.

Table 1. Normal QP test for  G-579 in HM3.0.

| JCTVC-G579 vs HM3.0 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Intra |  |  | Intra LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.2 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class E | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | -0.1 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | -0.2 | 0.1 | 0.3 | 0.0 |
| Class B | 0.0 | 0.0 | 0.2 | 0.0 | 0.1 | 0.2 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.2 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.1 | 0.1 | 0.2 | -0.3 |
| Class C | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.1 | 0.0 | 0.0 | 0.3 | 0.1 |
| Class E | 0.1 | 1.0 | 0.7 | 0.0 | 1.1 | -1.0 |
| All | 0.0 | 0.3 | 0.2 | 0.0 | 0.3 | -0.3 |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
| Avg 6 cases: | 0.0 | 0.2 | 0.0 |  |  |  |

# HW test

HW test was done in scope of CE10. C-code 1d for 32pt inverse transform was provided by proponent and several times modified in order to reduce G/C.

We generated RTL automatically using CATAPULT C and synthesize it using Design Compiler (DC).

The same test was done by STMicro (independent CE10 expert). Instead of CATAPULT C they used SynthonyCC.

Accrodung to agreement in CE10 we made h/w test for tested different latency (L=2..9) and different frequency (F=150…400 MHz). Tables 2-5 show G/C measured by Samsung and STMicro for both G-579 and HM 32 ot core transform. G/C cannot be the same because of different RTL generators and different DC constrain (which cannot be disclosed).

Table 6 show ratio between G-579 and HM core transforms G/C.

Table 2. G/C measured for G-579 32pt 1D inverse trasnform by Samsung with RTL generated by CATAPULT C

| F¦L | 2 | 3 | 4 | 5 | 6 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 150 |  | 78283 | 74454 | 78938 | 83464 | 98011 | 104064 |
| 200 |  | 92613 | 91428 | 94368 | 92212 | 105239 | 107856 |
| 250 |  |  | 102174 | 103837 | 103290 | 114245 | 117588 |
| 300 |  |  |  | 113809 | 109861 | 121226 | 124192 |
| 350 |  |  |  |  | 115734 | 125770 | 126901 |
| 400 |  |  |  |  |  |  | 130972 |
| 450 |  |  |  |  |  |  |  |
| 500 |  |  |  |  |  |  |  |

Table 3. G/C measured HM4.0 32pt 1D inverse trasnform by Samsung with RTL generated by CATAPULT C

| F¦L | 2 | 3 | 4 | 5 | 6 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 150 | 119476 | 127993 | 139148 | 153902 |  |  |  |
| 200 |  | 149149 | 161630 | 168979 |  |  |  |
| 250 |  | 163631 | 178468 | 185112 |  |  |  |
| 300 |  |  |  | 199785 |  |  |  |
| 350 |  |  |  |  |  |  |  |
| 400 |  |  |  |  |  |  |  |
| 450 |  |  |  |  |  |  |  |
| 500 |  |  |  |  |  |  |  |

Table 4. G/C measured for G-579 32pt 1D inverse trasnform by STMicros with RTL generated by SynthonyCC.

| F¦L | 2 | 3 | 4 | 5 | 6 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 150 | 116031 | 94362 | 78452 | 75705 | 75390 | 79697 | 82231 |
| 200 |  | 129802 | 93773 | 95209 | 97408 | 95931 | 94642 |
| 250 |  |  | 118496 | 105109 | 101987 | 106014 | 103305 |
| 300 |  |  |  | 120813 | 111168 | 108841 | 109734 |
| 350 |  |  |  |  | 135386 | 123383 | 115746 |
| 400 |  |  |  |  |  |  | 129617 |
| 450 |  |  |  |  |  |  |  |
| 500 |  |  |  |  |  |  |  |

Table 5. G/C measured for HM4.0 32pt 1D inverse trasnform by STMicros with RTL generated by SynthonyCC.

| F¦L | 2 | 3 | 4 | 5 | 6 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 150 | 108364 | 134553 | 147701 |  |  | 182063 |  |
| 200 | 110269 | 134615 | 147711 |  |  | 182016 |  |
| 250 | 123155 | 143120 | 151092 |  |  | 182387 |  |
| 300 | 126322 | 143793 | 159014 |  |  | 204536 |  |
| 350 | 129150 | 153378 | 168162 |  |  | 204752 |  |
| 400 |  | 165196 | 178534 |  |  | 216075 |  |
| 450 |  |  |  |  |  |  |  |
| 500 |  |  |  |  |  |  |  |

Table 6. Ratio of G-579 and HM core trasnform G/C.

| By Samsung |  |  |  | By STMicro |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| F¦L | 3 | 4 | 5 | 2 | 3 | 4 | 7 |  |
| 150 | 61% | 54% | 51% | 107% | 70% | 53% | 44% |  |
| 200 | 62% | 57% | 56% |  | 96% | 63% | 53% |  |
| 250 |  | 57% | 56% |  |  | 78% | 58% |  |
| 300 |  |  | 57% |  |  |  | 53% |  |
| 350 |  |  |  |  |  |  | 60% |  |
| 400 |  |  |  |  |  |  |  |  |
| 450 |  |  |  |  |  |  |  |  |
| 500 |  |  |  |  |  |  |  |  |

In both measurements fro G-579 G/C has non-monoton befaviour with latency increment. Minimum of G/C is about L=4-5. With increment of frequency G/C increases in both tests.

Based on Sasmung’s measurement G579 requres 38-49% fewer gates than HM core transforms. According to STMicro measurement G579 uses less gates than HM core transform for all cases except L=2.

# Conclusions

We confirm performance reported by Qualcomm for G-579 in normal QP test:

0.0%(Y)/0.2%(U)/0.0%(V) BD-rate change.

According to our h/w test G-579 needs 38-49% fewer gates than HM 32 pt transform.

But G-579 cannot support partial butterfly or matrix multiplication form which can be desirable for specific applications and uses “non-flat” quantization.

# References

- P. Topiwala, M. Budagavi, A. Fuldseth, R. Joshi, I.-K. Kim, “CE10: Core Transform Design”JCTVC-F910, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011H.264/AVC Encoder”, IEEE TRANSACTIONS ON CIRCUITS AND SYSTEMS FOR VIDEO TECHNOLOGY, VOL. 16, NO. 6, JUNE 2006

APPENDIX : C-code used in h/w test

void fullButterflyInverse32(short src[32],short des[32])

{

int n=32, n1=16;

int y[32];

int w[32];

int v[32];

int u[32];

int t[32];

int s[32];

int r[32];

int p4, p7

int p20, p23, p28, p31

static const int A4_II=17, B4_II=41

static const int shift4_II=6;

static const int A8_II=17, B8_II=41

static const int C8_II=111, D8_II=22, E8_II=94, F8_II=63;

static const int shift8_II=7;

static const int N8_IV=100, L8_IV=297, J8_IV=483, H8_IV=650, G8_IV=792, I8_IV=903, K8_IV=980, M8_IV=1019;

static const int shift8_IV=10;

static const int B16_IV=50, D16_IV=150, F16_IV=249, H16_IV=345, J16_IV=438, L16_IV=526, N16_IV=610, P16_IV=688;

static const int O16_IV=759, M16_IV=822, K16_IV=878, I16_IV=926, G16_IV=964, E16_IV=993, C16_IV=1013, A16_IV=1023;

static const int shift16_IV=10;

w[0] = src[ 0];

w[1] = src[16];

w[2] = src[ 8];

w[3] = src[24];

w[4] = src[28];

w[5] = src[12];

w[6] = src[20];

w[7] = src[ 4];

v[0] = w[0] + w[1];

v[1] = w[0] - w[1];

v[2] = (A8_II * w[2] - B8_II * w[3]) >> shift4_II;

v[3] = (B8_II * w[2] + A8_II * w[3]) >> shift4_II;

u[0] = v[0] + v[3];

u[1] = v[1] + v[2];

u[2] = v[1] - v[2];

u[3] = v[0] - v[3];

p7 =  w[4] + w[7];

p4 = -w[4] + w[7];

v[4] =   p4  + w[6];

v[5] = -w[5] + p7;

v[6] =   p4  - w[6];

v[7] =  w[5] + p7;

u[4] = (E8_II * v[4] - F8_II * v[7]) >> shift8_II;

u[5] = (C8_II * v[5] - D8_II * v[6]) >> shift8_II;

u[6] = (D8_II * v[5] + C8_II * v[6]) >> shift8_II;

u[7] = (F8_II * v[4] + E8_II * v[7]) >> shift8_II;

s[0] = u[0] + u[7];

s[7] = u[0] - u[7];

s[1] = u[1] + u[6];

s[6] = u[1] - u[6];

s[2] = u[2] + u[5];

s[5] = u[2] - u[5];

s[3] = u[3] + u[4];

s[4] = u[3] - u[4];

w[ 8] = src[ 2];

w[ 9] = src[10];

w[10] = src[18];

w[11] = src[26];

w[12] = src[ 6];

w[13] = src[14];

w[14] = src[22];

w[15] = src[30];

v[ 8] = w[ 8];

v[ 9] = w[ 9] + w[12];

v[10] = w[10] + w[13];

v[11] = w[11] + w[14];

v[15] =  w[ 9] - w[12];

v[14] = -w[10] + w[13];

v[13] =  w[11] - w[14];

v[12] =  w[15];

u[ 8] = v[ 8] + v[10];

u[ 9] = v[ 8] - v[10];

u[10] = (A4_II * v[ 9] - B4_II * v[11]) >> shift4_II;

u[11] = (B4_II * v[ 9] + A4_II * v[11]) >> shift4_II;

t[ 8] = u[ 8] + u[11];

t[ 9] = u[ 9] + u[10];

t[10] = u[ 9] - u[10];

t[11] = u[ 8] - u[11];

u[12] = v[12] + v[14];

u[13] = v[12] - v[14];

u[14] = (A4_II * v[13] - B4_II * v[15]) >> shift4_II;

u[15] = (B4_II * v[13] + A4_II * v[15]) >> shift4_II;

t[12] =  u[12] + u[15];

t[13] = -u[13] - u[14];

t[14] =  u[13] - u[14];

t[15] = -u[12] + u[15];

s[ 8] = (M8_IV * t[ 8] - N8_IV * t[15]) >> shift8_IV;

s[ 9] = (K8_IV * t[ 9] - L8_IV * t[14]) >> shift8_IV;

s[10] = (I8_IV * t[10] - J8_IV * t[13]) >> shift8_IV;

s[11] = (G8_IV * t[11] - H8_IV * t[12]) >> shift8_IV;

s[12] = (H8_IV * t[11] + G8_IV * t[12]) >> shift8_IV;

s[13] = (J8_IV * t[10] + I8_IV * t[13]) >> shift8_IV;

s[14] = (L8_IV * t[ 9] + K8_IV * t[14]) >> shift8_IV;

s[15] = (N8_IV * t[ 8] + M8_IV * t[15]) >> shift8_IV;

y[ 0] = s[ 0] + s[ 8];

y[15] = s[ 0] - s[ 8];

y[ 1] = s[ 1] + s[ 9];

y[14] = s[ 1] - s[ 9];

y[ 2] = s[ 2] + s[10];

y[13] = s[ 2] - s[10];

y[ 3] = s[ 3] + s[11];

y[12] = s[ 3] - s[11];

y[ 4] = s[ 4] + s[12];

y[11] = s[ 4] - s[12];

y[ 5] = s[ 5] + s[13];

y[10] = s[ 5] - s[13];

y[ 6] = s[ 6] + s[14];

y[ 9] = s[ 6] - s[14];

y[ 7] = s[ 7] + s[15];

y[ 8] = s[ 7] - s[15];

u[16] = src[ 1];

u[17] = src[17] + src[15];

u[18] = src[ 9] + src[ 7];

u[19] = src[25] + src[23];

u[20] = src[29] + src[27];

u[21] = src[13] + src[11];

u[22] = src[21] + src[19];

u[23] = src[ 5] + src[ 3];

t[16] = u[16] + u[17];

t[17] = u[16] - u[17];

t[18] = (A8_II * u[18] - B8_II * u[19]) >> shift4_II;

t[19] = (B8_II * u[18] + A8_II * u[19]) >> shift4_II;

s[16] = t[16] + t[19];

s[17] = t[17] + t[18];

s[18] = t[17] - t[18];

s[19] = t[16] - t[19];

p23 =  u[20] + u[23];

p20 = -u[20] + u[23];

t[20] =  p20   + u[22];

t[21] = -u[21] + p23;

t[22] =  p20   - u[22];

t[23] =  u[21] + p23;

s[20] = (E8_II * t[20] - F8_II * t[23]) >> shift8_II;

s[21] = (C8_II * t[21] - D8_II * t[22]) >> shift8_II;

s[22] = (D8_II * t[21] + C8_II * t[22]) >> shift8_II;

s[23] = (F8_II * t[20] + E8_II * t[23]) >> shift8_II;

r[16] = s[16] + s[23];

r[23] = s[16] - s[23];

r[17] = s[17] + s[22];

r[22] = s[17] - s[22];

r[18] = s[18] + s[21];

r[21] = s[18] - s[21];

r[19] = s[19] + s[20];

r[20] = s[19] - s[20];

u[24] = src[31];

u[25] = src[15] - src[17];

u[26] = src[23] - src[25];

u[27] = src[ 7] - src[ 9];

u[28] = src[ 5] - src[ 3];

u[29] = src[21] - src[19];

u[30] = src[13] - src[11];

u[31] = src[29] - src[27];

t[24] = u[24] + u[25];

t[25] = u[24] - u[25];

t[26] = (A8_II * u[26] - B8_II * u[27]) >> shift4_II;

t[27] = (B8_II * u[26] + A8_II * u[27]) >> shift4_II;

s[24] = t[24] + t[27];

s[25] = t[25] + t[26];

s[26] = t[25] - t[26];

s[27] = t[24] - t[27];

p31 =  u[28] + u[31];

p28 = -u[28] + u[31];

t[28] =   p28  + u[30];

t[29] = -u[29] +  p31;

t[30] =   p28  - u[30];

t[31] =  u[29] +  p31;

s[28] = (E8_II * t[28] - F8_II * t[31]) >> shift8_II;

s[29] = (C8_II * t[29] - D8_II * t[30]) >> shift8_II;

s[30] = (D8_II * t[29] + C8_II * t[30]) >> shift8_II;

s[31] = (F8_II * t[28] + E8_II * t[31]) >> shift8_II;

r[24] =  s[24] + s[31];

r[31] = -s[24] + s[31];

r[25] = -s[25] - s[30];

r[30] =  s[25] - s[30];

r[26] =  s[26] + s[29];

r[29] = -s[26] + s[29];

r[27] = -s[27] - s[28];

r[28] =  s[27] - s[28];

y[16] = (A16_IV * r[16] - B16_IV * r[31]) >> shift16_IV;

y[17] = (C16_IV * r[17] - D16_IV * r[30]) >> shift16_IV;

y[18] = (E16_IV * r[18] - F16_IV * r[29]) >> shift16_IV;

y[19] = (G16_IV * r[19] - H16_IV * r[28]) >> shift16_IV;

y[20] = (I16_IV * r[20] - J16_IV * r[27]) >> shift16_IV;

y[21] = (K16_IV * r[21] - L16_IV * r[26]) >> shift16_IV;

y[22] = (M16_IV * r[22] - N16_IV * r[25]) >> shift16_IV;

y[23] = (O16_IV * r[23] - P16_IV * r[24]) >> shift16_IV;

y[31] = (B16_IV * r[16] + A16_IV * r[31]) >> shift16_IV;

y[30] = (D16_IV * r[17] + C16_IV * r[30]) >> shift16_IV;

y[29] = (F16_IV * r[18] + E16_IV * r[29]) >> shift16_IV;

y[28] = (H16_IV * r[19] + G16_IV * r[28]) >> shift16_IV;

y[27] = (J16_IV * r[20] + I16_IV * r[27]) >> shift16_IV;

y[26] = (L16_IV * r[21] + K16_IV * r[26]) >> shift16_IV;

y[25] = (N16_IV * r[22] + M16_IV * r[25]) >> shift16_IV;

y[24] = (P16_IV * r[23] + O16_IV * r[24]) >> shift16_IV;

des[ 0] =( y[ 0] + y[16]+ 32)>>6;

des[ 1] =( y[ 1] + y[17]+ 32)>>6;

des[ 2] =( y[ 2] + y[18]+ 32)>>6;

des[ 3] =( y[ 3] + y[19]+ 32)>>6;

des[ 4] =( y[ 4] + y[20]+ 32)>>6;

des[ 5] =( y[ 5] + y[21]+ 32)>>6;

des[ 6] =( y[ 6] + y[22]+ 32)>>6;

des[ 7] =( y[ 7] + y[23]+ 32)>>6;

des[ 8] =( y[ 8] + y[24]+ 32)>>6;

des[ 9] =( y[ 9] + y[25]+ 32)>>6;

des[10] =( y[10] + y[26]+ 32)>>6;

des[11] =( y[11] + y[27]+ 32)>>6;

des[12] =( y[12] + y[28]+ 32)>>6;

des[13] =( y[13] + y[29]+ 32)>>6;

des[14] =( y[14] + y[30]+ 32)>>6;

des[15] =( y[15] + y[31]+ 32)>>6;

des[31] =( y[ 0] - y[16]+ 32)>>6;

des[30] =( y[ 1] - y[17]+ 32)>>6;

des[29] =( y[ 2] - y[18]+ 32)>>6;

des[28] =( y[ 3] - y[19]+ 32)>>6;

des[27] =( y[ 4] - y[20]+ 32)>>6;

des[26] =( y[ 5] - y[21]+ 32)>>6;

des[25] =( y[ 6] - y[22]+ 32)>>6;

des[24] =( y[ 7] - y[23]+ 32)>>6;

des[23] =( y[ 8] - y[24]+ 32)>>6;

des[22] =( y[ 9] - y[25]+ 32)>>6;

des[21] =( y[10] - y[26]+ 32)>>6;

des[20] =( y[11] - y[27]+ 32)>>6;

des[19] =( y[12] - y[28]+ 32)>>6;

des[18] =( y[13] - y[29]+ 32)>>6;

des[17] =( y[14] - y[30]+ 32)>>6;

des[16] =( y[15] - y[31]+ 32)>>6;

}