| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G820 |
| --- | --- |

| Title: | CE3: Cross-check for Motorola Mobility proposals on interpolation MC (JCTVC-G696, 697) by Samsung. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin Kim Jaemoon JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.net alexander_b.alshin@samsung.net jaemoonc.kim@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

# Abstract

This is cross-check report for 2 contributions form Motorola Mobility in CE3. In first contribution 4 variations of solid interpolation filter for HEVC are studied. All variations use efficient phase shift different from 1/4 for quarter-pel position. We conclude that there are no benefits from this under common test conditions.

The second contribution is on joint sub-pixel interpolation in bi-prediction (JSPIF). This one provides performance improvement with computational complexity reduction. But according to our analysis JSPIS increases ~5 times h/w implementation cost. For this test proponent provided bug-fix version of s/w, so we plan to up-date this report after we’ll complete simulations.

# Solid interpolation filters

In both 4 proposed modification of solid interpolation filter proponent changes phase shift for quarter-pel interpolation filter from 1/4 to smaller one: 13/64, 3/16 or 15/64. Chroma filter uses phase shifts: 3/32 instead 1/8 and 13/32 instead 3/8 in order to be synchronized with Luma MC.

Tables 1 and 2 show interpolation filter coefficients used in the tests.

Table 1 Luma interpolation filter coefficients

|  | Position | Coefficients |
| --- | --- | --- |
| 6H+7Q (13/64) | 1/4 | {-1, 4, -10, 61, 13, -4, 1, 0,} |
|  | 1/2 | {0, 2, -9, 39, 39, -9, 2, 0,} |
|  | 3/4 | {0, 1, -4, 13, 61, -10, 4, -1,} |
| 6H+7Q (3/16) | 1/4 | {-1, 3, -8, 60, 13, -4, 1, 0,} |
|  | 1/2 | {0, 2, -9, 39, 39, -9, 2, 0,} |
|  | 3/4 | {0, 1, -4, 13, 60, -8, 3, -1,} |
| 6H+7Q (15/64) | 1/4 | {-2, 5, -11, 60, 15, -4, 1, 0,} |
|  | 1/2 | {0, 2, -9, 39, 39, -9, 2, 0,} |
|  | 3/4 | {0, 1, -4, 15, 60, -11, 5, -2,} |
| 8H+8Q (3/16) | 1/4 | {-1, 3, -8, 60, 14, -6, 3, -1,} |
|  | 1/2 | {-1, 4, -11, 40, 40, -11, 4, -1,} |
|  | 3/4 | {-1, 3, -6, 14, 60, -8, 3, -1,} |

Table 2 Chroma interpolation filter coefficients

| Phase shift | Position | Coefficients |
| --- | --- | --- |
| 3/32 | 1/8 | {-2, 61, 6, -1, } |
| 1/4 | 1/4 | {-4, 54, 16, -2,} |
| 13/32 | 3/8 | {-5, 44, 29, -4,} |
| 1/2 | 1/2 | {-4, 36, 36, -4,} |
| 19/32 | 5/8 | {-4, 29, 44, -5,} |
| 3/4 | 3/4 | {-2, 16, 54, -4,} |
| 29/32 | 7/8 | {-1, 6, 61, -2,} |

All tests were run using test conditions [1] for RA-HE/LC, LD(B)-HE/LC and LD(P)-HE/LC according to [2].

Summary of all 4 tests is shown in Table 3. All values are averaged across only common test cases RA-HE/LC, LD(B)-HE/LC.

According to the theoretical analysis in [3] phase shift less than 1/4 for quarter-pel filter leads to statistical increment of fractional positions in MC process and so increment of computations. Proponent uses phase shifts 12/64 (=3/16), 13/64, 15/64 for 7Q filter and computation complexity increases if efficient phase shift goes far from 1/4.

In the family of 7Q6H filters tested by Motorola 7Q(15/64) 6H with closest to 1/4 phase shift looks the most attractive: smallest performance drop compare to anchor with maximum reduction of computations.

Third line in Table 3 is one more test in CE3 done by Samsung [4] for normal phase shift 1/4 for quarter-pel position filter 6H+7Q (1/4). It seems that for common test condition there is no benefits from usage phase shift different from 1/4 for quarter-pel position filter.

Table 3. Summary of solid interpolation filter tests.

| Filter | BD-rate Y | BD-rate U | BD-rate V | mults | adds |
| --- | --- | --- | --- | --- | --- |
| 6H+7Q (12/64) | 0.5% | 0.5% | 0.4% | -5% | -7% |
| 6H+7Q (13/64) | 0.2% | 0.3% | 0.2% | -6% | -8% |
| 6H+7Q (15/64) | 0.1% | 0.4% | 0.3% | -10% | -13% |
| 6H+7Q (16/64)* | 0.1% | -0.1% | -0.1% | -13% | -15% |
| 8H+8Q (3/16) | -0.2% | 0.2% | 0.2% | 8% | 8% |

For 8Q(3/16)8H interpolation filter proposed by Motorola we found Luma performance improvement which is equal to Chroma drop. Reduction of efficient phase shift compare to 1/4 here also leads to computational complexity increment (8%).

More details are reported in Tables 4-7 (in APPENDIX A) and excel spread-sheets attached to this contribution. Average gain and complexity in CE3 template (and Tables 4-7) are reported across 6 test cases (including LD(P) tests).

We need to emphasize that for optional test LD(P) used in CE3 only almost all tested filters provides performance improvement up to 1.2% (7Q(3/16)6H, LD(P)-LC test).

Variations of encoding decoding time were no more 3% in all tests for solid interpolation filters proposed by Motorola.

# Joint sub-pixel interpolation in bi-prediction

# S/w for this test was provided few weeks after CE3 dead-line. Unfortunately one bug was found in implementation during cross-check few days before the meeting. Proponent has fixed a code and new version arrived Nov, 18. We are re-testing everything urgently and hope to up-date this report with correct data shortly after meeting start. Please note that data in Table 9 (performance and complexity) will be slightly different after bug-fix.

Proponent has presented 3 variants of design based on idea described in JCTVC-F601 [4]. In s/w switch between these 3 designs are controlled by macro IF_Y_BI_MC_SET.

# In opposite to G-696 this is multi-filter solution for MC. In HM4.0 we have solid interpolation filter. For Luma 1/4-pel MC only 2 different filters are needed:

1/4: {  -1,    4,   -10,    57,    19,    -7,    3,    -1 },

1/2: {  -1,    4,   -11,    40,    40,   -11,    4,    -1 },

3/4: {  -1,    3,    -7,    19,    57,   -10,    4,    -1 }.

Filter for 3/4 is mirror reflection of 1/4-pel filter, by re-ordering the input oone my use the same module to performe 3/4 and 1/4-pel interpoaltions.

In G-697 proponent modified 1/4-pel position filter to 7 taps:

{  -1,    3,    -8,    60,    13,    -4,    1,     0 }.

This variant of solid interpolation filter is tested in [5].

Additional filters implementation cost.

For joint sub-pixel interpoaltion in bi-prediction proponent has intoruduced 9 additional “no more than 6 taps” filters (we don’t count additional filters which are mirror reflection of mentioned below):

{       1,      -3,       6,     -15,      61,      14,       0,       0}, // ( 1, 1)

{      -9,      40,      44,     -15,       6,      -2,       0,       0}, // ( 2, 1)

{      -4,      13,      61,     -10,       5,      -1,       0,       0}, // ( 3, 1)

{       1,      -3,       9,     -13,      61,       9,       0,       0}, // ( 1, 2)

{      37,      43,     -24,      12,      -6,       2,       0,       0}, // ( 2, 2)

{      -2,       6,     -16,      45,      39,      -8,       0,       0}, // ( 2, 3)

{       1,      -3,       6,     -12,      15,      57,       0,       0}, // ( 3, 3)

{      57,      15,     -12,       6,      -3,       1,       0,       0}, // ( 1, 1)

{      10,      60,     -12,       8,      -2,       0,       0,       0}. // ( 3, 2)

Here we listed filetrs in variant IF_Y_BI_MC_SET = 2. Number of additional filters is the same in all designs (only coefficents are different).

In total for Luma 1/4-pel MC 11 different interpolation filters are needed in G-697 design. Of course this leads to increment of h/w implemmentation cost. Before Daegue meeeting TI provides excellent analysis and comparison for several interpolation filters studied in CE3 that time [6]. Among filters studied by TI there was SIFO 6 taps which is set of 8 different 6 taps filters (which is smaller than G-697 we analyse ghere). According to [6] Area cost of SIFO6 is 4 times higher than DCTIF8 we now use in HM4.0 and 2 times higher than DCTIF 12 taps. So we may conclude that joint sub-pixel interpolation proposed by motorola has at least 4 times higher h/w implementation cost compare to current HM4.0 filter and at least 2 times higer than 12 taps DCTIF. In Daegu together with DCTIF 8 taps adopted for HM we show 12 taps DCTIF which [7] performes 1.7% better than DCTIF8 taps (average gain across RA and LD test cases, LD(P) test was not used that time, test data from D-344 are attached to this report). So Daegu meeting decision was that 2 times higher h/w implementation cost of DCTIF12 taps cann not be justified by 1.7% gain it shows over DCTIF 8 taps. So to justify 4 times higer h/w implementation cost much higher than 1.7% gain is needed.

H/w implementation of inetrpoaltionf iletr

Additionally to h/w analysis made by TI [6] we would like to present our h/w implementation of interpoaltion filter for HEVC. We do belive that this analysis will be usefull for CE3 members.

Following is a hardware architecture of DCTIF 8H+8Q with four-pixel parallel processing. Iinterpolation processing for 4x4 block is operated within 11 clock cycles.

This architecture is very similar to described in [9] and it is commonly used.

Fig. 1. H/w architecture of DCTIF 8H8Q interpoaltion filetr.

The hardware resources required to implement this architecture are listed in Table 7.

According to the table, ~ 60k (~28k x 2, L0 / L1) gate count is required to implement 8H+8Q interpolation ilter. L0 and L1 are processed in parralel in order to achive realy time decoding og high resolution images, so area is doubled.

If we use multiple different filters, then the gate count will increase according to the multiple filters. In hardware design, different types of filters must be implemented respectively.

As the JSPIF uses at least 11 different types of filters instead of 2 in HM4.0, the total gate count of JSPIF will be ~5 times bigger than that of DCTIF 8H+8Q. There is very rought prediciton for resulting G/C: 28.0/(8+4) *(8*6+5+8+4) *2  303 kGates (we took into account symmetry of half-pel filter and shorter size of additional filtrers in .

One more possible difficulty JSPIF introduce to h/w implementation is dependency between L0 and L1: to choose interpolation filter in L0 one needs to know that will be in L1 and in opposite L1 needs MV from L0. This doesn’t break parallel processing of L0/L1 but requires additional information exchange.

Table 7. H/w resources needed for implementation of DCTIF 8Q8H.

|  | Gate count | # of elements | Total G/C |
| --- | --- | --- | --- |
| 1/4 DCT-IF (8b in / 16b out) | 784.2 | 4 | 3.1k |
| 2/4 DCT-IF (8b in / 16b out) | 762.8 | 4 | 3.1k |
| 1/4 DCT-IF (16b in / 8b out) | 2065.2 | 4 | 8.3k |
| 2/4 DCT-IF (16b in / 8b out) | 1870.7 | 4 | 7.5k |
| register (D-flip-flop) | 10 | 600 bit =(8x16b)x4 +11x8b | 6.0k |
| Total gate count |  |  | 28.0k |

Complexity

In case then both reference lists are identical and MVs are equal in bi-prediction we don’t need second interpoaltion. But HM s/w doesn’t have this restriction. This is pure implementation issue and solution is proposed in [8]. Proponent of [8] report complexity reduction even visible in term of decodign time. Unfortunately the have not ptovided complexity measurement in CE3 format. JSPIF also resolves this issue on slightly different way. But true anchor in terms of operations count and memory access for JSPIF should be [8], and it whould be nice to compare them with CE complexity metric.

Performance test.
Both 3 variants were tested using test conditions [1] for RA-HE/LC, LD(B)-HE/LC and LD(P)-HE/LC according to [2].

Summary of all 3 tests is shown in Table 9. BD-rate is reported compare to HM4.0. All values are averaged across only common test cases RA-HE/LC, LD(B)-HE/LC. We need to mention that there are 2 changes are tested together here: quarter-pel position filter change to 7Q(3/16)8H [5] and JSPIF.

Main gain of JSPIF comes from small resolution video (BQSquare gain reaches 5.7%).

Table 9. Summary of tests for joint sub-pixel interpoaltion.

| Filter | BD-rate Y | BD-rate U | BD-rate V | mults | adds |
| --- | --- | --- | --- | --- | --- |
| SET 0 | -0.6% | 0.0% | 0.0% | -9% | -12% |
| SET 1 | -0.4% | 0.1% | 0.1% | -11% | -14% |
| SET 2 | -0.6% | 0.0% | 0.0% | -10% | -13% |

Based on summary in Table 8 we may conclude that SET2 is the most promissing from 3 variations provided by Motorola. We tested this variant in optional test for class F. Test data is reported in Table 10. Average gain for class F is 0.2% only (which is 3 times smaller than in averaged for other classes).

Table 10. JSPIF2 performance for class F.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class F | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
| Class F | -0.5% | 0.2% | 0.3% | -0.1% | -0.1% | -0.2% |

More details including DL(P) test are presented in Tables 11-13 (in APPENDIX B).

Please note that data in tables 9-10 will be up-dated after testing bug-fix version of JSPIF.

# Conclusions

We confirm performance reported by Motorola in CE3 tests for solid interpolation filters (refer Table 3). Their investigation proves that there is no benefits from usage efficient phase shift different from 1/4 for quarter-pel interpolation filter in 7Q6H solid filter scheme.

We try to do our best to confirm JSPIF performance after bug-fix. Based on old data reported by proponent we conclude that variant 2 is the most promising among variations provided by Motorola.

But we are worrying about h/w implementation cost of multi-filter solution which in case of JSPIF will be 4~5 times higher than for current HM4.0 MC.

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- T.Chujoh, E. Alshina, “CE3: interpolation MC”, JCTVC-F903, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- E. Alshina, “CE3: Cross-check by Samsung for Motorola Mobility tests on interpolation filters (JCTVC-F574, JCTVC-F576)”, JCTVC-F443 JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- Jian Lou, Koohyar Minoo, “Joint Sub-pixel Interpolation for bi-predicted Motion Compensation”, JCTVC-F601, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- Alexander Alshin, Elena Alshina, JeongHoon Park, Jian Lou, Koohyar Minoo “CE3: 7 taps interpolation filters for quarter pel position MC from Samsung and Motorola Mobility”, JCTVC-G778, Geneva, Switzerland, November 2011

- Vivienne Sze, Minhua Zhou, Madhukar Budagavi, “Investigation on Luma interpolation filters”, JCTVC-D266, 4h JCT-VC Meeting, Daegue, Korea, 2011

- Elena Alshina, Jianle Chen, Alexander Alshin, Nikolay Shlyakhov, Woo-Jin Han “CE3: Experimental results of DCTIF by Samsung”, JCTVC-D344, 4h JCT-VC Meeting, Daegue, Korea, 2011

- Hui Yong Kim, Kyung Yong Kim, Gwang Hoon Park, Sung-Chang Lim, Jinho Lee, and Jin Soo Choi “MC Complexity Reduction for Bi-prediction”, JCTVC-G415, Geneva, Switzerland, November 2011

- Tung-Chien Chen, Shao-Yi Chien, Yu-Wen Huang, Chen-Han Tsai, Ching-Yeh Chen, To-Wei Chen, and Liang-Gee Chen “Analysis and Architecture Design of an HDTV720p 30 Frames/s H.264/AVC Encoder”, IEEE TRANSACTIONS ON CIRCUITS AND SYSTEMS FOR VIDEO TECHNOLOGY, VOL. 16, NO. 6, JUNE 2006

APPENDIX A: Details of solid filter tests

Table 4. Test results summary for 6H+7Q (13/64) interpolation filter from Motorola.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2% | -0.2% | -0.1% | 0.1% | 0.2% | 0.1% |
| Class B | -0.2% | 0.0% | -0.1% | 0.1% | 0.1% | 0.0% |
| Class C | 0.2% | 0.3% | 0.2% | 0.2% | 0.2% | 0.3% |
| Class D | 0.3% | 0.2% | 0.3% | 0.6% | 0.4% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.3% | 0.2% | 0.2% |
|  | 0.0% | 0.1% | 0.1% | 0.3% | 0.2% | 0.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
| Mult[%] | -5% |  |  | -7% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | 2% |  |  | 2% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 1% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 5% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.1% | 0.1% | 0.0% | -0.1% | -0.2% |
| Class C | 0.2% | 0.4% | 0.4% | -0.1% | 0.6% | 0.3% |
| Class D | 1.2% | 1.0% | 1.1% | 0.9% | 1.0% | 0.9% |
| Class E | -0.7% | -0.2% | -0.1% | 0.2% | 0.3% | -0.4% |
| Overall | 0.1% | 0.3% | 0.4% | 0.2% | 0.4% | 0.2% |
|  | 0.1% | 0.3% | 0.3% | 0.2% | 0.4% | 0.2% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |
| Mult[%] | -6% |  |  | -7% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | 2% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 4% |  |  | 6% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 5% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | -0.3% | 0.0% | -0.7% | -0.3% | -0.5% |
| Class C | -0.3% | 0.1% | 0.2% | -0.4% | 0.5% | 0.6% |
| Class D | 0.8% | 0.3% | 1.2% | 0.8% | 1.0% | 0.6% |
| Class E | -1.5% | -1.5% | -0.5% | -1.2% | -0.8% | -1.0% |
| Overall | -0.3% | -0.3% | 0.3% | -0.4% | 0.1% | 0.0% |
|  | -0.3% | -0.3% | 0.2% | -0.4% | 0.1% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
| Mult[%] | -5% |  |  | -7% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | 2% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 4% |  |  |

| Gain Ave.[%] | -0.01% | 0.15% | 0.17% |
| --- | --- | --- | --- |
| Gain Min. [%] | -2.25% | -1.86% | -1.47% |
| Gain Max.[%] | 2.55% | 2.51% | 4.08% |
| Enc Time[%] | 100.60% |  |  |
| Dec Time[%] | 101.85% |  |  |
| Mult Ave.[%] | -6% |  |  |
| Mult Worst[%] | -9% |  |  |
| Adds Ave.[%] | -8% |  |  |
| Adds Worst[%] | -11% |  |  |
| Access[%] | 2% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |
| MemBand(2D) Max.[%] | 4% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |
| MemBand(1D) Max.[%] | 4% |  |  |
| MemBand(1D) Worst[%] | -5% |  |  |

Table 5. Test results summary for 6H+7Q (3/16) interpolation filter from Motorola.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | -0.2% | -0.4% | 0.3% | 0.3% |
| Class B | -0.1% | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% |
| Class C | 0.6% | 0.3% | 0.5% | 1.3% | 0.5% | 0.5% |
| Class D | 1.1% | 1.1% | 1.0% | 3.1% | 1.3% | 1.3% |
| Class E |  |  |  |  |  |  |
| Overall | 0.4% | 0.3% | 0.3% | 0.9% | 0.5% | 0.5% |
|  | 0.3% | 0.3% | 0.3% | 0.9% | 0.5% | 0.5% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 103% |  |  |
| Mult[%] | -6% |  |  | -6% |  |  |
| Adds[%] | -8% |  |  | -9% |  |  |
| Access[%] | 2% |  |  | 2% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 1% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 1% |  |  | 4% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% |
| Class C | 0.4% | 0.6% | 0.5% | 0.9% | 0.7% | 0.4% |
| Class D | 1.6% | 1.5% | 1.8% | 2.5% | 1.7% | 1.5% |
| Class E | -0.6% | -0.2% | 0.5% | -1.3% | -0.1% | -0.9% |
| Overall | 0.3% | 0.5% | 0.6% | 0.5% | 0.6% | 0.3% |
|  | 0.3% | 0.5% | 0.5% | 0.5% | 0.5% | 0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
| Mult[%] | -4% |  |  | -4% |  |  |
| Adds[%] | -6% |  |  | -7% |  |  |
| Access[%] | 2% |  |  | 2% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 8% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 7% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.6% | -0.5% | -1.6% | -0.3% | -0.7% |
| Class C | -0.5% | -0.3% | 0.1% | -0.4% | 0.1% | 0.1% |
| Class D | 0.7% | 0.5% | 0.1% | 0.7% | 0.5% | 0.4% |
| Class E | -1.6% | -0.9% | -0.6% | -4.1% | -1.9% | -1.2% |
| Overall | -0.4% | -0.3% | -0.2% | -1.2% | -0.3% | -0.3% |
|  | -0.4% | -0.4% | -0.3% | -1.2% | -0.3% | -0.4% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 102% |  |  |
| Mult[%] | -3% |  |  | -3% |  |  |
| Adds[%] | -6% |  |  | -6% |  |  |
| Access[%] | 3% |  |  | 3% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 2% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 9% |  |  |

| Gain Ave.[%] | 0.09% | 0.20% | 0.18% |
| --- | --- | --- | --- |
| Gain Min. [%] | -6.89% | -2.69% | -2.63% |
| Gain Max.[%] | 8.36% | 5.25% | 4.72% |
| Enc Time[%] | 100.67% |  |  |
| Dec Time[%] | 101.95% |  |  |
| Mult Ave.[%] | -4% |  |  |
| Mult Worst[%] | -9% |  |  |
| Adds Ave.[%] | -7% |  |  |
| Adds Worst[%] | -11% |  |  |
| Access[%] | 2% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  |
| MemBand(1D) Worst[%] | -5% |  |  |

Table 6. Test results summary for 6H+7Q (15/64) interpolation filter from Motorola.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.1% | -0.1% | -0.2% | 0.4% | 0.3% |
| Class B | -0.2% | 0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.2% | 0.2% | 0.2% | 0.3% |
| Class D | -0.3% | 0.2% | 0.1% | 0.3% | 0.3% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
|  | -0.1% | 0.1% | 0.1% | 0.0% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | -9% |  |  | -10% |  |  |
| Adds[%] | -11% |  |  | -12% |  |  |
| Access[%] | 0% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 5% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 5% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.2% | 0.0% | -0.2% | 0.2% | 0.1% |
| Class C | -0.1% | 0.6% | 0.5% | 0.2% | 0.6% | 0.2% |
| Class D | 0.5% | 1.9% | 1.5% | 1.0% | 1.3% | 0.9% |
| Class E | 0.2% | -0.1% | 0.7% | -0.2% | -0.3% | -0.3% |
| Overall | 0.1% | 0.7% | 0.6% | 0.2% | 0.5% | 0.3% |
|  | 0.1% | 0.6% | 0.6% | 0.2% | 0.5% | 0.3% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
| Mult[%] | -11% |  |  | -11% |  |  |
| Adds[%] | -13% |  |  | -14% |  |  |
| Access[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 2% |  |  |
| MemBand(1D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 2% |  |  | 1% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.3% | 0.5% | -0.8% | 0.2% | -0.1% |
| Class C | 0.1% | 0.8% | 0.9% | -0.1% | 0.5% | 0.6% |
| Class D | 1.3% | 1.5% | 1.7% | 1.2% | 1.8% | 1.7% |
| Class E | -0.2% | 0.0% | 0.1% | -1.7% | -1.5% | -0.9% |
| Overall | 0.3% | 0.7% | 0.8% | -0.3% | 0.4% | 0.4% |
|  | 0.3% | 0.7% | 0.8% | -0.3% | 0.4% | 0.4% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | -11% |  |  | -12% |  |  |
| Adds[%] | -14% |  |  | -14% |  |  |
| Access[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 0% |  |  | 2% |  |  |
| MemBand(1D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 0% |  |  | 2% |  |  |

| Gain Ave.[%] | 0.04% | 0.41% | 0.40% |
| --- | --- | --- | --- |
| Gain Min. [%] | -3.45% | -2.66% | -2.58% |
| Gain Max.[%] | 3.14% | 3.92% | 3.75% |
| Enc Time[%] | 100.09% |  |  |
| Dec Time[%] | 100.50% |  |  |
| Mult Ave.[%] | -11% |  |  |
| Mult Worst[%] | -9% |  |  |
| Adds Ave.[%] | -13% |  |  |
| Adds Worst[%] | -11% |  |  |
| Access[%] | -1% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  |
| MemBand(1D) Max.[%] | 3% |  |  |
| MemBand(1D) Worst[%] | -5% |  |  |

Table 7. Test results summary for 8H+7Q (3/16) interpolation filter from Motorola.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.1% | 0.0% | 0.3% | 0.3% |
| Class B | 0.0% | 0.1% | 0.1% | -0.3% | 0.0% | 0.1% |
| Class C | -0.1% | 0.1% | 0.1% | -0.3% | 0.1% | 0.1% |
| Class D | -0.2% | 0.1% | 0.0% | -0.4% | -0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | 0.1% | 0.1% | -0.3% | 0.1% | 0.1% |
|  | -0.1% | 0.1% | 0.1% | -0.3% | 0.1% | 0.1% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
| Mult[%] | 7% |  |  | 7% |  |  |
| Adds[%] | 7% |  |  | 7% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  | 2% |  |  |
| MemBand(2D) Max.[%] | 8% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | 2% |  |  | 2% |  |  |
| MemBand(1D) Max.[%] | 9% |  |  | 9% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | -0.2% | 0.4% | 0.7% |
| Class C | -0.1% | 0.1% | 0.1% | -0.5% | 0.5% | 0.2% |
| Class D | -0.1% | 0.3% | 0.4% | -0.6% | 0.5% | 0.5% |
| Class E | 0.4% | -0.2% | 0.5% | -0.2% | 0.0% | -0.6% |
| Overall | 0.1% | 0.1% | 0.3% | -0.4% | 0.4% | 0.3% |
|  | 0.1% | 0.0% | 0.2% | -0.4% | 0.3% | 0.3% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | 9% |  |  | 10% |  |  |
| Adds[%] | 10% |  |  | 10% |  |  |
| Access[%] | 5% |  |  | 5% |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(2D) Max.[%] | 10% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(1D) Max.[%] | 10% |  |  | 9% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.0% | 0.0% | 0.1% | 0.2% | 0.1% |
| Class C | -1.0% | -0.1% | -0.3% | -0.6% | 0.3% | 0.4% |
| Class D | -1.3% | -0.6% | -0.6% | -1.2% | 0.1% | 0.1% |
| Class E | -0.5% | 0.3% | 0.4% | 0.9% | -0.5% | 0.4% |
| Overall | -0.8% | -0.1% | -0.1% | -0.3% | 0.1% | 0.2% |
|  | -0.8% | -0.2% | -0.1% | -0.3% | 0.0% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
| Mult[%] | 10% |  |  | 9% |  |  |
| Adds[%] | 10% |  |  | 9% |  |  |
| Access[%] | 5% |  |  | 5% |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  | 3% |  |  |
| MemBand(2D) Max.[%] | 9% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | -100% |  |  | 3% |  |  |
| MemBand(1D) Max.[%] | -100% |  |  | 8% |  |  |

| Gain Ave.[%] | -0.28% | 0.09% | 0.13% |
| --- | --- | --- | --- |
| Gain Min. [%] | -3.60% | -1.26% | -1.58% |
| Gain Max.[%] | 1.39% | 1.26% | 1.84% |
| Enc Time[%] | 100.38% |  |  |
| Dec Time[%] | 101.00% |  |  |
| Mult Ave.[%] | 9% |  |  |
| Mult Worst[%] | 0% |  |  |
| Adds Ave.[%] | 9% |  |  |
| Adds Worst[%] | 0% |  |  |
| Access[%] | 5% |  |  |
| MemBand(2D) Ave.[%] | 2% |  |  |
| MemBand(2D) Max.[%] | 9% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | -15% |  |  |
| MemBand(1D) Max.[%] | -9% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |

APPENDIX B: Details of JSPIF tests

Table 11. Test results summary for JSPIF variant 0 (to be up-dated).

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.0% | 0.0% | 0.3% | 0.4% | 0.4% |
| Class B | 0.0% | 0.2% | 0.2% | 0.3% | 0.3% | 0.3% |
| Class C | -0.4% | -0.1% | -0.1% | -0.5% | 0.1% | 0.1% |
| Class D | -1.1% | -0.4% | -0.5% | -1.4% | -0.4% | -0.5% |
| Class E |  |  |  |  |  |  |
| Overall | -0.3% | -0.1% | -0.1% | -0.3% | 0.1% | 0.1% |
|  | -0.3% | -0.1% | -0.1% | -0.3% | 0.1% | 0.1% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
| Mult[%] | -11% |  |  | -12% |  |  |
| Adds[%] | -14% |  |  | -15% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 1% |  |  | 10% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 1% |  |  | 10% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5% | -0.1% | -0.4% | -0.4% | 0.3% | 0.4% |
| Class C | -0.8% | -0.3% | -0.2% | -0.8% | 0.4% | 0.0% |
| Class D | -1.2% | -0.6% | -0.3% | -1.2% | 0.0% | -0.1% |
| Class E | -0.7% | -0.1% | 0.4% | -0.9% | 0.2% | -0.8% |
| Overall | -0.8% | -0.3% | -0.2% | -0.8% | 0.2% | 0.0% |
|  | -0.8% | -0.3% | -0.2% | -0.8% | 0.2% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |
| Mult[%] | -7% |  |  | -7% |  |  |
| Adds[%] | -10% |  |  | -10% |  |  |
| Access[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 3% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 3% |  |  | 4% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.2% | -0.1% | -1.2% | 0.1% | -0.4% |
| Class C | -0.9% | -0.3% | -0.5% | -0.8% | -0.1% | 0.1% |
| Class D | -0.6% | -0.2% | -0.5% | -0.6% | 0.4% | 0.1% |
| Class E | -1.4% | -0.3% | -0.4% | -3.0% | -1.4% | -1.1% |
| Overall | -0.8% | -0.2% | -0.4% | -1.3% | -0.2% | -0.3% |
|  | -0.8% | -0.3% | -0.4% | -1.3% | -0.2% | -0.4% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
| Mult[%] | 3% |  |  | 4% |  |  |
| Adds[%] | 2% |  |  | 2% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 7% |  |  |

| Gain Ave.[%] | -0.72% | -0.07% | -0.14% |
| --- | --- | --- | --- |
| Gain Min. [%] | -5.60% | -2.59% | -2.78% |
| Gain Max.[%] | 0.70% | 1.17% | 1.43% |
| Enc Time[%] | 102.58% |  |  |
| Dec Time[%] | 101.47% |  |  |
| Mult Ave.[%] | -5% |  |  |
| Mult Worst[%] | 0% |  |  |
| Adds Ave.[%] | -7% |  |  |
| Adds Worst[%] | 0% |  |  |
| Access[%] | 2% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |

Table 12. Test results summary for JSPIF variant 1 (to be up-dated)..

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.2% | 0.0% | 0.4% | 0.3% | 0.5% |
| Class B | 0.2% | 0.4% | 0.4% | 0.6% | 0.4% | 0.5% |
| Class C | -0.4% | 0.0% | 0.0% | -0.5% | 0.1% | 0.2% |
| Class D | -1.1% | -0.4% | -0.3% | -1.4% | -0.5% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | -0.2% | 0.1% | 0.0% | -0.2% | 0.1% | 0.2% |
|  | -0.2% | 0.1% | 0.0% | -0.2% | 0.1% | 0.2% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
| Mult[%] | -12% |  |  | -13% |  |  |
| Adds[%] | -15% |  |  | -16% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 2% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -3% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 1% |  |  | 4% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.3% | 0.2% | -0.1% | 0.5% | 0.4% |
| Class C | -0.8% | -0.1% | -0.1% | -0.9% | 0.5% | 0.1% |
| Class D | -1.3% | -0.5% | -0.3% | -1.4% | 0.2% | 0.2% |
| Class E | -0.5% | -0.1% | -0.2% | -0.4% | 0.2% | -0.8% |
| Overall | -0.7% | -0.1% | -0.1% | -0.7% | 0.4% | 0.1% |
|  | -0.7% | -0.1% | -0.1% | -0.7% | 0.4% | 0.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |
| Mult[%] | -9% |  |  | -10% |  |  |
| Adds[%] | -12% |  |  | -13% |  |  |
| Access[%] | 6% |  |  | 5% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | 4% |  |  | 4% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.2% | -0.1% | -1.2% | 0.1% | -0.4% |
| Class C | -0.9% | -0.3% | -0.5% | -0.8% | -0.1% | 0.1% |
| Class D | -0.6% | -0.2% | -0.5% | -0.6% | 0.4% | 0.1% |
| Class E | -1.4% | -0.3% | -0.4% | -3.0% | -1.4% | -1.1% |
| Overall | -0.8% | -0.2% | -0.4% | -1.3% | -0.2% | -0.3% |
|  | -0.8% | -0.3% | -0.4% | -1.3% | -0.2% | -0.4% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
| Mult[%] | 3% |  |  | 4% |  |  |
| Adds[%] | 2% |  |  | 2% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 7% |  |  |

| Gain Ave.[%] | -0.64% | 0.01% | -0.07% |
| --- | --- | --- | --- |
| Gain Min. [%] | -5.73% | -2.59% | -2.29% |
| Gain Max.[%] | 0.94% | 1.44% | 1.22% |
| Enc Time[%] | 102.68% |  |  |
| Dec Time[%] | 101.94% |  |  |
| Mult Ave.[%] | -6% |  |  |
| Mult Worst[%] | 0% |  |  |
| Adds Ave.[%] | -9% |  |  |
| Adds Worst[%] | 0% |  |  |
| Access[%] | 5% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |
| MemBand(1D) Max.[%] | 4% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |

Table 13. Test results summary for JSPIF variant 2 (to be up-dated)..

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.0% | 0.1% | 0.2% | 0.2% |
| Class B | -0.1% | 0.2% | 0.1% | 0.1% | 0.2% | 0.2% |
| Class C | -0.4% | -0.1% | -0.1% | -0.5% | 0.0% | 0.0% |
| Class D | -1.1% | -0.5% | -0.4% | -1.3% | -0.5% | -0.3% |
| Class E |  |  |  |  |  |  |
| Overall | -0.4% | 0.0% | -0.1% | -0.4% | 0.0% | 0.1% |
|  | -0.4% | 0.0% | -0.1% | -0.4% | 0.0% | 0.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
| Mult[%] | -11% |  |  | -12% |  |  |
| Adds[%] | -14% |  |  | -15% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(2D) Max.[%] | 0% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -2% |  |  |
| MemBand(1D) Max.[%] | -1% |  |  | 4% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.0% | 0.1% | -0.5% | 0.4% | 0.5% |
| Class C | -0.9% | -0.1% | -0.2% | -0.6% | 0.1% | 0.0% |
| Class D | -1.2% | -0.9% | -0.1% | -1.0% | 0.3% | 0.2% |
| Class E | -0.7% | -0.7% | 0.1% | -1.0% | 0.5% | -0.5% |
| Overall | -0.8% | -0.4% | 0.0% | -0.7% | 0.3% | 0.1% |
|  | -0.8% | -0.4% | -0.1% | -0.7% | 0.3% | 0.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |
| Mult[%] | -7% |  |  | -8% |  |  |
| Adds[%] | -10% |  |  | -11% |  |  |
| Access[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 11% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | -2% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 10% |  |  | 4% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6% | -0.2% | -0.1% | -1.2% | 0.1% | -0.4% |
| Class C | -0.9% | -0.3% | -0.5% | -0.8% | -0.1% | 0.1% |
| Class D | -0.6% | -0.2% | -0.5% | -0.6% | 0.4% | 0.1% |
| Class E | -1.4% | -0.3% | -0.4% | -3.0% | -1.4% | -1.1% |
| Overall | -0.8% | -0.2% | -0.4% | -1.3% | -0.2% | -0.3% |
|  | -0.8% | -0.3% | -0.4% | -1.3% | -0.2% | -0.4% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
| Mult[%] | 3% |  |  | 4% |  |  |
| Adds[%] | 2% |  |  | 2% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 7% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 7% |  |  |

| Gain Ave.[%] | -0.72% | -0.09% | -0.10% |
| --- | --- | --- | --- |
| Gain Min. [%] | -5.37% | -3.31% | -2.48% |
| Gain Max.[%] | 0.59% | 0.83% | 1.87% |
| Enc Time[%] | 102.68% |  |  |
| Dec Time[%] | 101.94% |  |  |
| Mult Ave.[%] | -5% |  |  |
| Mult Worst[%] | 0% |  |  |
| Adds Ave.[%] | -8% |  |  |
| Adds Worst[%] | 0% |  |  |
| Access[%] | 2% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  |
| MemBand(2D) Max.[%] | 5% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |