| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G821 |
| --- | --- |

| Title: | CE3: Cross-check for eBrisk proposals on fixed (JCTVC-G058) and adaptive (JCTVC-G063) interpolation filter by Samsung. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Elena Alshina JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.net jeonghoon@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

# Abstract

This is cross-check report for 2 contributions form eBrisk in CE3. The first is combination of different, but beforehand known interpolation filters. Longer filter is used for horizontal interpolation and shorter filter is applied for vertical. The second is adaptive interpolation filter. S/w was shared to all CE participants in time sufficient for analysis and cross-check. The main performance improvement in both proposals comes from LD(P) test. Gain typically higher for high resolution video.

# Interpolation filter with fixed coefficients

Access to the vertical neighbor pixels is more difficult than horizontal memory access. So in first proposal proponent uses shorter filter for vertical and longer filter for horizontal interpolation. Both filters are fixed (beforehand known coefficients).

Filter coefficients are

Horizontal interpolation uses 7Q(3/16)8H filter for Luma:
{-1,  3,   -8,  60,   13,  -4,   1,   0}

{-1,  4,  -11,  40,   40, -11,   4,  -1},

and synchronized with it Chroma filter:

{-2,   61,   6,  -1}
{-4,   54,  16,  -2}
{-5,   44,  29,  -4}
{-4,   36,  36,  -4}

This is variant B from joint CE3 contribution on solid interpolation filter [1].

Vertical interpolation uses 7Q(1/4)6H filter:
{-2,  5,  -11,  58,  18,   -6,   2,   0}
{ 0,  2,   -9,  39,   39,  -9,   2,   0}

and following Chroma filter:
{-2,   58,  10,  -2}
{-4,   54,  16,  -2}
{-6,   46,  28,  -4}
{-4,   36,  36,  -4}

This variant was also tested in CE3 as solid (non-switchable) interpolation filter [2].

This test was run under common test conditions [3] for RA-HE/LC, LD(B)-HE/LC and LD(P)-HE/LC according to [4]. Tests result summary is show in Table 1. Main gain comes from LD(P) test, which is mandatory for CE3 only. Average BD-rate performance across 4 cases in common test conditions RA-HE/LC, LD(B)-HE/LC is 0.0%(Y), 0.1%(U), 0.1%(V).

Proposed interpolation scheme reduce an average number of computations: 5% less multiplications and 7% less additions compare to anchor.

Shorter filter is implemented as 8 taps filter with zero coefficients, so encoding/decoding time is comparable with anchor (variations within 3%).

We need to emphasize that gain is higher for high resolution videos.

Table 1. Test results summary for interpolation filter scheme with fixed coefficients.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.1% | 0.6% | 0.5% |
| Class B | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.1% |
| Class C | 0.1% | 0.1% | 0.2% | 0.3% | 0.2% | 0.3% |
| Class D | 0.1% | 0.3% | 0.2% | 0.7% | 0.2% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
|  | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -7% |  |  | -7% |  |  |
| Access[%] | 1% |  |  | 0% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.4% | -0.2% | -0.4% | -0.4% | -0.4% |
| Class C | 0.0% | 0.1% | 0.1% | 0.1% | 0.5% | 0.2% |
| Class D | 0.5% | 0.1% | 0.6% | 0.7% | 0.5% | 0.3% |
| Class E | -0.4% | 0.4% | 0.3% | -0.8% | -0.1% | -0.5% |
| Overall | 0.0% | 0.0% | 0.2% | -0.1% | 0.1% | -0.1% |
|  | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -7% |  |  | -7% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.2% | -0.1% | -0.8% | -0.2% | -0.5% |
| Class C | -0.2% | 0.1% | 0.3% | -0.3% | 0.2% | 0.2% |
| Class D | 0.2% | -0.1% | -0.2% | 0.1% | 0.2% | 0.1% |
| Class E | -0.9% | -0.3% | -0.2% | -2.1% | -1.4% | -0.7% |
| Overall | -0.2% | -0.1% | 0.0% | -0.7% | -0.2% | -0.2% |
|  | -0.2% | -0.1% | -0.1% | -0.7% | -0.3% | -0.3% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
| Mult[%] | -5% |  |  | -5% |  |  |
| Adds[%] | -7% |  |  | -7% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |

| Gain Ave.[%] | -0.13% | 0.00% | 0.02% |
| --- | --- | --- | --- |
| Gain Min. [%] | -3.50% | -2.01% | -1.65% |
| Gain Max.[%] | 1.58% | 1.83% | 1.73% |
| Enc Time[%] | 100.27% |  |  |
| Dec Time[%] | 101.66% |  |  |
| Mult Ave.[%] | -5% |  |  |
| Mult Worst[%] | -5% |  |  |
| Adds Ave.[%] | -7% |  |  |
| Adds Worst[%] | -6% |  |  |
| Access[%] | 0% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |

# Adaptive interpolation filter

The same idea is tested in second proposal but with content adaptive interpolation filter:

Horizontal Luma filter is 7Q8H:
{a0,  a1,  a2,  a3,  a4,  a5,  a6}
{b0,  b1,  b2,  b3,  b3,  b2,  b1,   b0}

Vertical Luma filter is  7Q6H:
{c0,  c1,  c2,  c3,  c4,  c5,  c6}
{d0,  d1,  d2,  d2,  d1,   d0}

The magnitude of filter coefficients is represented with 6 bits precision (the sane with anchor) and signaled explicitly to the decoder.

This test was run under common test conditions [3] for RA-HE/LC, LD(B)-HE/LC and LD(P)-HE/LC according to [4]. Tests result summary is show in Table 2. Main gain again comes from LD(P) test, which is mandatory for CE3 only. Average BD-rate performance across 4 cases in common test conditions RA-HE/LC, LD(B)-HE/LC is -0.2%(Y), 0.1%(U), 0.0%(V) which is 0.2% better than for variant with fixed (non-adaptive) filter.

In this version an average number of multiplications is 7% less than anchor. An average number of additions is 10% less than for anchor. So in version with content adaptive coefficients of interpolation filter the number of computations is smaller. For s/w implementation this means less complexity. But in h/w implementation cost of multiplications with variable multipliers is higher than for fixed coefficients.

Shorter filter is implemented as 8 taps filter with zero coefficients, so decoding time is comparable with anchor (variations within 3%). In terms of encoding time we have found up-to 10% increment which is slightly higher than reported by proponent.

We need to emphasize that gain is higher for high resolution videos.

Table 2. Test results summary for adaptive interpolation filter

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9% | -1.1% | -1.5% | -2.2% | -0.1% | -0.8% |
| Class B | 0.0% | -0.2% | -0.3% | -0.3% | -0.3% | -0.5% |
| Class C | 0.1% | 0.0% | 0.1% | 0.1% | 0.1% | 0.0% |
| Class D | 0.2% | 0.4% | 0.4% | 0.6% | 0.0% | 0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.2% | -0.3% | -0.4% | -0.1% | -0.3% |
|  | -0.1% | -0.2% | -0.3% | -0.4% | 0.0% | -0.3% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | 101% |  |  | 103% |  |  |
| Mult[%] | -7% |  |  | -7% |  |  |
| Adds[%] | -9% |  |  | -10% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.4% | -0.2% | -1.0% | -0.4% | -0.5% |
| Class C | 0.0% | 0.2% | 0.2% | -0.3% | 0.6% | 0.3% |
| Class D | 0.8% | 0.6% | 0.5% | 0.6% | 0.7% | 0.4% |
| Class E | 0.8% | 1.4% | 1.7% | -1.6% | 1.0% | 0.5% |
| Overall | 0.3% | 0.3% | 0.4% | -0.6% | 0.4% | 0.1% |
|  | 0.4% | 0.3% | 0.3% | -0.5% | 0.3% | 0.1% |
| Enc Time[%] | 109% |  |  | 110% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
| Mult[%] | -8% |  |  | -7% |  |  |
| Adds[%] | -11% |  |  | -10% |  |  |
| Access[%] | 0% |  |  | 0% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7% | -1.2% | -1.2% | -3.7% | -1.2% | -1.9% |
| Class C | -0.4% | -0.1% | -0.1% | -1.1% | 0.2% | 0.1% |
| Class D | 0.2% | -0.2% | -0.5% | -0.2% | 0.2% | -0.1% |
| Class E | -0.7% | 0.2% | 0.4% | -6.2% | -1.7% | -1.1% |
| Overall | -0.4% | -0.4% | -0.4% | -2.7% | -0.6% | -0.8% |
|  | -0.4% | -0.4% | -0.5% | -2.7% | -0.6% | -0.9% |
| Enc Time[%] | 108% |  |  | 109% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |
| Mult[%] | -7% |  |  | -6% |  |  |
| Adds[%] | -9% |  |  | -9% |  |  |
| Access[%] | 1% |  |  | 1% |  |  |

.

| Gain Ave.[%] | -0.65% | -0.09% | -0.22% |
| --- | --- | --- | --- |
| Gain Min. [%] | -11.10% | -4.27% | -5.74% |
| Gain Max.[%] | 1.30% | 2.66% | 2.38% |
| Enc Time[%] | 107.93% |  |  |
| Dec Time[%] | 101.67% |  |  |
| Mult Ave.[%] | -7% |  |  |
| Mult Worst[%] | -5% |  |  |
| Adds Ave.[%] | -10% |  |  |
| Adds Worst[%] | -6% |  |  |
| Access[%] | 0% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |

# Conclusions

We confirm performance reported by eBrisk in CE3 tests for fixed interpolation filters combination:

average for 4 cases (RA-HE/LC, LD(B)-HE/LC):  0.0% (Y) 0.1%(U) 0.1%(V)

average for 6 cases (including LD(P)-HE/LC):     -0.1% (Y) 0.0%(U) 0.0%(V)

and for adaptive interpolation filter

average for 4 cases (RA-HE/LC, LD(B)-HE/LC):  -0.2% (Y) 0.1%(U) 0.0%(V)

average for 6 cases (including LD(P)-HE/LC):     -0.6% (Y) -0.1%(U) -0.2%(V).

# References

- Alexander Alshin, Elena Alshina, JeongHoon Park, Jian Lou, Koohyar Minoo “CE3: 7 taps interpolation filters for quarter pel position MC from Samsung and Motorola Mobility”, JCTVC-G778, Geneva, Switzerland, November 2011

- Alexander Alshin, Elena Alshina, JeongHoon Park “CE3: 7Q6H taps interpolation filters test by Samsung”, JCTVC-G775, Geneva, Switzerland, November 2011

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- T.Chujoh, E. Alshina, “CE3: interpolation MC”, JCTVC-F903, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011