| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G822 |
| --- | --- |

| Title: | CE1: Cross-check for delayed probability up-date from Qualcomm (JCTVC-G576) by Samsung |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Elena Alshina JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.net jeonghoon@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

# Abstract

This is cross-check report delayed probability up-date (JCTVC-576) from Qualcomm tested in CE1 (subtest A, test 1). Proposal is targeting to throughput improvement. Depends on delay 1, 2 or 3 bins provides 0.1%, 0.6%  and 1.0% performance drop in avg. across AI-HE,RA-HE and LD(B) –HE. Performance matches with reported by proponent.

# Introduction

S/w was shared to all CE participants in time sufficient for analysis.

Implementation matches with description provided in [1], [2]. Delay for probability up-date uniformly applied for all syntax elements. The context states when coding or decoding bin(n) should be dependent only on bin(0), bin(1), … , bin(n-L-1) for a delay of L bins. This allows decoding L bins in parallel. Of course additional memory is needed to store L delayed bins values.

# Test results

This test was run under common test conditions [3] for IA-HE, RA-HE, LD(B)-HE cases. Proposed probability up-date delay was tested against anchor which is HM4.0. We run 3 tests for 1, 2 and 3 bins delay.

All 3 tests result summary is show in Table 1. As was expected drop becomes higher with delay increment.

Table 1. Test results summary:

| Delay | Gain, Y | Gain, U | Gain, V |
| --- | --- | --- | --- |
| 1 bin | 0,1% | 0,1% | 0,2% |
| 2 bins | 0,6% | 0,4% | 0,5% |
| 3 bins | 1,0% | 0,4% | 0,7% |

There are more details in Table 2 and spread-sheets attached to this report. Typically drop is higher for high resolution video. We have found small increment of encoding or decoding time in all tests, but not critical (<5%).

Table 2. Test results summary.

| All Intra HE. Y; U; V. Class A; 0,2%; 0,3%; 0,4%. Class B; 0,1%; 0,2%; 0,3%. Class C; 0,1%; 0,2%; 0,2%. Class D; 0,1%; 0,2%; 0,1%. Class E; 0,1%; 0,3%; 0,2%. Overall; 0,1%; 0,2%; 0,2%. 0,1%; 0,2%; 0,2%. Enc T[%]; 103%. Dec T[%]; 102%. Random Access HE. Y; U; V. Class A; 0,2%; 0,3%; 0,1%. Class B; 0,1%; 0,3%; 0,1%. Class C; 0,1%; 0,1%; 0,1%. Class D; 0,1%; 0,1%; -0,2%. Class E. Overall; 0,1%; 0,2%; 0,0%. 0,1%; 0,1%; 0,0%. Enc T[%]; 101%. Dec T[%]; 101%. Low delay B HE. Y; U; V. Class A. Class B; 0,1%; -0,1%; -0,2%. Class C; 0,1%; 0,1%; 0,0%. Class D; 0,0%; 0,2%; 0,2%. Class E; 0,0%; 0,1%; 1,2%. Overall; 0,1%; 0,1%; 0,2%. 0,1%; 0,0%; 0,1%. Enc T[%]; 101%. Dec T[%]; 101%. | All Intra HE. Y; U; V. 0,3%; 0,2%; 0,2%. 0,2%; 0,2%; 0,3%. 0,1%; 0,2%; 0,2%. 0,1%; 0,2%; 0,2%. 0,3%; 0,3%; 0,3%. 0,2%; 0,2%; 0,2%. 0,2%; 0,2%; 0,2%. 105%. 104%. Random Access HE. Y; U; V. 1,0%; 0,7%; 0,0%. 0,9%; 0,5%; 0,5%. 0,7%; 0,5%; 0,5%. 0,5%; 0,6%; 0,2%. 0,8%; 0,6%; 0,3%. 0,8%; 0,6%; 0,3%. 102%. 101%. Low delay B HE. Y; U; V. 1,0%; 0,7%; 0,7%. 0,7%; 0,4%; 0,5%. 0,5%; 0,3%; 0,7%. 1,8%; 1,0%; 2,3%. 0,9%; 0,6%; 0,9%. 0,9%; 0,6%; 0,9%. 102%. 101%. | All Intra HE. Y; U; V. 0,3%; 0,3%; 0,6%. 0,2%; 0,2%; 0,6%. 0,2%; 0,2%; 0,3%. 0,2%; 0,2%; 0,3%. 0,3%; 0,4%; 0,9%. 0,2%; 0,3%; 0,5%. 0,2%; 0,3%; 0,6%. 105%. 105%. Random Access HE. Y; U; V. 1,4%; 0,5%; 0,4%. 1,3%; 0,4%; 0,5%. 0,9%; 0,3%; 0,7%. 0,8%; 0,4%; 0,4%. 1,1%; 0,4%; 0,5%. 1,1%; 0,4%; 0,5%. 104%. 100%. Low delay B HE. Y; U; V. 1,6%; 0,7%; 0,6%. 1,0%; 0,6%; 0,5%. 0,7%; 0,2%; 0,3%. 3,2%; 1,0%; 3,0%. 1,5%; 0,6%; 0,9%. 1,5%; 0,7%; 0,9%. 103%. 100%. |
| --- | --- | --- |

# Conclusions

We confirm performance reported by Qualcomm in CE1 test for delayed probability up-date:

0.1% drop for 1 bin delay,

0.6% drop for 2 bins delay,

1.0% drop for 3 bins delay.

# References

- J. Sole, R. Joshi and M. Karczewicz, “Parallel Processing of Residual Data in HE”, JCTVC-F552, Torino, IT, July 2011

- “Description of Core Experiment 1: Entropy Coding”, JCTVC-F901, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011