| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G825 |
| --- | --- |

| Title: | Cross-check for Motorola Mobility proposals on MV scaling (JCTVC-G699) by Samsung. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | report |  |  |
| Author(s) or Contact(s): | Elena Alshina Alexander Alshin JeongHoon Park #416 Matean 3-dong, Yeongtong-gu, Suwon-si, Gyeonggi-do, 443-742, Korea | Tel: Email: | +82 10 30261305 elena_a.alshina@samsung.net alexander_b.alshin@samsung.net jeonghoon@samsung.com |
| Source: | Samsung Electronics Ltd. |  |  |

# Abstract

This is cross-check report for Motorola’s contribution from Motorola on Mv scaling for non-uniform interpolation MC..

# Introduction

There are two for modification quarter-pel interpolation are studied in CE3: 7Q(1/4)8H and 7Q(3/16)8H [1]. First variant uses uni-form phase shift for filter coefficients generation. The second one uses “non-uniform” phase shift 3/16 instead 1/4. Here we verify performance test results on MV prediction modification for interpolation MC with non-uniform phase shift.

# Test Results

We run tests tested using test conditions [2] for RA-HE/LC, LD(B)-HE/LC and LD(P)-HE/LC according to [3]. Because this modification is applied only for interpolation MC with non-uniform phase shift this is reasonable to report performance change compare to true anchor (which is 7Q(3/16)8H from [1] in this case). We also report performance change from both modifications together: 7Q(3/16)8H and modified MV prediction.

Summary of our test is shown in Table 1. All values are averaged across only common 4 test cases RA-HE/LC, LD(B)-HE/LC in Table 1. There is no performance improvement from any of tested tool under common test conditions. But there is increment of computational complexity due to higher hit-ration of fractional position in MC which we observe if non-uniform phase shift is used [4].

Table 1. Summary of tests averaged across RA-HE/LC, LD(B)-HE/LC.

| Filter | BD-rate Y | BD-rate U | BD-rate V | mults | adds |
| --- | --- | --- | --- | --- | --- |
| MV scaling vs 7Q(3/16)8H | 0.0 % | 0.0 % | 0.0 % | 1% | 2% |
| 7Q(3/16)8H vs HM4.0 | 0.0 % | 0.2 % | 0.2 % | 1% | 0% |
| 7Q(3/16)8H and MV scaling vs HM4.0 | 0.0 % | 0.2 % | 0.2 % | 3% | 2% |

In Table 2 we report an average performance/ complexity data across 6 test cases: RA-HE/LC, LD(B)-HE/LC, LD(P)-HE/LC. So this is clear that “non-uniform” phase shift in interpolation filter design for MC helps LD(P) test only.

Table 2. Summary of tests averaged across RA-HE/LC, LD(B)-HE/LC, LD(P)-HE/LC.

| Filter | BD-rate Y | BD-rate U | BD-rate V | mults | adds |
| --- | --- | --- | --- | --- | --- |
| MV scaling vs 7Q(3/16)8H | 0.0 % | 0.0 % | 0.0 % | 1% | 1% |
| 7Q(3/16)8H vs HM4.0 | -0.4 % | 0.0 % | 0.0 % | 2% | 1% |
| 7Q(3/16)8H and MV scaling vs HM4.0 | -0.4 % | 0.0 % | 0.0 % | 3% | 2% |

More details about test results can be found in Tables 3-4 (Appendix) and spread-sheet attached to this report..

# Conclusions

We confirm performance reported by Motorola on MV prediction change adjusted to 7Q(3/16)8H interpolation filter with non-uniform phase shift:

0.0%(Y) 0.0%(U) 0.0%(V) BD-rate change;

with 1% increment for number of multiplications;

and 1-2% increment for number of additions.

.

# References

- Alexander Alshin, Elena Alshina, JeongHoon Park, Jian Lou, Koohyar Minoo “CE3: 7 taps interpolation filters for quarter pel position MC from Samsung and Motorola Mobility”, JCTVC-G778, Geneva, Switzerland, November 2011

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- T.Chujoh, E. Alshina, “CE3: interpolation MC”, JCTVC-F903, 6th JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

- E. Alshina, “CE3: Cross-check by Samsung for Motorola Mobility tests on interpolation filters (JCTVC-F574, JCTVC-F576)”, JCTVC-F443 JCT-VC Meeting, Torino, Italy, 14-22 July, 2011

APPENDIX

Table 3. Test results for MV scaling compare to 7Q(3/16)8H

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% | 0.1% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 2% |  |  | 2% |  |  |
| Adds[%] | 2% |  |  | 2% |  |  |
| Access[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(2D) Max.[%] | 4% |  |  | 6% |  |  |
| MemBand(1D) Ave.[%] | -1% |  |  | -1% |  |  |
| MemBand(1D) Max.[%] | 3% |  |  | 5% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.2% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Class D | -0.1% | -0.2% | -0.3% | 0.0% | 0.3% | -0.2% |
| Class E | 0.0% | 0.4% | 0.3% | -0.1% | 0.3% | 0.5% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
| Mult[%] | 1% |  |  | 1% |  |  |
| Adds[%] | 1% |  |  | 1% |  |  |
| Access[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 12% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 11% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.1% | 0.1% | -0.1% | -0.1% | 0.0% |
| Class C | -0.1% | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% |
| Class D | -0.1% | -0.2% | -0.3% | -0.1% | -0.1% | 0.4% |
| Class E | 0.0% | 0.2% | 0.5% | 0.0% | 0.3% | -0.3% |
| Overall | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | 0.0% |
|  | 0.0% | 0.0% | -0.1% | -0.1% | -0.1% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
| Mult[%] | 1% |  |  | 1% |  |  |
| Adds[%] | 1% |  |  | 1% |  |  |
| Access[%] | 0% |  |  | 1% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  | 4% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 0% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 4% |  |  |

| Gain Ave.[%] | -0.03% | 0.01% | 0.01% |
| --- | --- | --- | --- |
| Gain Min. [%] | -0.29% | -0.74% | -1.37% |
| Gain Max.[%] | 0.36% | 1.52% | 0.91% |
| Enc Time[%] | 101.30% |  |  |
| Dec Time[%] | 100.22% |  |  |
| Mult Ave.[%] | 1% |  |  |
| Mult Worst[%] | 0% |  |  |
| Adds Ave.[%] | 1% |  |  |
| Adds Worst[%] | 0% |  |  |
| Access[%] | 1% |  |  |
| MemBand(2D) Ave.[%] | 0% |  |  |
| MemBand(2D) Max.[%] | 6% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |

Table 4. Test results for MV scaling compare together 7Q(3/16)8H compare to HM4.0

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | 0.0% | -0.1% | -0.3% | 0.2% | 0.3% |
| Class B | -0.2% | 0.0% | 0.0% | -0.3% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.1% | 0.4% | 0.2% | 0.3% |
| Class D | 0.2% | 0.4% | 0.2% | 1.0% | 0.4% | 0.5% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% | 0.3% |
|  | 0.0% | 0.1% | 0.1% | 0.2% | 0.2% | 0.3% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |
| Mult[%] | 2% |  |  | 2% |  |  |
| Adds[%] | 1% |  |  | 1% |  |  |
| Access[%] | 4% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 4% |  |  | 8% |  |  |
| MemBand(1D) Ave.[%] | 0% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 5% |  |  | 7% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | 0.1% | 0.0% | -0.5% | 0.2% | 0.2% |
| Class C | -0.1% | 0.2% | 0.3% | 0.1% | 0.4% | 0.4% |
| Class D | 0.3% | 0.4% | 0.5% | 0.7% | 1.1% | 0.6% |
| Class E | -0.5% | 0.1% | 0.3% | -1.1% | 0.1% | -0.6% |
| Overall | -0.1% | 0.2% | 0.2% | -0.2% | 0.5% | 0.2% |
|  | -0.1% | 0.1% | 0.2% | -0.2% | 0.4% | 0.2% |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
| Mult[%] | 4% |  |  | 4% |  |  |
| Adds[%] | 3% |  |  | 3% |  |  |
| Access[%] | 5% |  |  | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(2D) Max.[%] | 12% |  |  | 9% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 1% |  |  |
| MemBand(1D) Max.[%] | 11% |  |  | 9% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.7% | -0.3% | -0.2% | -1.2% | -0.1% | -0.3% |
| Class C | -1.0% | -0.3% | -0.4% | -0.9% | -0.2% | -0.1% |
| Class D | -0.7% | -0.5% | -0.5% | -0.7% | 0.0% | 0.3% |
| Class E | -1.4% | -0.6% | -0.1% | -2.9% | -1.2% | -1.4% |
| Overall | -0.9% | -0.4% | -0.3% | -1.3% | -0.3% | -0.3% |
|  | -0.9% | -0.4% | -0.4% | -1.3% | -0.3% | -0.4% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
| Mult[%] | 4% |  |  | 4% |  |  |
| Adds[%] | 3% |  |  | 3% |  |  |
| Access[%] | 5% |  |  | 5% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  | 2% |  |  |
| MemBand(2D) Max.[%] | 7% |  |  | 8% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  | 2% |  |  |
| MemBand(1D) Max.[%] | 6% |  |  | 8% |  |  |

| Gain Ave.[%] | -0.39% | 0.04% | 0.02% |
| --- | --- | --- | --- |
| Gain Min. [%] | -4.89% | -2.54% | -1.82% |
| Gain Max.[%] | 2.57% | 2.42% | 1.54% |
| Enc Time[%] | 101.95% |  |  |
| Dec Time[%] | 100.95% |  |  |
| Mult Ave.[%] | 3% |  |  |
| Mult Worst[%] | 0% |  |  |
| Adds Ave.[%] | 2% |  |  |
| Adds Worst[%] | 0% |  |  |
| Access[%] | 4% |  |  |
| MemBand(2D) Ave.[%] | 1% |  |  |
| MemBand(2D) Max.[%] | 8% |  |  |
| MemBand(2D) Worst[%] | 0% |  |  |
| MemBand(1D) Ave.[%] | 1% |  |  |
| MemBand(1D) Max.[%] | 8% |  |  |
| MemBand(1D) Worst[%] | 0% |  |  |