| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G826 |
| --- | --- |

| Title: | Non-CE4: Crosscheck report of Quantization matrix compression and signaling by Qualcomm (JCTVC-G578) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami Junichi Tanaka Teruhiko Suzuki 2-10-1, Osaki, Shinagawa-ku , Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2740 Yoshitaka.Morigami@jp.sony.com Tanaka.Junichi@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports the cross check result of JCTVC-G578. Compare with AVC method, it was confirmed that the proposal can reduce bitrate upto 25%. In addition, the source code provided by the proponent was investigated. The description in the contribution and the software implementation is consistent.

# Proposed method

The proposal inJCTVC-F578 includes the following tools,

- A simple raster scan

- matrix coefficient of above and left matrix value is used as reference matrix value

- Symbol number is remapped

# Simulation Results

In test, only the coding performance of matrix itself is measured, as test 1 in CE4. The proposal is compared with AVC method. The test results are summarized in the following table. The detailed results are shown in the attached excel-file.

|  |  |  | Symmetric quant matrix |  |  | Asymmetric quant matrix |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Down-sample | Sym45 | Sym135 | Bits (AVC) | Bits (proposed) | Avg. abs error | Bits (AVC) | Bits (proposed) | Avg. abs error |
| 0 | 0 | 0 | 8856 | 7171 (81%) | 0 | 9512 | 6450 (68%) | 0 |
| 0 | 0 | 1 | 5020 | 3828 (76%) | 0 | 5056 | 3503 (69%) | 3.53 |
| 0 | 1 | 0 | 5052 | 4201 (83%) | 5.06 | 4558 | 3552 (78%) | 3.14 |
| 0 | 1 | 1 | 2861 | 2305 (81%) | 5.06 | 2626 | 2016 (77%) | 5.25 |
| 1 | 0 | 0 | 3631 | 2889 (80%) | 0.43 | 3784 | 2539 (67%) | 0.32 |
| 1 | 0 | 1 | 2339 | 1645 (70%) | 0.43 | 2323 | 1501 (65%) | 3.76 |
| 1 | 1 | 0 | 2263 | 1858 (82%) | 4.73 | 1923 | 1472 (77%) | 3.54 |
| 1 | 1 | 1 | 1477 | 1120 (76%) | 4.73 | 1249 | 955 (76%) | 5.52 |

Table 1: Result of “no dither” test case

# Conclusion

This contribution reports the cross check result of JCTVC-G578 (proposed by Qualcomm). Compare with AVC method, It was confirmed that the proposal can reduce bitrate upto 25%. In addition, the source code provided by the proponent was investigated. The description in the contribution and the software implementation is consistent.

# Patent rights declaration(s)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).