| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G828 |
| --- | --- |

| Title: | Non-CE8: Crosscheck for Canon's additional SAO band offset classifications in JCTVC-G246 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chih-Ming Fu, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {chihming.fu, yuwen.huang }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the modified SAO band offset proposed by Canon in JCTVC-G246. The pixel classification method for band offset (BP) in SAO is modified by adding new sub-bands. The verification task was done successfully, and the results match exactly those provided by Canon. The decoding time increase is 0-0.1%, while the bit rate reduction is 0.7-2.7% for chroma.

# Verification

Canon provided the HM source codes of their proposed codec described in JCTVC-G246 [1]. The verification is carried out by setting the following parameters in the provided codes:

#define CANON_N_CLASSES_SUBDIV_N        1

#define CANON_RD_OPT                      1

Eight conditions (intra HE, intra LC, random access HE, random access LC, low delay B HE, low delay B LC, low delay P HE, low delay P LC) without class F were run for the modified SAO. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

# Results

The BD-rates from our experimental results match exactly those provided by Canon. The detailed results are listed in JCTVC-G828_Experiment_I.xls and JCTVC-G828_ Experiment_II.xls.

# Conclusion

The results presented by Canon in JCTVC-G246 are confirmed.

# References

- G. Laroche, T. Poirier, P. Onno, “On additional SAO band offset classifications,” Document of Joint Collaborative Team on Video Coding, JCTVC-G246, November 2011.