| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G829 |
| --- | --- |

| Title: | Context modeling of split flag for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | +1-858-845-1795 wchien@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

In this proposal,  the coding of  split_coding_unit_flag coding is investigated. The contribution proposes a context selection model of 4 contexts based on the coding unit depth of what the current split_coding_unit_flag coding represents and the coding unit depth of the coding unit on the left. Experimental results reportedly show 0.0%, 0.1% and 0.1% BD-rate gain in high efficiency intra-only, random access and low-delay test conditions respectively.

# Introduction

In HM4.0, the current design of context selection uses the spatial correlation of neighboring data for split_coding_unit_flag coding and skip flag. A line buffer is required in parsing engine to store information. The size of line buffer depends on the width of source images, so it could not be overlooked with 4Kx2K sequences. The contribution proposes a context selection model of 4 contexts to code split_coding_unit_flag to obtain a more balanced solution between the number of the contexts and coding performance, which is addressed in CE1.A.3[1].

# Proposed Solution

The proposed solution uses 4 contexts for split_coding_unit_flag coding. The selection is based on the current CU depth and the left CU depth.  Table 1 and 2 show the table of context selection when the left CU is available or not.

| ctxIdx | CUDepth=0 | CUDepth=1 | CUDepth=2 |
| --- | --- | --- | --- |
| LeftCUDepth=0 | 0 | 1 | 1 |
| LeftCUDepth=1 | 3 | 2 | 1 |
| LeftCUDepth=2 | 3 | 3 | 1 |
| LeftCUDepth=3 | 3 | 3 | 2 |

Table 1. Context index table for the case left CU is available.

| ctxIdx | HM4.0 |
| --- | --- |
| CUDepth=0 | 1 |
| CUDepth=1 | 1 |
| CUDepth=2 | 2 |

Table 2. Context index table for the case left CU is not available.

# Simulation Results

This method is implemented in HM4.0. Simulation results are presented for the high efficiency common test conditions [1]. Table 3 shows the results for this method. The BD-rate is 0.0℅, 0.0℅ and 0.1℅ for AI, RA and LB configurations, respectively.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |  |  |  |
| Class B | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | -0.1% | 0.1% | -0.1% | 0.2% | 0.1% | 0.0% | -0.1% |
| Class D | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% | -0.1% | 0.0% | 0.3% | -0.4% |
| Class E | 0.1% | -0.1% | -0.1% |  |  |  | 0.3% | -0.9% | 0.9% |
| All | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% |
| Enc T[%] | 96% |  |  | 96% |  |  | 99% |  |  |
| Dec T[%] | 99% |  |  | 100% |  |  | 98% |  |  |

Table 3. Coding performance of the proposed solution.

# Conclusions

Comparing to HM4.0, the number of the context increases by one in this contribution; however, the line buffer of CU depth upper boundary is no longer needed. Compared to [3] and [4], the proposed solution is a more balanced solution considering the context numbers and the coding performance. Based on the results, we suggest including this modification into HM.

# References

[1]  R. Joshi, E. Alshina, H. Sasai, H. Kirchhoffer, J. Lainema, “Description of Core Experiment 1: Entropy Coding”, JCTVC-F901, Torino, July, 2011.

[2]  F. Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July, 2011.

[3] T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S. Lei , “CE1.A.3: Reducing line buffers for CABAC”, JCTVC-G200, Nov., 2011.

[4] T. Lee, J. Chen, J. H. Park, “CE1 subtest A: Line buffer removal for CU split flag context model”, JCTVC-G769, Nov., 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).