#### Sample Adaptive Offset

| sao_param( ) { | Descriptor |
| --- | --- |
| sample_adaptive_offset_flag | u(1) |
| if ( sample_adaptive_offset_flag ) { |  |
| sao_LCU_param( 0 ) |  |
| sao_flag_cb | u(1) ¦ ae(v) |
| if( sao_flag_cb ) { |  |
| sao_LCU_param( 1 ) |  |
| } |  |
| sao_flag_cr | u(1) ¦ ae(v) |
| if( sao_flag_cr ) { |  |
| sao_LCU_param( 2 ) |  |
| } |  |
| } |  |
| } |  |

| sao_LCU_param(cIdx ) { | C | Descriptor |
| --- | --- | --- |
| sao_picture_level_flag | 2 | u(1)¦ae(v) |
| If(sao_picture_level_flag) { |  |  |
| sao_offset_param(cIdx); |  |  |
| } else { |  |  |
| CurrTbAddr = first_tb_in_slice |  |  |
| do { |  |  |
| xCU = HorLocation (CurrTbAddr) |  |  |
| yCU = VerLocation (CurrTbAddr) |  |  |
| if (xCU == left_most_tb_in_pic) { |  |  |
| sao_run_num = 0 |  |  |
| sao_repeat_row_flag = 0 |  |  |
| if (yCU != top_tb_in_pic) { |  |  |
| sao_repeat_row_flag | 2 | u(1)¦ae(v) |
| } |  |  |
| } |  |  |
| if ( !sao_repeat_row_flag ) { |  |  |
| if (sao_run_num!=0) { |  |  |
| sao_run_num -- |  |  |
| } else { |  |  |
| if (yCU == top_most_tb_in_pic) { |  |  |
| if (xCU == left_most_tb_in_pic) { |  |  |
| sao_run_num_max = NumLcuInWidth - 1 |  |  |
| } |  |  |
| sao_run_num [cIdx] [ xCU ][ yCU ] | 2 | ue(v)¦ae(v) |
| sao_run_num_max - = (sao_run_num + 1) |  |  |
| sao_offset_param ( xCU, yCU, cIdx ) |  |  |
| } else { |  |  |
| sao_merge_above_flag | 2 | u(1)¦ae(v) |
| sao_run_num_diff [cIdx] [ xCU][ yCU ] | 2 | se(v)¦ae(v) |
| if (sao_merge_above_flag == 0) |  |  |
| sao_offset_param (xCU, yCU, cIdx) |  |  |
| } |  |  |
| sao_run_num[cIdx][xCU][yCU]=sao_run_num_diff+sao_run_num[cIdx] [xCU][yCU-1] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| CurrTbAddr = NextTbAddress(CurrTbAddr) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

| sao_offset_param (xCU, yCU,cIdx) { | C | Descriptor |
| --- | --- | --- |
| sao_type_idx [cIdx][xCU][yCU] | 2 | ue(v)¦ae(v) |
| if( sao_type_idx != 0 ) { |  |  |
| for( i = 0; i < SaoOffsetNum[ sao_type_idx ]; i++ ) |  |  |
| sao_offset [cIdx] [xCU][yCU] [ i ] | 2 | se(v)¦ae(v) |
| } |  |  |

#### Sample adaptive offset parameter semantics

sample_adaptive_offset_flag specifies whether sample adaptive offset applies or not for the current picture.

sao_flag_cb equal to 1 denotes sample adaptive offset process for Cb shall be applied to the current picture.

sao_flag_cr equal to 1 denotes sample adaptive offset process for Cr shall be applied to the current picture.

sao_picture_level_flag equal to 1 denotes all LCUs in current picture share same SAO parameter; equal to 0 denotes that LCU-based syntax is applied.

sao_repeat_row_flag equal to 0 denotes current LCU row does not repeat the SAO parameters with those of previous LCU row; equal to 1 denotes that current LCU row repeats the SAO parameters with those of previous LCU row.

sao_run_num indicates the number of following LCUs to be applied with the SAO parameters of current LCU.

sao_run_num_diff indicates the difference between the sao_run_num of current LCU and the sao_run_num of the above LCU.

sao_merge_above_flag equal to 1 denotes current LCU uses the SAO parameters of the above LCU; equal to 0 denotes that current LCU has new SAO parameters including sao_type_idx and sao_offset.

sao_type_idx [cIdx][xCU][yCU] indicates the offset type to be applied in the LCU at location xCU and yCU of current slice.

sao_offset [cIdx] [xCU][yCU] [ i ] indicates the i-th category offset value to be applied in the LCU at location xCU and yCU of current slice. The sao_offset number of each offset type, nSaoLength [sao_type_idx], is specified in Table 7-7.

The variable sao_run_num [cIdx] [ xCU ][ yCU ] represents the number of following LCU will share the same SAO parameters in location (xCU, yCU).

If sao_picture_level _flag is equal to 1,

sao_type_idx [cIdx][0][0] and sao_offset [cIdx][0][0] [ i ] with i = 0.. (nSaoLength [sao_type_idx]-1) are decoded from bitstream and,

sao_type_idx [cIdx][xCU][yCU] = sao_type_idx [cIdx][0][0] with xCU = 0.. (NumLcuInWidth-1) and yCU = 0..(NumLcuInHeight-1)

sao_offset [cIdx][xCU][yCU] [ i ] = sao_offset [cIdx][0][0] [ i ] with i = 0..(nSaoLength [sao_type_idx]-1), and xCU = 0.. (NumLcuInWidth-1) and yCU = 0..(NumLcuInHeight-1).

Otherwise, if sao_repeat_row_flag is equal to 1

sao_run_num [cIdx][ xCU][ yCU ] = sao_run_num [cIdx][ xCU ][ yCU -1] with xCU = 0..(NumLcuInWidth-1),

sao_type_idx [cIdx][xCU][yCU] = sao_type_idx [cIdx][xCU][yCU-1] with xCU = 0..(NumLcuInWidth-1), and

sao_offset [cIdx][xCU][yCU][ i ] = sao_offset [cIdx][xCU-1][yCU-1] [ i ] with i = 0..(nSaoLength[sao_type_idx]-1), and xCU = 0..(NumLcuInWidth-1)

Otherwise, if sao_run_num [cIdx][ xCU ][ yCU ] is not equal to 0,

sao_run_num [cIdx][ xCU][ yCU ] = sao_run_num [cIdx][ xCU-1 ][ yCU ]-1,

sao_type_idx [cIdx][xCU][yCU] = sao_type_idx [cIdx][xCU-1][yCU], and

sao_offset [cIdx][xCU][yCU] [ i ] = sao_offset [cIdx][xCU-1][yCU] [ i ] with  i = 0..(nSaoLength[sao_type_idx]-1),

Otherwise, if sao_merge_above_flag is equal to 1 and yCU is not equal to zero,

sao_run_num [cIdx][ xCU][ yCU ] = sao_run_num [cIdx][ xCU ][ yCU -1] + sao_run_num_diff  [cIdx][ xCU][ yCU ],

sao_type_idx [cIdx][xCU][yCU] = sao_type_idx [cIdx][xCU][yCU-1], and

sao_offset [cIdx][xCU][yCU] [ i ] = sao_offset [cIdx][xCU-1][yCU-1] [ i ] with  i = 0..(nSaoLength[sao_type_idx]-1)

Otherwise the sao_type_idx [cIdx][xCU][yCU] and sao_offset [cIdx][xCU][yCU] [ i ] with i = 0..(nSaoLength[sao_type_idx]-1) are decoded from bitstream.

If sao_type_idx [cIdx][xCU][yCU] is equal to 7 or 8, sao_offset [cIdx][xCU][yCU] [ i ] is modified as follows.

If yCU is equal to 0,

sao_offset [cIdx][xCU][yCU] [ i ] = sao_offset [cIdx][xCU][yCU] [ i ] + sao_offset [cIdx][xCU-1][yCU] [ i ]

with  i = 0..(nSaoLength[sao_type_idx]-1)

Else

sao_offset [cIdx][xCU][yCU] [ i ] = sao_offset [cIdx][xCU][yCU] [ i ] + sao_offset [cIdx][xCU][yCU-1] [ i ]

with  i = 0..(nSaoLength[sao_type_idx]-1)

Table 7-1 – Specification of sao_type_idx

| - sao_type_idx | - sample adaptive offset type to be used | - Number of categories, nSaoLength [sao_type_idx] |
| --- | --- | --- |
| - 0 | - None | - 0 |
| - 1 | - 1-D 0-degree pattern edge offset | - 4 |
| - 2 | - 1-D 90-degree pattern edge offset | - 4 |
| - 3 | - 1-D 135-degree pattern edge offset | - 4 |
| - 4 | - 1-D 45-degree pattern edge offset | - 4 |
| - 5 | - central bands band offset | - 16 |
| - 6 | - side bands band offset | - 16 |
| - 7 | - predicted central bands band offset | - 16 |
| - 8 | - predicted side bands band offset | - 16 |

### -

    - -