| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G831 |
| --- | --- |

| Title: | Non-CE8: Sample Adaptive Offset with LCU-based Syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen1 Chih-Ming Fu1 Chia-Yang Tsai1 Yu-Wen Huang1 Shawmin Lei1 Marta Karczewicz2 In Suk Chong2 | Email: | chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com |
| Source: | MediaTek Inc.1and Qualcomm2 |  |  |

_____________________________

# Abstract

In HM-4.0, sample adaptive offset (SAO) parameters are coded for each region in a picture. In order to support localization of SAO parameters with higher flexibility, this contribution proposed a new syntax that allowed SAO parameters to be adaptively changed at any largest coding unit (LCU). Simulation results reportedly showed that the proposed syntax caused x%, 0.1%, 0.1%, 0.1%, x%, 0.1%, 0.2% and 0.2% bit rate increases for HE-AI, HE-RA, HE-LB, HE-LP, LC-AI, LC-RA, LC-LB, and LC-LP, respectively, with almost the same encoding and decoding times when the algorithm of deriving localized SAO parameters was unchanged.

- Introduction

In SAO, pixel classification of a selected SAO type is first done to classify pixels into different groups. Then, one offset is added for each pixel. In HM-4.0, the SAO uses a region-based syntax design. The region-based syntax is mainly designed for one picture partitioning algorithm. However, it is desirable to develop a simple syntax that can support many different picture partitioning algorithms for SAO optimization. In order to support localization of SAO with higher flexibility, a new syntax is proposed to allow SAO parameters to be changed at each LCU. Originally similar proposal was proposed in Torino [1], but in this proposal LCU based syntax is applied both luma and chroma.The rest of this contribution is organized as follows. The proposed algorithm will be introduced in Section 2, the syntax will be described in Section 3, simulation results will be shown in Section 4, and conclusions will be given in Section 5.

# Algorithm description

In the beginning, a first shortcut flag is used to allow disabling localization. The shortcut flag can switch between picture-based SAO and LCU-based SAO. As shown in Figure 1, in the picture-based SAO, the entire picture shares one SAO parameter set (SAOP), where the SAOP includes one SAO type and offsets of the selected SAO type. In the LCU-based SAO, each LCU has its own SAOP. Each LCU’s SAOP can be shared by following LCUs with a run signal where the run means the number of following LCUs that use the current LCU’s SAOP. Figure 2 shows an example. The SAOP of the first LCU is SAOP1, and SAOP1 is used for the next two LCUs. In this case, a run equal to two will be encoded. Because the next two LCUs share the SAOP with the first LCU, the rate of encoding their SAOPs can be saved.

Figure 1.  Switch between picture-based SAO and LCU-based SAO.

Figure 2.  Illustration of run coding.

In addition to a run for following LCUs in the raster scan order to copy the SAOP of a current LCU, we also design a merge-above flag to let the current LCU copy the SAOP of its above LCU if the above LCU is available. If the merge-above flag is equal to one, the current LCU will use the SAOP of the above LCU. As shown in Figure 2, SAOP2 is shared by four LCUs. The fourth LCU on the second row has a merge-above flag equal to one.

In order to reduce the bitrates of runs, run prediction is performed. We use the run of the above LCU as the predictor for the run of the current LCU and encode the difference of runs, which is denoted as d_run, as shown in Figure 2. For LCUs on the first row, we directly encode runs without prediction.

To further improve coding efficiency, a second shortcut flag is used for each non-first LCU row to repeat the SAOPs of its above LCU row, i.e. each LCU of one LCU row uses the SAOP of its above LCU.

To reduce the bits needed to send offset values, offset values are predicted from either its above LCU or left LCU, especially for band offset values which have 16 offset values to be sent. Left LCU is used for the first row of LCUs whose above LCUs are always not available. Otherwise, above LCU is used for prediction. In these cases, only residual values of offsets are transmitted to the decoder.

# Syntax Description

The proposed SAO syntax is shown in Table 1 and Table 2. Modifications are highlighted with yellow backgrounds.

Table 1.  The proposed SAO syntax.

| sao_param( ) { | C | Descriptor |
| --- | --- | --- |
| sao_flag | 2 | u(1)¦ae(v) |
| if( sao_flag ) { |  |  |
| sao_picture_level_flag | 2 | u(1)¦ae(v) |
| if( sao_picture_level_flag ) { |  |  |
| sao_offset_param (0, 0); |  |  |
| } else { |  |  |
| CurrTbAddr = first_tb_in_slice |  |  |
| do { |  |  |
| xCU = HorLumaLocation (CurrTbAddr) |  |  |
| yCU = VerLumaLocation (CurrTbAddr) |  |  |
| if( xCU == left_most_tb_in_pic ) { |  |  |
| sao_run_num = 0 |  |  |
| sao_repeat_row_flag = 0 |  |  |
| if( yCU != top_tb_in_pic ) { |  |  |
| sao_repeat_row_flag | 2 | u(1)¦ae(v) |
| } |  |  |
| } |  |  |
| if ( !sao_repeat_row_flag ) |  |  |
| if( sao_run_num != 0 ) { |  |  |
| sao_run_num -- |  |  |
| } else { |  |  |
| if( yCU == top_most_tb_in_pic ) { |  |  |
| if( xCU == left_most_tb_in_pic ) { |  |  |
| sao_run_num_max = NumLcuInWidth - 1 |  |  |
| } |  |  |
| sao_run_num [ xCU ] [ yCU ] | 2 | u(v)¦ue(v)¦ae(v) |
| sao_run_num_max - = (sao_run_num + 1) |  |  |
| sao_offset_param (xCU, yCU) |  |  |
| } else { |  |  |
| sao_merge_above_flag | 2 | u(1)¦ae(v) |
| sao_run_num_diff | 2 | se(v)¦ae(v) |
| if( sao_merge_above_flag == 0) |  |  |
| sao_offset_param (xCU, yCU) |  |  |
| } |  |  |
| sao_run_num [xCU] [yCU] = sao_run_num_diff + sao_run_num [xCU] [yCU-1] |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| CurrTbAddr = NextTbAddress(CurrTbAddr) |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

sao_flag equal to 0 denotes that SAO for luma is disabled for the entire picture; equal to 1 denotes that SAO for luma is enabled for at least a portion of the current picture.

sao_picture_level_flag equal to 1 denotes the picture-based SAO is used; equal to 0 denotes the LCU-based SAO is used.

sao_repeat_row_flag equal to 0 denotes the current LCU row does not repeat the SAO parameters of the previous LCU row; equal to 1 denotes that current LCU row repeats the SAO parameters of the previous LCU row.

sao_run_num indicates the number of following LCUs to be applied with the SAO parameters of the current LCU.

sao_run_num_diff indicates the difference between the sao_run_num of the current LCU and the sao_run_num of the above LCU.

sao_merge_above_flag equal to 1 denotes the current LCU uses the SAO parameters of the above LCU; equal to 0 denotes that the current LCU has new SAO parameters.

Table 2.  Syntax of SAO type and offsets.

| sao_offset_param (xCU, yCU) { | C | Descriptor |
| --- | --- | --- |
| sao_type_idx [xCU][yCU] | 2 | ue(v)¦ae(v) |
| if( sao_type_idx[xCU][yCU] != 0 ) { |  |  |
| for( i = 0; i < SaoOffsetNum[ sao_type_idx [xCU][yCU] ]; i++ ) |  |  |
| sao_offset [xCU] [yCU] [ i ] | 2 | se(v)¦ae(v) |
| } |  |  |

sao_type_idx indicates the SAO type to be applied for the current LCU. Two more SAO types are added, i.e., for predicted BO0 and predicted BO1.

sao_offset indicates the offset value to be applied for the current LCU. When predicted BO0/1 are chosen, these offset values are residual of the prediction.

# Simulation results

The software was based on HM-4.0. Table 3 shows the simulation results of using the same algorithm of deriving localized SAO parameters with the proposed syntax. It is reported the proposed syntax causes x%, 0.1%, 0.1%, 0.1%, x%, 0.1%, 0.2% and 0.2% bit rate increases for HE-AI, HE-RA, HE-LB, HE-LP, LC-AI, LC-RA, LC-LB, and LC-LP, respectively, with roughly unchanged encoding and decoding times, when the localization algorithm of SAO parameters in HM-4.0 is used.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class B | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class C | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
|  | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Enc Time[%] | 101% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class E | 0.3% | 0.3% | 0.3% | 0.5% | 0.5% | 0.5% |
| Overall | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
|  | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 99% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class C | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Class D | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% | 0.1% |
| Class E | 0.4% | 0.4% | 0.4% | 0.5% | 0.5% | 0.5% |
| Overall | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
|  | 0.1% | 0.1% | 0.1% | 0.2% | 0.2% | 0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution presented a new syntax that allows SAO parameters to be adaptively changed at each LCU in order to support localization of SAO parameters with higher flexibility. Simulation results reportedly showed that the proposed syntax caused x%, 0.1%, 0.1%, 0.1%, x%, 0.1%, 0.2%, and 0.2% bit rate increases for HE-AI, HE-RA, HE-LB, HE-LP, LC-AI, LC-RA, LC-LB, and LC-LP, respectively, with roughly unchanged encoding and decoding times when the localization algorithm in HM-4.0 was still used.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- C.-M. Fu, C.-Y. Chen, C-Y Tsai, Y.-W. Huang, S. Lei, I. S. Chong, M. Karczewicz, “Sample Adaptive Offset with LCU-based Syntax” Document of Joint Collaborative Team on Video Coding, JCTVC-F056, Jul. 2011.