| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G833 |
| --- | --- |

| Title: | Non-CE1: Adaptive initialization for CABAC with fixed probability contexts |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Joel Sole Xianglin Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes an adaptive initialization method for CABAC with fixed probability contexts. At the beginning of each slice, a context can be initialized in one of two ways: 1) initialized using the pre-defined (m,n) value (i.e., original HM method), or 2) initialized using a state value selected by the encoder. For each context, the encoder signals the selection of initialization method to the decoder as well as the new state value (if the second method is selected). No probability/state update will be performed in the encoding/decoding process, and thus the proposed method is friendly to possible parallel-processing applications. Experiments in HE compared to CABAC with context update show a BD rate change of 0.3%, 2.4%, 2.8%, 2.6% for AI, RA, LD and LDP, respectively. If combined with a new set of (m,n) that are more suitable for fixed probability contexts, the BD rate change of 0.1%, 1.0%, 1.3%, 1.3% for AI, RA, LD and LDP, respectively

# Introduction

In CABAC, each context is associated to a state that is an estimation of the probability of the symbol “1”. The initial state is set using the pre-defined parameters m and n

iInitState = Clip(1,126, (m*QP>>4) + n)

The state is continuously updated based on the coded symbol values. This context update makes parallel CABAC encoding/decoding difficult. In Torino meeting some contributions [1] were presented to disable the context update stage and use the pre-defined initial state during the whole encoding/decoding process. However, the pre-defined states are fixed values and do not adapt to different sequences or frames. There is coding efficiency drop when the pre-defined initial state derivates the true symbol statistics

This contribution describes an adaptive initialization method. For each context, a binary value is transmitted to indicate if this context is initialized using the pre-defined (m, n) or using an adaptive initial state value. The initial state can be derived from a previous frame whose statistics is similar to the current one. The encoder signals this adaptive initial state to the decoder. Details are introduced in the next section.

# Proposed method

At the beginning of each slice, the encoder first sends a binary map CtxMap of size N, where N equals to the total number of valid contexts for the current slice type. In this way, each element in this map corresponds to a CABAC context as follows:

- If the i-th map entry CtxMap (i) equals to 0, the i-th context, Ctx(i), will be initialized using state_hm derived from pre-defined (m,n) using eq (1).

- If CtxMap(i) equals to 1, Ctx(i) is initialized using an adaptive value state_adap transmitted by the encoder.

## Transmission of CtxMap

The CtxMap is encoded using run-length coding. A symbol “run” is sent to indicate the number of consecutive 0s before it hits a ‘1’ or the end of the array. Symbol “run” is encoded using Expo-Golomb codes.

## Calculation and transmission of state_adap

The encoder first calculates state_adap based on the statistics of a previous slice which has the same slice type and encoding QP as the current one. Then the encoder compares state_adap and state_hm and calculates a state offset, state_offset = state_adap - state_hm. The absolute value of state_offset, abs_state_offset, is further quantized,

level = (abs_state_offset)/Qstep

The combo of (level-1, sign) is encoded using Expo-Golomb and sent to the decoder. In our simulation, a value of Qstep =2 is used.

# Experimental results

The proposed method is implemented on top of HM 4.0. The coding efficiency is evaluated under common conditions as specified in [2]. The coding efficiency is measured by using 4 point BD-Rate. The encoding time is obtained from running encoding on cluster with same CPU type. The decoding time is measured on a single PC.

In the experiment, we first test the proposed method with the state initialization parameters (m, n) currently used in HM software. A second test is performed with a new set of (m, n) that is more suitable for the case of fixed probability. The results are presented in Table I and II respectively.

# Conclusions

This contribution describes an adaptive initialization method for CABAC with fixed probability contexts. The contexts are selectively initialized using a state transmitted by the encoder. Experimental results show that compared to the CABAC with context update, the BD rate change is 0.3%/2.4%/2.8%/2.6% (AI/RA/LD/LDP) in HE. If combined with a new set of (m,n), the gap can be further reduced to 0.1%/1.0%/1.3%/1.3% (AI/RA/LD/LDP) in HE.

I  Performance of the proposed method with (m, n) in HMI  Performance of the proposed method with (m, n) in HM

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.9% | -0.5% | -5.9% | -0.1% | 0.5% |
| Class B | 0.7% | -0.2% | 0.1% | -5.3% | 0.4% | 1.7% |
| Class C | 0.4% | 0.0% | 0.1% | -4.4% | 0.5% | 0.7% |
| Class D | 0.1% | 0.7% | 1.1% | -4.0% | 1.1% | 1.2% |
| Class E | 0.7% | -0.7% | -0.9% | -7.1% | 4.9% | 5.1% |
| Class F |  |  |  |  |  |  |
| Overall | 0.3% | -0.2% | 0.0% | -5.3% | 1.1% | 1.7% |
|  | 0.3% | -0.1% | 0.1% | -5.2% | 1.1% | 1.7% |
| Enc Time[%] | 99% |  |  | 107% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.6% | 4.6% | 4.0% | -7.0% | 11.4% | 10.2% |
| Class B | 2.7% | 2.8% | 2.8% | -5.1% | 3.4% | 7.2% |
| Class C | 2.4% | 3.4% | 3.6% | -3.4% | 5.0% | 5.5% |
| Class D | 3.0% | 4.7% | 4.7% | -2.1% | 7.5% | 7.9% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 2.4% | 3.8% | 3.7% | -4.5% | 6.6% | 7.7% |
|  | 2.8% | 3.9% | 3.8% | -4.1% | 7.1% | 7.7% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.5% | 3.1% | 3.2% | -5.1% | 8.8% | 13.1% |
| Class C | 2.2% | 3.7% | 3.6% | -3.7% | 7.8% | 8.8% |
| Class D | 3.1% | 6.5% | 7.1% | -2.1% | 13.7% | 14.2% |
| Class E | 3.7% | -0.1% | -1.1% | -3.9% | 5.6% | 5.9% |
| Class F |  |  |  |  |  |  |
| Overall | 2.8% | 3.5% | 3.5% | -3.8% | 9.2% | 10.9% |
|  | 2.8% | 3.6% | 3.6% | -3.7% | 9.2% | 10.9% |
| Enc Time[%] | #NUM! |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 2.3% | 1.5% | 1.7% | -5.5% | 8.7% | 12.9% |
| Class C | 2.0% | 2.2% | 2.1% | -4.1% | 6.7% | 7.7% |
| Class D | 2.6% | 3.9% | 4.5% | -2.8% | 12.2% | 12.2% |
| Class E | 4.0% | -0.8% | -3.1% | -4.2% | 5.4% | 5.5% |
| Class F |  |  |  |  |  |  |
| Overall | 2.6% | 1.9% | 1.6% | -4.3% | 8.4% | 10.0% |
|  | 2.6% | 2.0% | 1.6% | -4.1% | 8.6% | 10.1% |
| Enc Time[%] | 95% |  |  | 93% |  |  |
| Dec Time[%] | 98% |  |  | 96% |  |  |

I  Performance of the proposed method with (m, n) in HMII  Performance of the proposed method with new (m, n)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.3% | -0.6% | -0.3% | -6.0% | 0.2% | 0.7% |
| Class B | 0.5% | -0.1% | 0.1% | -5.4% | 0.6% | 1.8% |
| Class C | 0.3% | -0.1% | 0.0% | -4.4% | 0.4% | 0.6% |
| Class D | 0.0% | -1.3% | -1.1% | -4.0% | -0.1% | 0.1% |
| Class E | 0.2% | -0.9% | -1.1% | -7.3% | 5.1% | 5.2% |
| Class F |  |  |  |  |  |  |
| Overall | 0.1% | -0.6% | -0.4% | -5.3% | 1.0% | 1.5% |
|  | 0.1% | -0.5% | -0.4% | -5.3% | 1.0% | 1.6% |
| Enc Time[%] | 86% |  |  | 111% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 4.0% | 3.2% | -8.1% | 11.4% | 10.0% |
| Class B | 1.3% | 1.1% | 1.2% | -6.2% | 2.2% | 6.1% |
| Class C | 1.0% | 1.2% | 1.2% | -4.4% | 3.2% | 3.9% |
| Class D | 1.4% | 1.5% | 1.4% | -3.3% | 5.6% | 5.3% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 1.0% | 1.9% | 1.7% | -5.5% | 5.4% | 6.3% |
|  | 1.4% | 2.0% | 1.8% | -5.1% | 5.8% | 6.2% |
| Enc Time[%] | 88% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.2% | 0.9% | 1.0% | -6.1% | 6.9% | 11.5% |
| Class C | 0.8% | 1.6% | 1.3% | -4.7% | 5.7% | 6.4% |
| Class D | 1.2% | 2.8% | 2.1% | -3.6% | 11.4% | 11.6% |
| Class E | 2.1% | -0.7% | -0.4% | -4.8% | 5.8% | 6.4% |
| Class F |  |  |  |  |  |  |
| Overall | 1.3% | 1.3% | 1.1% | -4.9% | 7.5% | 9.3% |
|  | 1.3% | 1.3% | 1.2% | -4.8% | 7.4% | 9.3% |
| Enc Time[%] | 88% |  |  | 106% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.2% | 1.5% | 1.7% | -6.2% | 9.1% | 13.6% |
| Class C | 0.9% | 3.5% | 1.7% | -4.7% | 6.5% | 7.7% |
| Class D | 1.3% | 3.7% | 3.3% | -3.6% | 12.5% | 12.7% |
| Class E | 1.8% | -1.3% | -1.8% | -5.3% | 7.6% | 7.3% |
| Class F |  |  |  |  |  |  |
| Overall | 1.3% | 2.0% | 1.5% | -5.0% | 9.0% | 10.7% |
|  | 1.3% | 1.8% | 1.5% | -4.9% | 9.0% | 10.8% |
| Enc Time[%] | 87% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Draft Text

### 7.3.3  Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| } |  |
| if( entropy_coding_mode_flag && slice_type != I) |  |
| cabac_init_idc | ue(v) |
| if( entropy_coding_mode_flag) |  |
| adaptive_initialization_flag | u(1) |
| if (adaptive_initialization_flag == 1) |  |
| ctx_update_info() |  |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| if( !lightweight_slice_flag ) { |  |
| slice_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| } |  |
| } |  |

| ctx_update_info ( ) { | Descriptor |
| --- | --- |
| ctx_idx = 0 |  |
| while (ctx_idx < total_idx_num) { |  |
| run | u(v) |
| ctx_idx = ctx_idx + run |  |
| If (ctx_idx > total_idx_num) { |  |
| break |  |
| } |  |
| ctxUpdateMap[ctx_idx] =1 |  |
| ctx_idx ++ |  |
| } |  |
| ctx_idx = 0 |  |
| while (ctx_idx < total_idx_num) { |  |
| if (ctxUpdateMap[ctx_idx] == 1){ |  |
| level_minus1_and_sign | u(v) |
| ctxUpdateLevel [ctx_idx] = level_sign |  |
| } |  |
| ctx_idx++ |  |
| } |  |
| } |  |

### 4. 3. Slice header semantics

adaptive_initialization_flag: equal to 1 specifies that the adaptive initialization process is invoked.

run specifies the number of consecutive elements in ctxUpdateMap with zero value before a non-zero valued element

level_minus1_and_sign specifies the codewords of a different initial state for CABAC context.

#### 3. 1. 1 Initialisation process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Error: Reference source not found toError: Reference source not found contain the values of the variables n and m used in the initialisation of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end-of-slice flag and those related to CAVLC.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause Error: Reference source not found.

The two values assigned to pStateIdx and valMPS for the initialisation are derived from SliceQPY, which is derived in Equation Error: Reference source not found. Given the two table entries ( m, n ), the calculation of an initial state is specified by the following pseudo-coded process: the initialisation is specified by the following pseudo-code process:

- InitState preCtxState = Clip3( 1, 126, ( ( m  Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )

- If ctxUpdateMap[ctx_idx] == 1

sign = (ctxUpdateLevel [ctx_idx]%2)

level = ctxUpdateLevel [ctx_idx]>>1+1;

CtxStateOffset = sign? –level<<1: level<<1;

preCtxState  = InitState + CtxStateOffset;

else if ctxUpdateMap[ctx_idx] == 0

preCtxState = InitState

- if( preCtxState  <=  63 )  {
	pStateIdx = 63 − preCtxState
	valMPS = 0	(9-1)
} else {
	pStateIdx = preCtxState − 64
	valMPS = 1
}

In Error: Reference source not found, the ctxIdxTable and the ctxIdx for which initialisation is needed for each of the slice types are listed. The referenced context index tables ctxIdxTable include the values of m and n needed for the initialisation.

# References

- D.Marpe, “Unified PIPE-Based Entropy Coding for HEVC”, JCTVC-F268, JCT-VC Meeting, Torino, Italy, Jul. 2011.

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC Meeting, Torino, Italy, Jul. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).