| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G835 WG11 Number: m22407 |
| --- | --- |

| Title: | Cross verification of LG’s Block-based filter adaptation with intra prediction mode and CU depth information (JCTVC-G463) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides the cross-check results of LG’s proposed simplification of block-based filter adaptation for intra slices with intra prediction modes and CU depth information. In the block adaptation of HM4.0, a picture is divided into non-overlapping 4x4 blocks and each 4x4 block is classified based on pixels’ directional and Laplacian features. In JCTVC-G463, it is proposed to classify a 4x4 block based on its intra prediction modes and CU depth information instead of pixels’ directional and Laplacian features, which can reduce the computational complexity of block classification.

The software package obtained from the proponent was successfully compiled and tested under the common test conditions for AI_HE case. The BD rate results match exactly to the data provided by the proponent, and the differences of enc/dec time are within normal variations. Detailed performance data are shown in Table 1.

Table 1 Crosscheck results

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.26% | 0.03% | 0.03% |
| Class B | 0.10% | 0.00% | 0.00% |
| Class C | 0.07% | 0.03% | 0.04% |
| Class D | 0.05% | -0.02% | -0.02% |
| Class E | 0.13% | -0.01% | -0.02% |
| Class F | 0.00% | 0.01% | 0.01% |
| Overall A-E | 0.12% | 0.01% | 0.01% |
| Overall A-F | 0.10% | 0.01% | 0.01% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 97% |  |  |