| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G837 |
| --- | --- |

| Title: | Non-CE1: 8-bit Initialization for CABAC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Rajan Joshi Joel Sole Xianglin Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-845-7532 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes an 8-bit initialization method for CABAC. The 8-bit m (slope) and 8-bit n (intersection) are replaced by a single 8-bit InitIdx for each context. SlopeTable (16 elements) and IntersecTable (16 elements) are introduced to convert the 8-bit InitIdx to a CABAC probability state in the initialization stage. Two sets of table values are presented in this contribution. For both of these two sets, the table look-up operations can be implemented using formula calculation and thus the table storage is not necessary. The average BD-rate reduction is 0.0%/0.3%/0.5% /0.3% for AI/RA/LD/LDP (HE) for the first set of tables, and 0.0%/0.2%/0.4% /0.3% for AI/RA/LD/LDP (HE) for the second set of tables.

# Introduction

In current HM, the CABAC context initialization is as follows:

iInitState = Clip(1,126, (m*QP>>4) + n)                             (1)

where m and n are initialization parameters that can be stored in 8 bits (with a offset) and thus each context needs a total of 16-bit storage. This contribution first describes an 8-bit initialization method where the 8-bit m and 8-bit n are replaced by a single 8-bit InitIdx.

# Initialization using 8-bit InitIdx

The 8-bit InitIdx consists of 2 parts: a 4-bit SlopeIdx (b0-b3) and a 4-bit IntersectiontIdx (b4-b7).

Using the 4-bit SlopeIdx, a slope value m can be derived by looking up a 16-element SlopeTable:

m = SlopeTable[SlopeIdx]

Similarly, an intersection value n can be derived by looking up a 16-element IntersecTable:

n = IntersecTable [IntersecIdx]

The CABAC probability state is derived using SlopeVal and IntersecVal,

iInitState = Clip(1,126, (m*QP>>4) + n)

In this contribution, we present two set of tables. For both sets of tables, the table look-up operation can be replaced by formula calculations, and thus the storage of tables can be saved.

## First set of tables:

SlopeTable[16]   = {-44,  -39,  -33,  -28,  -22,  -17, -11,  -5,    0,    6,   11,  17,  22,  28,   33,  39};

IntersecTable[16] = {-13,  -5,    3,  11,   19,  27,  35,  43,  51,  59,  67,  75,  83,  91,  99,  107}

The table look-up operation m = SlopeTable[SlopeIdx] can be replaced by

m = (SlopeIdx *89-703)>>4                   (2)

The table look-up operation  n = IntersecTable [IntersecIdx] can be replaced by

n = (IntersecIdx <<3)-13                     (3)

In the implementation, table SlopeTable can be pre-calculated using (3) at the beginning of each slice, and the computation of equation (2) for each context can be saved.

## Second set of Tables

In the second set, a new SlopeTable table is used while the IntersecTable table is the same as the first set.

SlopeTable[16]  = {-32, -28, -24, -20,  -16, -12,  -8,  -4 ,  0,  4,   8,  12,  16,  20, 24,   28};

IntersecTable[16] = {-13,  -5,  3,  11,  19,  27,  35,  43,  51,  59,  67,  75,  83,  91,  99,  107}

The table look-up operation m = SlopeTable[SlopeIdx] can be replaced by

m = (SlopeIdx <<2)-32                      (4)

The table look-up operation  n = IntersecTable [IntersecIdx] can be replaced by

n = (IntersecIdx <<3)-13

# Experimental results

The proposed 8-bit initialization method has been implemented on top of HM 4.0. The coding efficiency is evaluated under the common conditions specified in [1]. The coding efficiency is measured by using 4 point BD-Rate. The encoding and decoding time is measured on clusters with same CPU.

3.1 Results of the first set of tables

BD rate change of 8-bit initialization using the first set of Tables (Anchor: HM 4.0)

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.2% | 0.1% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | -0.2% | 0.1% | 0.1% |
| Class C | 0.0% | -0.1% | -0.4% | -0.3% | 0.0% | -0.2% |
| Class D | 0.0% | -0.6% | -1.1% | -0.5% | -0.1% | -0.9% |
| Class E | -0.1% | -0.3% | -0.1% |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.2% | -0.3% | -0.3% | 0.0% | -0.2% |
|  | 0.0% | -0.2% | -0.3% | -0.3% | 0.0% | -0.2% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

|  | Low delay B HE |  |  | Low delay P HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.4% | -0.2% | -0.2% | 0.5% | 0.3% |
| Class C | -0.4% | 0.3% | -0.1% | -0.3% | 0.3% | 0.3% |
| Class D | -0.7% | 0.4% | -0.7% | -0.4% | 1.3% | 0.7% |
| Class E | -1.0% | -0.8% | 0.8% | -0.7% | -0.8% | 0.0% |
| Class F |  |  |  |  |  |  |
| Overall | -0.5% | 0.2% | -0.1% | -0.3% | 0.4% | 0.3% |
|  | -0.5% | 0.1% | -0.1% | -0.3% | 0.3% | 0.3% |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

3.2 Results of the second set of tables

I BD rate change of 8-bit initialization using the second set of Tables (Anchor: HM 4.0)

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.0% | -0.1% | -0.1% | -0.4% |
| Class B | 0.0% | 0.0% | 0.0% | -0.2% | 0.2% | 0.1% |
| Class C | 0.0% | -0.2% | -0.4% | -0.3% | 0.0% | 0.0% |
| Class D | 0.0% | -0.9% | -1.1% | -0.4% | -0.3% | -0.5% |
| Class E | -0.1% | -0.3% | -0.1% |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.3% | -0.3% | -0.2% | 0.0% | -0.2% |
|  | 0.0% | -0.3% | -0.3% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 94% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.3% | -0.3% | -0.2% | 0.2% | 0.1% |
| Class C | -0.3% | 0.3% | -0.1% | -0.2% | 0.3% | 0.1% |
| Class D | -0.7% | 0.3% | 0.2% | -0.5% | 0.9% | 0.3% |
| Class E | -0.8% | 0.1% | 0.8% | -0.6% | 0.0% | 0.5% |
| Class F |  |  |  |  |  |  |
| Overall | -0.4% | 0.3% | 0.1% | -0.3% | 0.4% | 0.2% |
|  | -0.4% | 0.2% | 0.1% | -0.3% | 0.3% | 0.2% |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusion

This contribution described an 8-bit initialization method. Each context is assigned an 8-bit InitIdx. At the initialization stage, a slope value m and a intersection value n are derived using InitIdx. Two set of conversation tables are presented in this contribution. The average BD-rate reduction is 0.0%/0.3%/0.5% /0.3% for AI/RA/LD/LDP (HE) for the first set of tables, and 0.0%/0.2%/0.4% /0.3% for AI/RA/LD/LDP (HE) for the second set of tables.

# Working Draft Texts

5.1 Working draft for the first set of tables

### 9.3.1 Initialisation process

Outputs of this process are initialised CABAC internal variables.

The processes in subclauses  and Error: Reference source not found are invoked when starting the parsing of the slice data of a slice in subclause Error: Reference source not found.

#### 9.3.1.1 Initialisation process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Error: Reference source not found to Error: Reference source not found contain the values of the variables n and m the value of InitIdx used in the initialisation of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end-of-slice flag and those related to CAVLC.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause Error: Reference source not found.

The two values assigned to pStateIdx and valMPS for the initialisation are derived from SliceQPY, which is derived in Equation Error: Reference source not found. Given the two table entries ( m, n ),  the table entry InitIdx, the initialisation is specified by the following pseudo-code process:

m = ((InitIdx>>4)*89-703)>>4

n = ((InitIdx&15) <<3)-13

preCtxState = Clip3( 1, 126, ( ( m  Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )

valMPS  = ( preCtxState  <=  63)? 0: 1;
                pStateIdx = ( preCtxState  <=  63)? (63 – preCtxState): (pStateIdx = preCtxState – 64);

if( preCtxState  <=  63 )  {
	pStateIdx = 63 − preCtxState
	valMPS = 0	(9-1)
} else {
	pStateIdx = preCtxState − 64
	valMPS = 1
}

In Error: Reference source not found, the ctxIdxTable and the ctxIdx for which initialisation is needed for each of the slice types are listed. The referenced context index tables ctxIdxTable include the values of m and n needed for the initialisation.

5.2 Working draft for the second set of tables

### 9.3.1 Initialisation process

Outputs of this process are initialised CABAC internal variables.

The processes in subclauses  and Error: Reference source not found are invoked when starting the parsing of the slice data of a slice in subclause Error: Reference source not found.

#### 9.3.1.1 Initialisation process for context variables

Outputs of this process are the initialised CABAC context variables indexed by ctxIdx.

Error: Reference source not found to Error: Reference source not found contain the values of the variables n and m the value of InitIdx used in the initialisation of context variables that are assigned to all syntax elements in subclauses Error: Reference source not found to Error: Reference source not found except for the end-of-slice flag and those related to CAVLC.

For each context variable, the two variables pStateIdx and valMPS are initialised.

NOTE 1 – The variable pStateIdx corresponds to a probability state index and the variable valMPS corresponds to the value of the most probable symbol as further described in subclause Error: Reference source not found.

The two values assigned to pStateIdx and valMPS for the initialisation are derived from SliceQPY, which is derived in Equation Error: Reference source not found. Given the two table entries ( m, n ),  the table entry InitIdx, the initialisation is specified by the following pseudo-code process:

m = ((InitIdx>>4) <<2)-32

n = ((InitIdx&15) <<3)-13

preCtxState = Clip3( 1, 126, ( ( m  Clip3( 0, 51, SliceQPY ) ) >> 4 ) + n )

valMPS = ( preCtxState  <=  63)? 0: 1;
               pStateIdx =( preCtxState  <=  63)? (63 – preCtxState): (pStateIdx = preCtxState – 64);

if( preCtxState  <=  63 )  {
	pStateIdx = 63 − preCtxState
	valMPS = 0	(9-2)
} else {
	pStateIdx = preCtxState − 64
	valMPS = 1
}

In Error: Reference source not found, the ctxIdxTable and the ctxIdx for which initialisation is needed for each of the slice types are listed. The referenced context index tables ctxIdxTable include the values of m and n needed for the initialisation.

# References

- F.Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, JCT-VC Meeting, Torino, Italy, Jul. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).