| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G839 M22411 |
| --- | --- |

| Title: | Non-CE13: cross-verification of Samsung, Canon, MediaTek proposal JCTVC-G787 on additional merge candidates with MV dependent offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yongjoon Jeon Byeongmoon Jeon #221 Yangjae-dong, Seoucho-gu, Seoul 137-130, Korea | Tel: Email: : | yongjoon.jeon@lge.com bm.jeon@lge.com |
| Source: | LG |  |  |

_____________________________

# Abstract

JCTVC-G787 [1] includes results for the three schemes, MV-dependent offset candidate, MV-dependent offset candidate + rounding candidate, and MV-dependent offset candidate + rounding candidate with fast encoding algorithm. This document is a crosscheck report of ‘MV-dependent offset candidate + rounding candidate’ only. It is reported that the proposed merging candidates brings about 0.1%, 0.2%, 0.1%, 0.2%, 0.3%, and 0.6% BD rate reduction for RA-HE, RA-LC, LB-HE, LB-LC, LP-HE and LP-LC respectively, and that these measurements match the results reported by the proponent. The gain seems to be beneficial but the scheme increases the encoding time about 3~6%. Therefore it is difficult to determine whether the proposed scheme is actually beneficial in terms of both coding efficiency and computational complexity. Also, there are several similar proposals related to this technology which have been studied in the CE13 and newly proposed at this meeting. Therefore it is recommended the proposed scheme of JCTVC-G787 to be studied together with those similar proposals to find the optimal solution related to this technology in terms of both coding efficiency and computational complexity.

# Verification

The proponent provided their software and results. The code was successfully compiled and run on the following platform : Intel Xeon5570@2.93GHz 64 bits Linux machines. The common test conditions and reference configurations specified in [1] are followed. The generated bit-streams have decoded on the PCs with the following configurations: Microsoft® Windows® XP 64bits, Intel® Xeon® CPU E5355 @2.66GHz, 16GB of RAM. In the simulation, -o decoder option (YUV writing) was not used.

# Experimental results

The simulation results are shown Table 1. All of the results exactly match those provided by proponents.

Table 1: Crosscheck results

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.3% | -0.2% | -0.1% | -0.1% | -0.3% |
| Class B | -0.2% | 0.0% | 0.1% | -0.3% | 0.0% | -0.2% |
| Class C | -0.1% | 0.0% | -0.1% | -0.2% | -0.1% | -0.2% |
| Class D | -0.2% | 0.0% | -0.1% | -0.2% | -0.3% | -0.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
|  | -0.1% | -0.1% | -0.1% | -0.2% | -0.1% | -0.2% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | 0.2% | 0.1% | -0.3% | 0.1% | 0.0% |
| Class C | -0.3% | -0.5% | 0.0% | -0.2% | 0.2% | 0.2% |
| Class D | -0.3% | 0.5% | 0.1% | -0.3% | -0.4% | -0.1% |
| Class E | 0.3% | 0.6% | 0.2% | 0.2% | 1.2% | 0.5% |
| Overall | -0.1% | 0.2% | 0.1% | -0.2% | 0.2% | 0.1% |
|  | -0.1% | 0.2% | 0.1% | -0.2% | 0.1% | 0.1% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | 0.4% | 0.2% | -0.8% | 0.0% | -0.4% |
| Class C | -0.3% | -0.4% | -0.2% | -0.6% | -0.4% | -0.5% |
| Class D | -0.4% | -0.3% | -0.2% | -0.6% | -0.3% | -0.5% |
| Class E | 0.2% | 0.5% | 1.0% | -0.3% | 1.4% | 1.1% |
| Overall | -0.3% | 0.0% | 0.1% | -0.6% | 0.1% | -0.2% |
|  | -0.3% | 0.1% | 0.2% | -0.6% | 0.1% | -0.1% |
| Enc Time[%] | 105% |  |  | 106% |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

# Study results

The proponent added ‘rounding predictor’ and ‘MV dependent offset predictor’ if the number of entries in the mergeCandList does not reach the maximum (5) as follows.

Rounding predictor

After the non-scaled bi predictor, if the list does not reach the maximum, only one predictor is added to the list. The shift predictor is generated by rounding the MV of the first candidate in the list as follows.

Let mvLXCand be the MV of the first candidate in the list.

mvLX[0] = mvLXCand [0] >> 2

mvLX[1] = mvLXCand [1] << 2

MV dependent offset predictor

After the rounding predictor, if the list does not reach the maximum, four predictors can be added at maximum.

The four predictors are generated as follows.

If the first candidate in the list is bi-predicted and the following condition is satisfied, the order is as follows.

- (pocL0 < pocCur && pocCur < pocL1) || (pocL1 < pocCur && pocCur < pocL0)

,where pocL0 and pocL1 are the POCs of the reference pictures indicated by refIdxL0 and refIdxL1 of the first candidate, and pocCur is the POC of the current picture.

- 0 : add delta to x component. delta is set equal to 8 if mvLXCand [0] > 16, 4 if mvLXCand [0] > 8, 2 if mvLXCand [0] > 4, 1 otherwise.

- 1 : add delta to x component. delta is set equal to -8 if mvLXCand[0] > 16, -4 if mvLXCand [0] > 8, -2 if mvLXCand [0] > 4, -1 otherwise.

- 2 : add delta to y component. delta is set equal to 8 if mvLXCand [1] > 16, 4 if mvLXCand [1] > 8, 2 if mvLXCand [1] > 4, 1 otherwise.

- 3 : add delta to y component. delta is set equal to -8 if mvLXCand [1] > 16, -4 if mvLXCand [1] > 8, -2 if mvLXCand [1] > 4, -1 otherwise.

Otherwise, the order is as follows.

- 0 : add delta to x component. delta is set equal to -8 if mvLXCand[0] > 16, -4 if mvLXCand [0] > 8, -2 if mvLXCand [0] > 4, -1 otherwise.

- 1 : add delta to x component. delta is set equal to 8 if mvLXCand [0] > 16, 4 if mvLXCand [0] > 8, 2 if mvLXCand [0] > 4, 1 otherwise.

- 2 : add delta to y component. delta is set equal to -8 if mvLXCand [1] > 16, -4 if mvLXCand [1] > 8, -2 if mvLXCand [1] > 4, -1 otherwise.

- 3 : add delta to y component. delta is set equal to 8 if mvLXCand [1] > 16, 4 if mvLXCand [1] > 8, 2 if mvLXCand [1] > 4, 1 otherwise.

Our simulation results showed that the proposed method (MV dependent offset predictor + rounding predictor) of G787 achieves the following gains with 3~6% encoding time increase. Note that, in our simulation, the fast encoding algorithm introduced in JCTVC-G787 is not enabled.

From out simulations, the proposed non-redundant predictors of JCTVC-G787 were efficient in terms of coding efficiency but the encoding time increase is not small. Several similar contributions are proposed at this meeting. The proposed method (MV dependent offset predictor + Rounding predictor) of JCTVC-G787 should be studied together with the similar proposals to find the best non-redundant predictor in terms of both coding efficiency and computational complexity.

# References

[1]	T. Lee, J. Chen, J. H. Park (Samsung), G. Laroche, P. Onno (Canon), J.-L. Lin, Y.-W. Chen, Y.-W. Huang, S. Lei (MediaTek), “Non-CE13: Additional merge candidates with MV dependent offsets” JCT-VC Document, JCTVC-G787,  Geneva, CH, 19-30 November, 2011.