| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G842 |
| --- | --- |

| Title: | CE13 Section 3.5: Cross-verification of Canon’s test 3 (JCTVC-G240) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports cross-checking results of CE13 section 3.5 test 3 in JCTVC-G240. The results confirm the BD-rates and encoding/decoding time from the proponents. The proponents’ code has also been analysed and found to match to the context in the proposal.

# Cross-check results

In this cross-checking report, CE13 section 3.5 test 3 in JCTVC-G240 is verified. Software was compiled on 64-bit Linux and executed on Linux cluster. Cross-check of Canon’s proposal [2] was carried out by setting the macro definitions in the source code as in Table 1.

Table 1. Macro definition of CE13 section 3.5 test 3

#define CE13_3_5_CANON_TEST_3 1 #if CE13_3_5_CANON_TEST_3 #define CE13_3_5_NO_ZERO_PREDICTORS 1 // No add zero predictors for Merge #define CE13_3_5_NO_SCALED_PREDICTORS 1 // No add scaled predictors for Merge #define CE13_3_5_MAX_NUM_COMBINED 3 // Maximum number of combined predictors #define CE13_3_5_ADD_NON_REDUNDANT_PREDICTORS 1 // Add non-redundant predictors for AMVP/Merge #endif

Experiments were conducted under four conditions (RA-HE, RA-LC, LB-HE, and LB-LC) as specified in common test condition [2]. Table 2 shows test results compared the anchor of HM4.0 and Table 3 shows test results compared the anchor of HM4.0 with ENC_MRG_FIX. The results are consistent to those from the proponent. More details can be found in the accompanying Excel sheets.

Table 2. CE13 section 3.5 test 3 (Anc: HM4.0)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% | -0.1% | -0.1% | -0.3% |
| Class B | -0.2% | -0.2% | -0.1% | -0.2% | -0.3% | -0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% |
| Class D | 0.1% | -0.2% | 0.0% | 0.0% | -0.2% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |
|  | 0.0% | -0.2% | -0.1% | -0.1% | -0.2% | -0.2% |
| Enc Time[%] | 105% |  |  | 105% |  |  |
| Dec Time[%] | 102% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.2% | -0.3% | -0.3% | -0.3% | -0.3% |
| Class C | -0.1% | -0.3% | -0.1% | -0.1% | -0.2% | -0.3% |
| Class D | -0.1% | -0.1% | -0.5% | -0.1% | 0.0% | -0.2% |
| Class E | 0.2% | 0.4% | 1.6% | 0.3% | 1.3% | 1.0% |
| Class F | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | -0.1% | -0.1% | 0.1% | -0.1% | 0.1% | 0.0% |
|  | -0.1% | -0.1% | 0.0% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 95% |  |  |

Table 3. CE13 section 3.5 test 3 with ENC_MRG_FIX (Anc: HM4.0 with ENC_MRG_FIX)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | -0.1% | -0.5% |
| Class B | -0.2% | -0.2% | 0.0% | -0.2% | -0.1% | -0.3% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | -0.1% |
| Class D | 0.0% | -0.1% | 0.1% | 0.0% | -0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% |
|  | -0.1% | -0.1% | 0.0% | -0.1% | -0.1% | -0.2% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.3% | -0.1% | -0.4% | -0.4% | -0.4% | -0.5% |
| Class C | -0.2% | -0.5% | -0.3% | -0.2% | 0.1% | 0.1% |
| Class D | -0.1% | 0.1% | 0.0% | -0.2% | -0.4% | -0.1% |
| Class E | 0.2% | 0.2% | -0.3% | 0.2% | 1.6% | 1.1% |
| Overall | -0.2% | -0.1% | -0.2% | -0.2% | 0.1% | 0.1% |
|  | -0.2% | -0.1% | -0.3% | -0.2% | 0.1% | 0.0% |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# References

[1]	G. Laroche, T. Poirier, and P. Onno, "CE13: Experiment regarding section 3.5," JCTVC-G240, Jul. 2011.

[2]	Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900, Jul. 2011.