| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G843 |
| --- | --- |

| Title: | CE4: Fine granularity QP offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xianglin Wang, Rajan Joshi, Geert Van Der Auwera, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-651-5135 Email: xianglin@qualcomm.com rajanj@qualcomm.com geertv@qualcomm.com martak@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports CE results based on JCTVC-F610. According to this proposal, offset values are sent at slice level to modify the quantizer step size. The offset is signaled as a percentage of the original quantizer step-size, resulting in much higher granularity compared to what could be achieved through a QP change. The BD-rate for RA-HE and LB-HE configurations is -0.9℅ and 1.2℅, respectively. The BD-rate for RA-LC and LB-LC configurations is -1.3℅ and 1.8℅, respectively.

- Introduction

This contribution reports CE results based on JCTVC-F610.

# Proposal

According to JCTVC-F610, an offset is signaled in the slice header to change quantizer step size. For each non-intra slice, up to four offsets (intra/inter and luma/chroma) may be signaled to the decoder. The modified quantizer step-size is represented as the original step-size plus the signaled offset. The offset is represented as a percentage of the quantizer step-size.

On the encoder side, the previous slice with the same QP value is used to gather level offset statistics. Offset values are calculated based on the statistics. Then, based on the offsets calculated, a modified quantizer step-size is derived. The modified quantizer step-size is used for deriving the reconstruction levels on the encoder and decoder side.

The proposed technique can be thought of as a QP change at much finer granularity.

# Results

The proposed method is implemented in HM4.0. Simulation results are presented according to the CE conditions specified in [1]. For the common test conditions. Table 1 shows the results. The BD-rate for RA-HE and LB-HE configurations is -0.9℅ and 1.2℅, respectively. The BD-rate for RA-LC and LB-LC configurations is -1.3℅ and 1.8℅, respectively. Detailed results are provided in the accompanying excel file.

Table 1: BD-rate comparison against HM4.0

The proposed scheme is also compared against adaptive rounding/EEM with RDOQ off.  Table 2 shows the results. It can be seen that the proposed scheme provides additional coding gain on top of EEM.

Table 2: BD-rate comparison against EEM with RDOQ off

- Conclusions

Based on the CE conditions, the proposed scheme of signaling of a fine granularity QP offset can provide coding gain for both RDOQ on and RDOQ off cases.  We propose such a scheme to be adopted into HM.

- References

- "Description of Core Experiment 4: Quantization", K. Sato et al, JCTVC-F904, Torino, Italy, July 2011.

- “Fine granularity QP offset”, JCTVC-F610, Torino, Italy, July, 2011

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).