| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G845 |
| --- | --- |

| Title: | CE13 Section 3.1: Cross-verification of MediaTek’s test 3d (JCTVC-G231) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports cross-checking results of CE13 section 3.1 test 3d in JCTVC-G231. The results confirm the BD-rates and encoding/decoding time from the proponents. The proponents’ code has also been analysed and found to match to the context in the proposal.

# Cross-check results

In this cross-checking report, CE13 section 3.1 test 3d of JCTVC-G231 is verified. Test 3d includes the method of additional MVPs and adaptive list size by neighboring Merge indices. The source code is analyzed and matches to the content mentioned in JCTVC-G231. Software was compiled on 64-bit Linux and executed on Linux cluster.

Experiments were conducted under six conditions (RA-HE, RA-LC, LB-HE, LB-LC, LP-HE, LP-LC) as specified in common test condition [2]. Table 1 shows test results with the anchor of HM4.0. The results are consistent to those from the proponent. More details can be found in the accompanying Excel sheets.

Table 1. CE13 section 3.1 test 3d

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.4% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.1% | -0.1% | 0.0% | 0.1% |
| Class C | -0.1% | -0.2% | 0.0% | -0.1% | -0.2% | -0.1% |
| Class D | 0.1% | 0.0% | 0.1% | 0.0% | -0.2% | 0.0% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% | 0.0% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.2% | -0.1% | -0.3% | -0.2% | 0.0% | 0.0% |
| Class C | -0.3% | -0.2% | -0.2% | -0.3% | 0.0% | -0.4% |
| Class D | -0.2% | -0.1% | 0.1% | -0.2% | -0.3% | 0.1% |
| Class E | 0.1% | 0.6% | 0.8% | -0.1% | -0.2% | -0.4% |
| Overall | -0.2% | 0.0% | 0.1% | -0.2% | -0.1% | -0.1% |
|  | -0.1% | 0.0% | 0.0% | -0.2% | -0.2% | -0.2% |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4% | -0.2% | 0.1% | -0.6% | -0.3% | -0.4% |
| Class C | -0.3% | -0.2% | -0.2% | -0.5% | -0.5% | -0.4% |
| Class D | -0.1% | -0.2% | -0.2% | -0.3% | -0.4% | -0.4% |
| Class E | -0.1% | 0.0% | -0.3% | -0.3% | 0.1% | 0.0% |
| Overall | -0.2% | -0.2% | -0.1% | -0.4% | -0.3% | -0.3% |
|  | -0.2% | -0.2% | -0.2% | -0.4% | -0.3% | -0.4% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# References

- J.L. Lin, Y.W. Chen, et al, " CE13: Results of section 3.1 tests 1, 3d, and 3e on replacing redundant MVPs and its combination with adaptive MVP list size" JCTVC-G231, Jul. 2011.

- Frank Bossen, "Common test conditions and software reference configurations," JCTVC-F900, Jul. 2011.