| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G848 |
| --- | --- |

| Title: | Non-CE1: Crosscheck for Qualcomm's context reduction for CABAC in JCTVC-G718 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Yu-Wen Huang No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Tel: Email: | Yu-Wen Huang +886-3-5670766 ext. 26297 {peter.chuang, yuwen.huang }@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The purpose of this document is to crosscheck the context reduction methods proposed by Qualcomm in JCTVC-G718. The context models of pred_type, merge_idx, inter_pred_flag, ref_idx_lx, cbf_cb, cbf_cr, coeff_abs_level_greater1_flag, and coeff_abs_level_greater2_flag are modified with fewer contexts. Totally, 56 contexts are reduced. The verification task was done successfully, and the results match exactly those provided by Qualcomm.

# Verification

Qualcomm provided the HM source codes of their proposed codec described in JCTVC-G718 [1]. Ten tests were performed. The first 8 tests evaluated the impact of the context reduction of each syntax element. The last two tests are the combined results of all proposed context reduction methods. Eight macros are defined as follows:

#define INTER_P                         	0     //G718-2.1.1
#define AMP_MOD                           	0     //G718-2.1.2 
#define MRG_IDX                           	0     //G718-2.2
#define INTER_DIR_MOD_INIT              	0     //G718-2.3.1 || G718-2.3.2
#define INTER_DIR_MOD                  	0     //G718-2.3.2 
#define REF_IDX_MOD                      	0     //G718-2.4
#define CHROMA_CBF_UV                    	0     //G718-2.5
#define COEF_LEVEL_CTX_C1C2             	0     //G718-2.6

The verifications are carried out by setting the parameters in Table 1 in the provided codes.

One to four conditions (includes intra HE, random access HE, low delay B HE, low delay P HE) were run for different tests for the proposed context reduction methods. The running of the tests includes encoding, decoding, and collecting the run time of encoding and decoding processes.

Table 1.  Parameter setting of 10 tests

| Sections | 2.1.1 | 2.1.2 | 2.2 | 2.3.1 | 2.3.2 | 2.4 | 2.5 | 2.6 | 3.1 | 3.2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| test | LD_P | LD,RA | LD,RA | LD,RA | LD,RA | LD,RA | AI,LD,RA | AI,LD,RA | LD,RA | AI,LD,RA |
| INTER_P | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| AMP_MOD | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| MRG_IDX | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| INTER_DIR_MOD_INIT | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |
| INTER_DIR_MOD | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| REF_IDX_MOD | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| CHROMA_CBF_UV | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 |
| COEF_LEVEL_CTX_C1C2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |

# Results

The BD-rates from our experimental results match exactly those provided by Qualcomm. The encoding time and decoding time are also similar to those provided by Qualcomm. The detailed results are listed in JCTVC-G848_2.1.1.xls to JCTVC-G848_3.2.xls.

# Conclusion

The results presented by Qualcomm in JCTVC-G718 are confirmed.

# References

- W. -J. Chien, J. Sole, M. Karczewicz, “Context reduction for CABAC,” Document of Joint Collaborative Team on Video Coding, JCTVC-G718, Nov. 2011.