| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G850 |
| --- | --- |

| Title: | Non CE4: Fine granularity QP offset |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | *Xiang Li *Xun Guo #Shawmin Lei (MediaTek) **Xianglin Wang **Rajan Joshi **Geert Van Der Auwera **Marta Karczewicz (Qualcomm) *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 **5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: Email: | [phone contact number] x.li@mediatek.com xun.guo@mediatek.com shawmin.lei@mediatek.com xianglin@qualcomm.com rajanj@qualcomm.com geertv@qualcomm.com martak@qualcomm.com |
| Source: | MediaTek Inc, Qualcomm Inc |  |  |

_____________________________

# Abstract

This contribution reports simulation results using the idea of fine granularity QP offset proposed in JCTVC-F610. Additionally, the actual signaling scheme of the offset values is re-designed so that the signaling of the offset values in each slice is independent from previous slices..The proposed scheme is tested accoding to test cases specified in CE4.2.

# Introduction

According to JCTVC-F610, an offset is signaled in the slice header to change quantizer step size. For each non-intra slice, up to four offsets (intra/inter and luma/chroma) may be signaled to the decoder. The modified quantizer step-size is represented as the original step-size plus the signaled offset. The offset is represented as a percentage of the quantizer step-size.

On the encoder side, the previous slice with the same QP value is used to gather level offset statistics. Offset values are calculated based on the statistics. Then, based on the offsets calculated, a modified quantizer step-size is derived. The modified quantizer step-size is used for deriving the reconstruction levels on the encoder and decoder side.

Based on such a coding scheme, the actual signaling scheme of the offset values is re-designed so that the signaling of the offset values in each slice is independent from previous slices.

# QP Rounding

Essentially, fine-QP achieves the effect of fractional QP with the help of linear de-quantization offset. Intuitively, an encoder optimization method can be obtained by rounding the fractional QP to the closest integer QP. When QPs of luma and chroma signal are allowed to be different, luma QP rounding and chroma QP rounding can be performed as follows.

## Luma QP rounding

Based on the statistics of transformed coefficients in luma inter blocks, luma QP rounding can be easily obtained by rounding the derived luma offset to the nearest integer QP according to the following table.

| Condition | dQP |
| --- | --- |
| 87<= offset | +5 |
| 64<= offset < 87 | +4 |
| 43 <= offset < 64 | +3 |
| 24<= offset < 43 | +2 |
| 7<= offset < 24 | +1 |
| -7 <= offset < 7 | 0 |
| -20<= offset < -7 | -1 |
| -32 <= offset < -20 | -2 |
| -42 <= offset < -32 | -3 |
| -52 <= offset < -42 | -4 |
| offset< -52 | -5 |

The initial offsets of luma QP rounding are empirically set to 19 for inter frames and 0 for intra frames.

## Chroma QP rounding

Similar to luma QP rounding, chroma QP rounding can be obtained based on the statistics of chroma transformed coefficients and the following table.

| Condition | dQP |
| --- | --- |
| 98<= offset | +5 |
| 73 <= offset < 98 | +4 |
| 51 <= offset < 73 | +3 |
| 32 <= offset < 51 | +2 |
| 14 <= offset < 32 | +1 |
| -13 <= offset < 14 | 0 |
| -25<= offset < -13 | -1 |
| -36 <= offset < -25 | -2 |
| -46 <= offset < -36 | -3 |
| -55 <= offset < -46 | -4 |
| offset< -55 | -5 |

The initial offsets of chroma QP rounding are empirically set to -20 and -10 for intra and inter frames.

It should be noted that in HM4.0, chroma QP is derived from luma QP according to a lookup table. With such settings chroma QP rounding cannot be applied since chroma QP totally depends on luma QP. To enable adaptive selection of chroma QP, a syntax “chroma_qp_index_offset” is copied from H.264/AVC. Consequently, chroma QP rounding proposed above can be easily achieved.

# Proposed Specification for the WD

## Revisions in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| interpolation_filter_flag | u(1) |
| seq_qstep_scaler_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

seq_qstep_scaler_flag equal to 1 specifies that the fine scaling of quantization step is enabled. Otherwise, the feature is disabled.

## Revisions in Slice Header

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| slice_qp_delta | se(v) |
| if(seq_qstep_scaler_flag) { |  |
| slice_qstep_scaler_luma | se(v) |
| slice_qstep_scaler_chroma0 | se(v) |
| slice_qstep_scaler_chroma1 | se(v) |
| } |  |
| alf_param() |  |
| …… |  |
| } |  |

slice_qstep_scaler_luma specifies the fine scaling factor of luma quantization step in this slice. When slice_qstep_scaler_luma is not presented, slice_qstep_scaler_luma shall be inferred as zero. The range of slice_qstep_scaler_luma shall be within [-15, 15], inclusively.

slice_qstep_scaler_chroma0 specifies the fine scaling factor of the quantization step for the first chroma component in this slice. When slice_qstep_scaler_chroma0 is not presented, slice_qstep_scaler_chroma0 shall be inferred as zero. The range of slice_qstep_scaler_chroma0 shall be within [-15, 15], inclusively.

slice_qstep_scaler_chroma1 specifies the fine scaling factor of the quantization step for the second chroma component in this slice. When slice_qstep_scaler_chroma0 is not presented, slice_qstep_scaler_chroma0 shall be inferred as zero. The range of slice_qstep_scaler_chroma1 shall be within [-15, 15], inclusively.

## Signaling cost at slice header

slice_qstep_scaler_luma, slice_qstep_scaler_chroma0, and slice_qstep_scaler_chroma1, are coded with se(v). The range of these values shall be within [-15, 15], inclusively. Therefore, 9 bits x 3 = 27 bits will be signaled at most for each slice.

Assuming that the original quantization step is Q, the new quantization step Q’ with the proposed method is

Q’=Q* (128+delta)/128,

where delta denotes the proposed syntax element (slice_qstep_scaler_luma, slice_qstep_scaler_chroma0, or slice_qstep_scaler_chroma1). Generally, delta can be regarded as a scaling factor to fine tune the quantization step. When delta is zero, the new quantization step Q’ is the same as the original quantization step Q. That is, the proposed method is backward compatible with the current quantization design in HEVC.

As mentioned above, the range of proposed scaler is within [-15, 15]. That is when FINE-QP is on, the change of quantization step is within [-15/128, 15/128] = [-11.7%, 11.7%]. Considering that QP change by one roughly leads to 12.5% of change in quantization step, the proposed FINE-QP may be regarded as a fractional part of QP. Therefore, FINE-QP will not cause dynamic range issues.

## Revisions in de-quantization process and complexity analysis

For convenience, terminology from JCTVC-E243 is used, i.e.,

B	= source bit width (8 or 10 bit in the experiments described below)

DB	= B-8 (internal bit-depth increase with 8-bit input)

N 	= transform size

M	= log2(N)

Q	= f(QP%6), 	where f(x)   =   {26214,23302,20560,18396,16384,14564}, x=0,…,5

IQ 	= g(QP%6), 	where g(x)  =   {40,45,51,57,64,72}, x=0,…,5

In case of non-square transform, let nW and nH be the width and height of transform, N=sqrt(nWxnH).

Following the definitions above, dequantizer is specified as follows (JCTVC-E243)

coeffQ = ((level*IQ[QP%6] << (QP/6)) + offset)>>(M-1+DB),  offset = 1<<(M-2+DB)	(1)

coeffQ = min(32767,max(-32768,coeffQ))                      			(2)

The clipping ensures that the value of the quantized transform coefficients, coeffQ, can be represented with 16 bits.

The proposed de-quantization process will be illustrated in cases of without and with quantization matrices based on the above formula.

### De-quantization without quantization matrices

In case of no quantization matrices, (1) is to be modified to (3) to support FINE-QP.

coeffQ = ((level*IQ’[QP%6] << (QP/6)) + offset)>>(M-1+DB),  offset = 1<<(M-2+DB)	(3)

where IQ’[QP%6] = ( IQ[QP%6] * (128+delta ) +64) >> 7, and delta is the proposed FINE-QP Y/U/V scaler whose range is within [-15,15] inclusively

Basically, IQ’[QP%6] is used instead of IQ[QP%6]. Since delta is within [-15,15], the max value in IQ’[QP%6] is (72*143+64)>>7=80. Considering that both 72 (max value in IQ[QP%6]) and 80 (max value in IQ’[QP%6]) cost 7 bits, using IQ’[QP%6] instead of IQ[QP%6] will not need bigger dynamic range.

As the scaler delta is signalled at slice level, lookup tables of IQ’[QP%6] may be built for Y, U and V at slice level to save computation. Since there are 6 entries of IQ table, each lookup table only has 6 entries. For each table 1x6 multiplications, 1x6+1=7 additions (1 for “128+delta”, 6 for “+64”) and 1x6 right shifts are needed. Considering 3 lookup tables are needed for Y, U and V, 18 multiplications, 21 additions, and 18 right shifts are required at slice level. Besides the lookup tables, no additional complexity is introduced.

### De-quantization with quantization matrices

In case of quantization matrices, (1) is modified to (4).

Let iShift= M-1+DB+4

If(iShift > QP/6)

coeffQ ij = ((levelij*Mij*IQ[QP%6]+ offset)>>(iShift-QP/6),  offset = 1<<(iShift-1-QP/6), with i=0…nW-1, j = 0..nH-1

else

coeffQ ij= ((levelij*Mij*IQ[QP%6])<<(QP/6-iShift),  with i=0…nW-1, j = 0..nH-1	(4)

where Mij denotes quantization matrix.

To support FINE-QP, it is proposed to change (4) to (5).

If(iShift > QP/6)

coeffQ ij= ((levelij*Mij*IQ’[QP%6]+ offset)>>(iShift-QP/6),  offset = 1<<(iShift-1-QP/6), with i=0…nW-1, j = 0..nH-1

else

coeffQ ij= ((levelij*Mij*IQ’[QP%6])<<(QP/6-iShift),  with i=0…nW-1, j = 0..nH-1	(5)

where IQ’[QP%6] = ( IQ[QP%6] * (128+delta ) +64) >> 7

The same as the case of no quantization matrix, IQ’[QP%6] is used instead of IQ[QP%6] and no bigger dynamic range is needed. To save computation, a lookup table may be built for IQ’[QP%6]. The same as the case of no quantization matrix, 18 multiplications, 21 additions, and 18 right shifts are required at slice level, besides which no additional complexity is required.

# Simulation results

The proposed method is tested according to CE4 description JCTVC-F904. The simulation results are summarized as follows. Please refer to the attached excel files for details. Please also note that the timing information is not reliable.

## Test A

In this test case, AQO is compared to HM4.0 anchor.

Table 1 Test A

## Test B

In this test case, AQO is compared to HM4.0 anchor with following rate constraint: The average bitrate mismatch between AQO and HM4.0 anchor shall be within 2% for each temporal level of in the tests of all sequences and QP settings. For the purpose of obtaining the target bitrate of each temporal level, Lambda tuning is allowed.

To meet the rate constraints, a script tool JCTVC-G678 was used to iteratively tune lambda. Note that this tool is not designed for a best RD performance. Therefore, the coding performance is not as good as that reported in test A.

Note that class A sequences have not been finished for RAHE configuration.

Table 2 Test B

## Test C

In this test case, AQO is compared to HM4.0 + QP rounding.

Table 3 Test C

## Test D

In this test case, AQO is compared to HM4.0 with RDOQ off and EEM on.

Table 4 Test D

## Additional results

In this section, additional results are provided.

### Test A with low QP settings

As suggested in JCTVC-G403, test A with low QP settings (QP=2, 7, 12, 17) is performed in this section. Please note that the reference data is copied from JCTVC-G403.

Table 5 Test A with low QP settings

### Test C’

In this test case, AQO is compared to HM4.0 + MS modified QP rounding (JCTVC-G403).

Table 6 Test C' (Compared to MS’s modified QP rounding)

### Test C’ with low QP settings

In this test case, AQO is compared to HM4.0 + MS modified QP rounding (JCTVC-G403) with low QP settings (QP=2, 7, 12, 17). On average, over 2% luma PCHIP-BD-rate reduction was observed for the four inter cases. In fact, QP rounding does not work properly in low QP cases while FINE-QP performs well.

Table 7 Test C' with low QP settings

### Test A with code patch on decoding complexity and dynamic range issues

Issues of decoding complexity and related dynamic range were raised in quantization BoG. To address these issues, section 3.4 was updated. Accordingly, test A was conducted to check the performance of the proposed method with updated de-quantization process defined in section 3.4.

Compared to Table 1, same coding performance for luma and slightly difference performance for chroma are observed, which indicates that the performance difference introduced by the code update/patch is negligible.

Table 8 Test A with code patch on decoding complexity and dynamic range issues

# Conclusions

FINE-QP is an efficient method with light signaling. It brings gain in all test scenarios including low QP cases. As QP rounding like encoder only techniques causes loss in some cases, it is proposed to adopt FINE-QP in HEVC and HM5.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).