| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G852 |
| --- | --- |

| Title: | Cross-check report on harmonization of implicit TU, AMP and NSQT (JCTVC-G519) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Il-Koo Kim | Email: | ilkoo.kim@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Cross-check on JCTVC-G519

In this document, report on harmonization of implicit TU, AMP and NSQT (JCTVC-G519) is provided. In JCTVC-G519, implicit TU split is additionally defined for AMP. More specifically, 2NxnU and 2NxnD have same split with 2NxN, and nLx2N and nRx2N have same split for Nx2N.

Coding efficiency gains from this modification are 0.1% for RA, 0.1% for RA_LC, 0.2% for LD_B, 0.1% for LD_B_LC, 0.1% for LD_P and 0.1% for LD_P_LC, respectively without significant encoding and decoding run time change.

Software was provided by the proponent and it is confirmed that the source code has modified according to the description and the results under common test condition [1] are same with the results of JCTVC-G519.

Table 1. Experimental results of JCTVC-G519 using provided source code by proponent

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.0% | 0.1% | 0.0% | -0.5% | -0.4% |
| Class B | -0.1% | -0.5% | -0.3% | -0.1% | -0.3% | -0.2% |
| Class C | -0.2% | -0.2% | -0.2% | -0.2% | 0.0% | 0.0% |
| Class D | -0.1% | 0.0% | 0.0% | -0.1% | 0.1% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | -0.1% | -0.2% | -0.2% | -0.1% | -0.1% | -0.1% |
| Overall | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
|  | -0.1% | -0.2% | -0.1% | -0.1% | -0.2% | -0.1% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.8% | -0.8% | -0.1% | -0.5% | -0.6% |
| Class C | -0.1% | -0.4% | -0.4% | -0.1% | -0.2% | 0.0% |
| Class D | -0.2% | -0.3% | -0.2% | -0.3% | 0.1% | 0.1% |
| Class E | -0.3% | -0.8% | 0.3% | -0.2% | -0.6% | -0.5% |
| Class F | -0.2% | -0.3% | 0.0% | -0.1% | -0.4% | -0.1% |
| Overall | -0.2% | -0.5% | -0.3% | -0.1% | -0.3% | -0.2% |
|  | -0.2% | -0.5% | -0.3% | -0.1% | -0.3% | -0.2% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.8% | -1.0% | -0.1% | -0.4% | -0.2% |
| Class C | -0.1% | -0.5% | -0.2% | -0.1% | 0.0% | 0.0% |
| Class D | -0.2% | -0.1% | -0.2% | -0.2% | 0.3% | 0.1% |
| Class E | -0.2% | -0.8% | -0.1% | -0.2% | 0.5% | -0.2% |
| Class F | -0.2% | -0.1% | -0.1% | -0.1% | 0.1% | 0.2% |
| Overall | -0.1% | -0.5% | -0.4% | -0.1% | 0.0% | 0.0% |
|  | -0.2% | -0.4% | -0.3% | -0.1% | 0.0% | 0.0% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# References

- F. Bossen, “Common test conditions and software reference configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-F900, Jul. 2011.