| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G854 |
| --- | --- |

| Title: | Crosscheck – On coeff_abs_level_minus3 coding (G700) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | T. Nguyen Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49-30-31002-230 tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results from Fraunhofer HHI for the proposal G700: “On coeff_abs_minus_3 coding”. The software was analyzed and no mismatch was found compared to the results provided by Motorola Mobility.

# Tested Proposal

The tested proposal has been integrated into HEVC HM 4.0. G700 proposes to change control parameters for the binarization of the syntax element coeff_abs_level_minus3. Three modifications are proposed.

- Disable the reset of Rice parameter at the beginning of each subset

- Change the prefix length for the case of Rice parameter is equal to 2

- Minor modification in update rule of the Rice parameter

The second modification can be regarded as a bug fix and has no impact on the coding performance. Regarding the first and the third modification, it should be note that these changes also have minor impact on coding performance in the common test condition. However, further tests should be done for the case of high bitrates, where a critical amount of bins are bypassed with Rice binarization.

# Simulation Environment

The code base for this experiment is the HM 4.0 software. Only high efficiency configuration is verified.

| Type | Cores | CPU | RAM | OS |
| --- | --- | --- | --- | --- |
| 1 | 4x6 | Intel® Xeon® CPU X7460 @ 2.66GHz | 96 GB | Ubuntu 10.10, 64bit |
| 2 | 2x6 | Intel® Xeon® CPU X5670 @ 2.93GHz | 48 GB | Ubuntu 10.10, 64bit |
| 3 | 2x4 | Intel® Xeon® CPU X5570 @ 2.93GHz | 32 GB | Ubuntu 10.10, 64bit |
| 4 | 2x6 | Intel® Xeon® CPU X5675 @ 3.06GHz | 64 GB | Ubuntu 10.10, 64bit |

Due to limited resources in the simulation environment, the runtime is heavily inaccurate and should not be used to infer the complexity of the proposed method.

# Results

Table 1: Fixed prefix length for Rice parameter equal to 2

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00% | -0.02% | 0.00% | -0.02% | -0.07% | -0.04% |
| Class B | 0.00% | 0.00% | 0.01% | 0.00% | 0.01% | 0.01% |
| Class C | 0.00% | 0.00% | 0.01% | 0.00% | 0.01% | 0.00% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.03% | 0.02% |
| Class E | 0.00% | 0.00% | 0.00% |  |  |  |
| Class F | -0.01% | -0.01% | -0.06% | 0.02% | 0.00% | -0.02% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | 0.00% | 0.00% | 0.01% | 0.00% | 0.00% | -0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.01% | -0.12% | 0.03% | 0.01% | -0.06% | 0.03% |
| Class C | 0.01% | -0.03% | 0.04% | 0.00% | 0.01% | -0.02% |
| Class D | 0.02% | -0.04% | 0.01% | 0.00% | 0.01% | 0.00% |
| Class E | 0.00% | -0.07% | 0.05% | 0.00% | -0.06% | -0.06% |
| Class F | 0.03% | -0.12% | 0.17% | -0.01% | 0.04% | -0.11% |
| Overall | 0.01% | -0.07% | 0.03% | 0.00% | -0.02% | 0.00% |
|  | 0.01% | -0.07% | 0.03% | 0.00% | -0.02% | -0.01% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: Non-reset of Rice parameter and new update rules

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.07% | -0.02% | 0.01% | 0.13% | 0.11% | -0.39% |
| Class B | 0.00% | 0.02% | 0.00% | 0.01% | 0.15% | 0.03% |
| Class C | 0.02% | 0.06% | 0.06% | 0.03% | 0.01% | 0.20% |
| Class D | 0.01% | 0.04% | 0.03% | 0.03% | 0.09% | 0.14% |
| Class E | 0.01% | -0.01% | -0.02% |  |  |  |
| Class F | 0.01% | 0.05% | 0.04% | -0.02% | 0.08% | -0.13% |
| Overall | 0.02% | 0.02% | 0.02% | 0.05% | 0.10% | 0.00% |
|  | 0.02% | 0.02% | 0.00% | 0.06% | 0.07% | -0.02% |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.00% | -0.22% | -0.47% | -0.01% | 0.10% | -0.17% |
| Class C | 0.05% | -0.01% | -0.17% | -0.02% | 0.39% | -0.09% |
| Class D | 0.00% | 0.15% | 0.13% | 0.03% | -0.11% | -0.44% |
| Class E | 0.10% | -0.84% | -0.23% | 0.06% | -1.05% | -0.61% |
| Class F | -0.08% | -0.25% | 0.07% | -0.03% | 0.46% | -0.10% |
| Overall | 0.03% | -0.19% | -0.20% | 0.01% | -0.10% | -0.30% |
|  | 0.03% | -0.27% | -0.22% | 0.02% | -0.14% | -0.36% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3: Non-reset of Rice parameter

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.16% | 0.04% | 0.04% | 0.15% | -0.24% | -0.01% |
| Class B | 0.02% | 0.03% | 0.04% | -0.01% | 0.02% | -0.13% |
| Class C | 0.03% | 0.09% | 0.08% | 0.01% | 0.02% | 0.06% |
| Class D | 0.01% | 0.03% | 0.02% | 0.03% | 0.18% | -0.09% |
| Class E | 0.05% | -0.03% | 0.04% |  |  |  |
| Class F | 0.03% | 0.08% | 0.06% | 0.00% | 0.01% | -0.19% |
| Overall | 0.05% | 0.03% | 0.05% | 0.04% | 0.00% | -0.05% |
|  | 0.05% | 0.03% | 0.04% | 0.06% | -0.02% | -0.04% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.04% | -0.03% | -0.13% | 0.03% | -0.14% | -0.08% |
| Class C | 0.04% | -0.04% | -0.14% | 0.00% | -0.01% | -0.01% |
| Class D | 0.01% | 0.00% | 0.00% | 0.09% | -0.35% | 0.28% |
| Class E | 0.07% | 0.08% | 0.85% | 0.13% | -0.42% | -0.71% |
| Class F | -0.17% | -0.13% | 0.02% | -0.05% | 0.03% | -0.08% |
| Overall | 0.04% | 0.00% | 0.09% | 0.06% | -0.21% | -0.09% |
|  | 0.04% | -0.05% | 0.03% | 0.06% | -0.17% | -0.15% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |