| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, 21-30 November, 2011 | Document: JCTVC-G855 |
| --- | --- |

| Title: | Performance evaluation of full search mode decision for Intra of HM4.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Changcai Lai Yongbing Lin Lingzhi Liu Jianhua Zheng | Email: | laichangcai@huawei.com |
| Source: | HiSilicon |  |  |

_____________________________

# Abstract

To evaluate the performance of HM4.0 when the fast intra mode search method is turned off, this information document reports the experimental results of HM4.0 without fast intra mode search method. The average gain is 0.2%/0.3% (AIHE/AILC) with 183%/216% encoding time and 101%/98% decoding time.

# Test description and results

In case 12# of CE10 report proposal JCTVC-D100[1], the performance of TMuC0.9 with full intra mode search was evaluated, in which the best intra prediction mode is selected by evaluating R-D cost for all modes. Table 1 was the results of TMuC0.9 with full intra mode search from JCTVC-D100.

Table 1 performance of TMuC0.9 with full intra mode search

| TEST#12 | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.2 | 0.2 | -1.0 | -0.1 | -0.2 |
| Class B | -0.3 | 0.0 | 0.0 | -1.0 | -0.1 | -0.2 |
| Class C | -0.1 | 0.3 | 0.2 | -0.4 | 0.0 | 0.0 |
| Class D | -0.1 | 0.3 | 0.3 | -0.4 | 0.0 | 0.0 |
| Class E | -0.4 | 0.1 | 0.0 | -0.9 | -0.5 | -0.4 |
| All | -0.2 | 0.2 | 0.1 | -0.7 | -0.1 | -0.1 |
| Enc Time[%] | 208% |  |  | 213% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Since HM4.0 is much better than TMuC0.9, in this proposal, the performance of HM4.0 with full intra mode search was evaluated again for study. A simple code change was made to enable the full intra mode search in HM4.0.

#if INTRA_FULLRD

Int numModesForFullRD = g_aucIntraModeNumAng[ uiWidthBit ];

#else

Int numModesForFullRD = g_aucIntraModeNumFast[ uiWidthBit ];

#endif

Table 2 is the results of HM4.0 with full intra mode search, compared with the anchor of HM4.0 in common test conditions. The result show that enable the full intra mode search, 0.2%/0.3% (AIHE/AILC) bit rate saving is achieved, while the encoding time is about double compared with HM4.0 default configuration for intra case.

Table 2 Performance of HM4.0 with full intra mode search

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.1% | -0.1% | 0.1% | -0.3% | 0.0% | 0.0% |
| Class B | -0.2% | 0.1% | 0.2% | -0.3% | 0.1% | 0.1% |
| Class C | -0.2% | 0.2% | 0.2% | -0.3% | 0.1% | 0.1% |
| Class D | -0.2% | 0.1% | 0.2% | -0.3% | 0.1% | 0.2% |
| Class E | -0.2% | 0.0% | 0.0% | -0.3% | -0.2% | -0.1% |
| Overall | -0.2% | 0.1% | 0.1% | -0.3% | 0.0% | 0.1% |
|  | -0.2% | 0.1% | 0.1% | -0.3% | 0.0% | 0.1% |
| Enc Time[%] | 183% |  |  | 216% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

# References

- Kazuo Sugimoto, “CE10: Summary of CE10 on number of intra prediction directions”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D100, 4th Meeting: Daegu, KR, 20-28 January, 2011.