| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G856 M22429 |
| --- | --- |

| Title: | AHG7: IDCT output range after T+Q+IQ+IT with valid residual inputs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This contribution shares a theoretical proof of the IDCT output range after processing chain of transform, quantization, inverse quantization, and inverse transform. It has been generally understood that quantization errors can lead to significant data range expansion in IDCT output. The attached document proved that for a valid residual input of any kind, the final IDCT output is the range of [-(N+1)*a*ηmax/2 : (N+1)*a*ηmax/2], where N x N is transform size, [-a: a] is input residual range, and ηmax is maximum quantization (coefficient expansion) factor. For the current HM4.0 design in which 32x32 transform is employed, the IDCT output range will be expanded by roughly 5 bits when compared to the input residual range, under the assumption that a reasonable quantizer would only leads to a maximum of factor 2 coefficient expansion (i.e. ηmax <=2). For detailed proof please refer to attached pdf document.