| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G857 WG11 Doc: m22430 |
| --- | --- |

| Title: | SIMD Analysis of Some Core Spatial Transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shevach Riabtsev Advanced Technology Center, P.O. Box 15015, Haifa Israel 31905 | Tel: Email: | +972-522480308 sr22@csr.com |
| Source: | CSR plc group (formerly Zoran Corp.) |  |  |

_____________________________

# Abstract

The purpose of this contribution is to show feasibility and to emphasize approaches for potential SIMD implementations of some core HEVC transforms, namely from F251 and F446. We show that an efficient SIMD implementation is feasible for all transform sizes up to 32x32 for both proposals, but evidence is presented to indicate that the proposal JCTVC-F251 looks more beneficial due to full factorization.

# Introduction

Transform design based on approximations of the discrete-cosine transform have a long history.  The KLT approximation is one of the underlying motivators for the transform design. As it is shown in [4], DCT approaches to KLT as N→∞. This fact motivates to use large transform size to approximate better KLT. On the other hand large transform sizes require dedicated and complex architecture to keep real-time decoding performance.

We take the proposals JCTVC-E243 [1] (and modified JCTVC-F446 [3]) and JCTVC-F251 [2] as real candidates for the final HEVC transform.

Transforms in both proposals can be specified in term of matrix calculation as follows:

Y=TXT'	(0)

Where X is NxN input matrix, T is NxN transform-coefficients matrix. The symbol

“ ′ “ denotes the transpose operation.

Due to matrix structure of the transform (0), one can execute it in two stages: horizontal and vertical:

Z=TX    		horizontal stage

Y=ZT'		vertical stage

For illustration let’s consider the following transform matrices as proposed in JCTVC-E243, see [1]:

4x4 transform matrix:

{64, 64, 64, 64}

{83, 36,-36,-83}

{64,-64,-64, 64}

{36,-83, 83,-36}

8x8 transform matrix:

{64, 64, 64, 64, 64, 64, 64, 64}

{89, 75, 50, 18,-18,-50,-75,-89}

{83, 36,-36,-83,-83,-36, 36, 83}

{75,-18,-89,-50, 50, 89, 18,-75}

{64,-64,-64, 64, 64,-64,-64, 64}

{50,-89, 18, 75,-75,-18, 89,-50}

{36,-83, 83,-36,-36, 83,-83, 36}

{18,-50, 75,-89, 89,-75, 50,-18}

16x16 transform matrix:

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 87 80 70 57 43 25  9 -9-25-43-57-70-80-87-90}

{89 75 50 18-18-50-75-89-89-75-50-18 18 50 75 89}

{87 57  9-43-80-90-70-25 25 70 90 80 43 -9-57-87}

{83 36-36-83-83-36 36 83 83 36-36-83-83-36 36 83}

{80  9-70-87-25 57 90 43-43-90-57 25 87 70 -9-80}

{75-18-89-50 50 89 18-75-75 18 89 50-50-89-18 75}

{70-43-87  9 90 25-80-57 57 80-25-90 -9 87 43-70}

{64-64-64 64 64-64-64 64 64-64-64 64 64-64-64 64}

{57-80-25 90 -9-87 43 70-70-43 87  9-90 25 80-57}

{50-89 18 75-75-18 89-50-50 89-18-75 75 18-89 50}

{43-90 57 25-87 70  9-80 80 -9-70 87-25-57 90-43}

{36-83 83-36-36 83-83 36 36-83 83-36-36 83-83 36}

{25-70 90-80 43  9-57 87-87 57 -9-43 80-90 70-25}

{18-50 75-89 89-75 50-18-18 50-75 89-89 75-50 18}

{ 9-25 43-57 70-80 87-90 90-87 80-70 57-43 25 -9}

32x32 transform matrix:

{64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64 64}

{90 90 88 85 82 78 73 67 61 54 46 38 31 22 13 4 -4 -13- 22 -31 -38 -46 -54 -61 -67 -73 -78 -82 -85 -88 -90 -90}

{90 87 80 70 57 43 25  9 -9 -25 -43 -57 -70 -80 -87 -90 -90 -87 -80 -70 -57 -43 -25 -9  9 25 43 57 70 80 87 90}

{90 82 67 46 22 -4 -31 -54 -73 -85 -90 -88 -78 -61 -38 -13 13 38 61 78 88 90 85 73 54 31  4 -22 -46 -67 -82 -90}

{89 75 50 18 -18 -50 -75 -89 -89 -75 -50 -18 18 50 75 89 89 75 50 18 -18 -50 -75 -89 -89 -75 -50 -18 18 50 75 89}

{88 67 31 -13 -54 -82 -90 -78 -46 -4 38 73 90 85 61 22 -22 -61 -85 -90 -73 -38  4 46 78 90 82 54 13 -31 -67 -88}

{87 57  9 -43 -80 -90 -70 -25 25 70 90 80 43 -9 -57 -87 -87 -57 -9 43 80 90 70 25 -25 -70 -90 -80 -43  9 57 87}

{85 46 -13 -67 -90 -73 -22 38 82 88 54  -4 -61 -90 -78 -31 31 78 90 61  4 -54 -88 -82 -38 22 73 90 67 13 -46 -85}

{83 36 -36 -83 -83 -36 36 83 83 36 -36 -83 -83 -36 36 83 83 36 -36 -83 -83 -36 36 83 83 36 -36 -83 -83 -36 36 83}

{82 22 -54 -90 -61 13 78 85 31 -46 -90 -67  4 73 88 38 -38 -88 -73  -4 67 90 46 -31 -85 -78 -13 61 90 54 -22 -82}

{80  9 -70 -87 -25 57 90 43 -43 -90 -57 25 87 70  -9 -80 -80  -9 70 87 25 -57 -90 -43 43 90 57 -25 -87 -70  9 80}

{78  -4 -82 -73 13 85 67 -22 -88 -61 31 90 54 -38 -90 -46 46 90 38 -54 -90 -31 61 88 22 -67 -85 -13 73 82  4 -78}

{75 -18 -89 -50 50 89 18 -75 -75 18 89 50 -50 -89 -18 75 75 -18 -89 -50 50 89 18 -75 -75 18 89 50 -50 -89 -18 75}

{73 -31 -90 -22 78 67 -38 -90 -13 82 61 -46 -88  -4 85 54 -54 -85  4 88 46 -61 -82 13 90 38 -67 -78 22 90 31 -73}

{70 -43 -87  9 90 25 -80 -57 57 80 -25 -90  -9 87 43 -70 -70 43 87  -9 -90 -25 80 57 -57 -80 25 90  9 -87 -43 70}

{67 -54 -78 38 85 -22 -90  4 90 13 -88 -31 82 46 -73 -61 61 73 -46 -82 31 88 -13 -90  -4 90 22 -85 -38 78 54 -67}

{64 -64 -64 64 64 -64 -64 64 64 -64 -64 64 64 -64 -64 64 64 -64 -64 64 64 -64 -64 64 64 -64 -64 64 64 -64 -64 64}

{61 -73 -46 82 31 -88 -13 90  -4 -90 22 85 -38 -78 54 67 -67 -54 78 38 -85 -22 90  4 -90 13 88 -31 -82 46 73 -61}

{57 -80 -25 90  -9 -87 43 70 -70 -43 87  9 -90 25 80 -57 -57 80 25 -90  9 87 -43 -70 70 43 -87  -9 90 -25 -80 57}

{54 -85  -4 88 -46 -61 82 13 -90 38 67 -78 -22 90 -31 -73 73 31 -90 22 78 -67 -38 90 -13 -82 61 46 -88  4 85 -54}

{50 -89 18 75 -75 -18 89 -50 -50 89 -18 -75 75 18 -89 50 50 -89 18 75 -75 -18 89 -50 -50 89 -18 -75 75 18 -89 50}

{46 -90 38 54 -90 31 61 -88 22 67 -85 13 73 -82  4 78 -78  -4 82 -73 -13 85 -67 -22 88 -61 -31 90 -54 -38 90 -46}

{43 -90 57 25 -87 70  9 -80 80  -9 -70 87 -25 -57 90 -43 -43 90 -57 -25 87 -70  -9 80 -80  9 70 -87 25 57 -90 43}

{38 -88 73  -4 -67 90 -46 -31 85 -78 13 61 -90 54 22 -82 82 -22 -54 90 -61 -13 78 -85 31 46 -90 67  4 -73 88 -38}

{36 -83 83 -36 -36 83 -83 36 36 -83 83 -36 -36 83 -83 36 36 -83 83 -36 -36 83 -83 36 36 -83 83 -36 -36 83 -83 36}

{31 -78 90 -61  4 54 -88 82 -38 -22 73 -90 67 -13 -46 85 -85 46 13 -67 90 -73 22 38 -82 88 -54  -4 61 -90 78 -31}

{25 -70 90 -80 43  9 -57 87 -87 57  -9 -43 80 -90 70 -25 -25 70 -90 80 -43  -9 57 -87 87 -57  9 43 -80 90 -70 25}

{22 -61 85 -90 73 -38  -4 46 -78 90 -82 54 -13 -31 67 -88 88 -67 31 13 -54 82 -90 78 -46  4 38 -73 90 -85 61 -22}

{18 -50 75 -89 89 -75 50 -18 -18 50 -75 89 -89 75 -50 18 18 -50 75 -89 89 -75 50 -18 -18 50 -75 89 -89 75 -50 18}

{13 -38 61  -78 88 -90 85  -73 54 -31  4 22 -46 67 -82 90 -90 82 -67 46 -22  -4 31 -54 73 -85 90 -88 78 -61 38 -13}

{ 9 -25 43  -57 70 -80 87 -90 90 -87 80 -70 57 -43 25  -9  -9 25 -43 57 -70 80 -87 90 -90 87 -80 70 -57 43 -25  9}
{ 4 -13 22 -31 38  -46 54  -61 67 -73 78 -82 85 -88 90 -90 90 -90 88  -85 82  -78 73  -67 61 -54 46 -38 31  -22 13  -4}

For our discussion we insert the following two definitions:

- Mirror symmetry

The 2Nelements vector V possesses the mirror symmetry if  Vj=Vj+2N−1 for each j=1...2N−1

- Mirror anti -symmetry

The 2Nvector V possesses mirror anti -symmetry if  Vj=−Vj+2N−1 for each j=1...2N−1

One can see that each row of the above transform matrices indeed possesses either mirror symmetry or anti -symmetry. The same is true for transform matrices in JCTVC-F251 proposal. This feature is crucial for SIMD implementations since it facilitates of using butterfly operations.

# Points on SIMD Architecture with 32-bits precision

In this section we describe a schematic SIMD architecture which is tailored to support all transform sizes for both JCTVC-F446 and JCTVC-F251 proposed transforms with the assumption that all transform operations never exceed 32 bits precision. Despite in both JCTVC-F251 and JCTVC-F446 proposals the dynamic range between transform stages is limited to 16 bits but intermediate results can exceed 16-bits precision therefore in this section we build the SIMD architecture with 32-bits precision. In the next section we provide a schema for the SIMD architecture taking into account 16-bits restrictions between transform stages.

Because the minimal transform granularity is 4x4 we design the SIMD processor to execute simultaneously four identical arithmetic operations. Therefore all register in the SIMD are designed to be 32*4 bits length to keep four result values. In other words each register contains four 32 -bits words, denoted as x3, x2, x1, x0. We also denote Rx[r] as r-th 32-bits word in the register Rx, where r=0,1,2 or 3.

For example Ra[0] denotes the first word of the register Ra, i.e. Ra[0] equals to [31:0] lsb bits in the Ra register, Ra[1] are [63:32] bits and so forth.

## Instructions Set

We specify the following basic (and very limited in number) set of SIMD instructions to execute proposed transforms:

### Vadd – vector addition

Syntax:

Rd = vadd Ra, Rb

where Ra is the first input operand

Rb is the second input operant

Rd is destination register.

Description:

For each r=0 to 3 do

Rd[r]=Ra[r]+Rb[r]

### Vpbtfly – butterfly operation with addition

Syntax:

Rd = vpbtfly Ra, Rb, gain

Ra is first input operand

Rb is the second input operant

gain is the immediate value from the range [0..127]

Rd is destination register

Description:

For each r=0 to 3 do

Rd[r]=gain*Ra[r]+gain*Rb[r]

### Vnbtfly – butterfly operation with subtraction

Syntax:

Rd = vnbtfly Ra, Rb, gain

Ra is first input operand

Rb is the second input operant

gain is immediate value from the range [0..127]

Rd is destination register

Description:

For each r=0 to 3 do

Rd[r]=gain*Ra[r]  - gain*Rb[r]

### Vmrgh – merging operation (h -mode)

Syntax:

Rd = vmrgh Ra, Rb

Ra is first input operand

Rb is the second input operant

Rd is destination register

Description:

Required for transpose.

Let Ra=[a3, a2,a1,a0]

Rb = [b3,b2,b1,b0]

Then

Rd = [a3, b3, a2, b2]

### Vmrgl – merging operation (l -mode)

Syntax:

Rd = vmrgl Ra, Rb

Ra is first input operand

Rb is the second input operant

Rd is destination register

Description:

Required for transpose.

Let Ra=[a3, a2,a1,a0]

Rb = [b3,b2,b1,b0]

Then

Rd = [a1, b1, a0, b0]

### Vload – load command

Syntax:

Rd = vload AddrReg, Offset

AddrReg – base address where data is located

Offset – immediate value, denotes the offset from the base address

Rd is destination register

Description:

Rd = MEM[ArddrReg+Offset]

### Vsave – save/store command

Syntax:

vsave Ra, AddrReg, Offset

AddrReg – base address where data is located

Offset – immediate value, denotes the offset from the base address

Ra – data to save

Description:

MEM[ArddrReg+Offset]=Ra

## SIMD Code Design

Basing on the assumption that transform matrices of any potential HEVC transform possesses mirror symmetry or anti -symmetry we present the following SIMD code, where transform matrices are taken for the sake of illustration from JCTVC -E243 [1].

### Transform 4x4 Code

The output matrix Y is calculated as follows:

y00y01y02y03y10y11y12y13y20y21y22y23y30y31y32y33=646464648336−36−8364−64−646436−8383−36x00x01x02x03x10x11x12x13x20x21x22x23x30x31x32x33648364366436−64−8364−36−648364−8364−36

where

X=x00x01x02x03x10x11x12x13x20x21x22x23x30x31x32x33 is input matrix

T=646464648336−36−8364−64−646436−8383−36  is transform matrix

We execute 4x4 transform in two stages: horizontal transform Z=TXand then vertical transform Y=ZT'.

The elements of the output matrix Z (after horizontal transform) can be expressed as follows:

Z0k=64x0k+64x1k+64x2k+64x3k	(1)

Z1k=83x0k+36x1k−36x2k−83x3k		(2)

Z2k=64x2k−64x2k−64x2k+64x3k	(3)

Z3k=36x0k−83x1k+83x2k−36x3k	(4)

where k=0..3

We hold the input 4x4 matrix X in four registers, each register actually keeps a single row of X:

R0=x00x01x02x03

R1=x10x11x12x13

R2=x20x21x22x23

R3=x30x31x32x33

The following SIMD pseudo -code performs the horizontal transform, i.e. computes all elements of Z:

Horizontal Transform Code

{load input matrix }

R0 = vload A0, 0   { load first row}

R1 = vload A0, 1  { load second row}

R2 = vload A0, 2  { load third row}

R3 = vload A0, 3 { load fourth row}

{from formula (1) compute first row of Z, keep in R4 register}

R8 = vpbtfly R0, R1, 64

R9 = vpbtfly  R2,R3, 64

R4 = vadd R8,R9

{from formula (2) compute second row of Z, keep in R5 register}

R8 = vnbtfly R0, R3, 83

R9 = vnbtfly  R1,R2, 36

R5 = vadd R8,R9

{from formula (3) compute third row of Z, keep in R6 register}

R8 = vnbtfly R0, R1, 64

R9 = vnbtfly  R3,R2, 36

R6 = vadd R8,R9

{from formula (4) compute  fourth row of Z, keep in R7 register}

R8 = vnbtfly R0, R3, 36

R9 = vnbtfly  R2,R1, 83

R7 = vadd R8,R9

Notice that after the horizontal transform, the matrix Z is entirely kept in four registers - R4, R5, R6 and R7.

The output matrix Y can be expressed as follows:

Y'=(ZT')'=TZ'

So, if we transpose the matrix Z we can perform the vertical transform in the same manner as the horizontal one, i.e. we firstly transpose the matrix Z then apply the same code of the horizontal transform and finally transpose the output matrix. More schematically the vertical transform is divided into the following functional blocks:

- Transpose Z

- Compute W=TZ' (remind that W=Y')

- Transpose W (to get Y, since W=Y')

- Store Y

Vertical Transform Code

{from horizontal transform the matrix Z is kept in R4,R5,R6 and R7 registers}

{R4 = [z00, z01, z02, z03]  }

{R5 = [z10, z11, z12, z13] }

{R6 = [z20, z21, z22, z23]}

{R7 = [z30, z31, z32, z33]}

{transpose Z, transposed matrix is kept in registers R0,R1,R2 and R3}

R8 = vmrgh R4, R6  	  { R8 = [z00, z20,z01,z21]}

R9 = vmrgl R4, R6 	 { R9=[z02,z22,z03,z23] }

R10 = vmrgh R5, R7 	{ R10=[z10,z30,z11,z31]}

R11 = vmrgl R5,R7    { R11 = [z12,z32,z13,z33] }

R0 = vmrgh R8,R10   { R0 = [z00, z10,z20,z30] }

R1 = vmrgl R8, R10  { R1 = [z01, z11, z21,z31] }

R2 = vmrgh R9,R11   { R2 = [z02, z12,z22,z32] }

R3 = vmrgl R9, R11  { R3 = [z03, z13, z23,z33] }

{from formula (1),compute  first row of W, keep in R4 register}

R8 = vpbtfly R0, R1, 64

R9 = vpbtfly  R2,R3, 64

R4 = vadd R8,R9

{from formula (2) compute second row of W, keep in R5 register}

R8 = vnbtfly R0, R3, 83

R9 = vnbtfly  R1,R2, 36

R5 = vadd R8,R9

{from formula (3) compute third row of W, keep in R6 register}

R8 = vnbtfly R0, R1, 64

R9 = vnbtfly  R3,R2, 36

R6 = vadd R8,R9

{from formula (4) compute fourth row of W, keep in R7 register}

R8 = vnbtfly R0, R3, 36

R9 = vnbtfly  R2,R1, 83

R7 = vadd R8,R9

{ registers R4,R5,R6 and R7 contains transposed Y matrix, i.e. W matrix }

{ transpose W to get Y}

R8 = vmrgh R4, R6

R9 = vmrgl R4, R6

R10 = vmrgh R5, R7

R11 = vmrgl R5,R7

R0 = vmrgh R8,R10

R1 = vmrgl R8, R10

R2 = vmrgh R9,R11

R3 = vmrgl R9, R11

{ Output Y is stored in registers R0, R1, R2 and R3 }

Vsave R0, A1, 0

Vsave R1, A1, 1

Vsave R2, A1, 2

Vsave R3, A1, 3

The whole 4x4 transform can be executed by SIMD with

- 16 merge operations (two transposes of 8 merge commands each)

- 16 butterfly operations (8 butterflies in horizontal transform and 8 butterflies in vertical transform).

- 8 addition operations (4 additions in horizontal transform and 4 additions in vertical transform).

- 4 loads and 4 stores.

Total: 48 operations

### Transform 8x8 Code

Like in 4x4 case the 8x8 transform is divided into horizontal and vertical transforms.

The output of the horizontal transform is Z=TXand the output of vertical transform is Y=ZT'.

Horizontal Transform:

Due to mirror symmetry in the 8x8 transform matrices in both proposals, 2 loads, 4 butterflies and 3 additions are required for each row. Totally (4+3)*8+16= 72 commands.

Vertical Transform:

Like in 4x4 transform the vertical 8x8 transform consists of the following three parts:

- Transpose Z

- Compute W=TZ'

- Transpose W to get Y

In order to show how transpose 8x8 matrix with 4x4 transposes let’s take an arbitrary 8x8 matrix R and represent it as composed by four 4x4 blocks:

R=ABCD

Then the transposed matrix R' is:

R'=A'C'B'D'

In order to transpose the matrix R we need transpose four sub-matrices A,B,C and D.

According to the previous section the transpose of a single 4x4 matrix takes 8 merge operations (vmrgh and vmrgl commands). Hence the transpose of 8x8 matrix R requires 8x4=32 merge operations. So, two transposes (one of Z and another of W) in the vertical 8x8 transform take totally 64 merge commands.

The whole 8x8 transform can be executed by SIMD with

- 16 loads (to load 8x8 input matrix)

- 56 arithmetic commands to perform 8x8 horizontal transform.

- 32 + 56 + 32 = 120 commands to perform 8x8 vertical transform and transposes.

- 16 stores

Total: 16 + 56 + 120 + 16 = 208 commands.

### Transform 16x16

Like in 4x4 and 8x8 modes the 16x16 transform is also divided into horizontal and vertical stages.

Horizontal Transform:

Due to mirror symmetry/anti-symmetry of the 16x16 transform matrices in both proposals, 4 loads, 8 butterflies and 7 additions are required for each row. Totally (8+7)*16+4*16= 240+64=304 commands.

Vertical Transform:

Like in 8x8 transform the vertical 16x16 transform consists of the following three parts:

- Transpose Z

- Compute W=TZ'

- Transpose W to get Y

Let’s take an arbitrary 16x16 matrix R and represent it as composed by four 8x8 blocks:

R=ABCD

Then the transposed matrix R' is expressed as:

R'=A'C'B'D'

In the previous section we show that the transpose of 8x8 matrix takes 32 merge instructions. Hence the transpose of the entire 6x16 matrix can be executed with 32*4=128 merge instructions.

The whole 16x16 transform can be executed by SIMD with

- 64 loads

- 240  commands to perform 16x16 horizontal transform.

- 128 + 240 + 128 = 752 commands to perform 16x16 vertical transform and transposes

- 64 stores

Total:  64 + 240 + 752 + 64 = 1120 commands.

### Transform 32x32

Like in 4x4, 8x8 and 16x16 modes the 32x32 transform is divided into horizontal and vertical phases.

Horizontal Transform:

Due to mirror symmetry/anti-symmetry of the 32x32 transform matrices in both proposals, 8 loads, 16 butterflies and 15 additions are required for each row. Totally (16+15)*32+8*32= 992+256=1248 commands.

Vertical Transform:

Like in 16x16 transform the vertical 32x32 transform consists of the following three parts:

- Transpose Z

- Compute W=TZ'

- Transpose W to get Y

Let’s take an arbitrary 32x32 matrix R and represent it as composed by four 16x16 blocks:

R=ABCD

Then the transposed matrix R' is expressed as:

R'=A'C'B'D'

In the previous section we show that the transpose of an arbitrary 16x16 matrix takes 128 merge instructions. Hence the transpose of the whole 32x32 matrix can be executed with 128*4=1024 merge instructions.

The whole 32x32 transform can be executed by SIMD with

- 256 loads

- 992 commands to perform 32x32 horizontal transform.

- 1024 + 992 + 1024 = 2016 commands to perform 32x32 vertical transform.

- 256 stores

Total:  256 + 992 + 2016 + 256 = 3552 commands.

## Conclusion

Count of SIMD operations per transform mode is summarized in the following table:

Table 1:

| Transform Mode | Total instructions | Merge cmds | Butterflies | Additions | Loads/Stores |
| --- | --- | --- | --- | --- | --- |
| 4x4 | 48 | 16 | 16 | 8 | 8 |
| 8x8 | 208 | 64 | 64 | 48 | 32 |
| 16x16 | 1120 | 512 | 256 | 224 | 128 |
| 32x32 | 3552 | 2048 | 1024 | 960 | 512 |

### Performance Analysis

Basing on data in the Table 1 we provide the performance analysis of all transforms of 1920x1080 frame. Notice that we restrict our analysis on luma blocks only.

Table 2:

| Block Size | # Luma Blocks | # instructions to execute | Time in ms per frame (300Mhz) |
| --- | --- | --- | --- |
| 4x4 | 129600 | 6220800 | ~20 ms |
| 8x8 | 32400 | 6739200 | ~22 ms |
| 16x16 | 8100 | 9072000 | ~30 ms |
| 32x32 | 2025 | 7192800 | ~23 ms |

The figures in the above table show that all transforms in frame can be executed at 30 fps rate on 300Mhz SIMD processor.

Notes:

- Chroma blocks are not taken into consideration since transform of chroma data can be carried out in parallel to luma blocks.

# Points on SIMD architecture with 16-bits precision

According to JCTVC-F446 [3] and JCTVC-F251 [2] the 16-bits dynamic range is kept by shifts between horizontal and vertical stages of the transform but intermediate results can exceed 16-bits range.

Therefore a potential SIMD architecture should include two kinds of registers: regular registers of 16xN bits (where N=4 to hold four results) and accumulator registers of 32xN bits.

In spirit of said above the butterfly instruction:

Racc0 = vpbtfly R0, R1, 64

is fed by two regular registers R0 and R1 of 16x4 bits width each and puts the result into the accumulator register Racc0 of 32x4 bits width.

Because the results of butterfly operations (which of course are intermediate) are further added/subtracted the adder has to support 32-bits precision to hold all intermediate results but the multiplier sufficiently supports 16 bits by 16 bits multiplication.

For illustration let's consider the horizontal 4x4 transform code:

Racc8 = vpbtfly R0, R1, 64

Racc9 = vpbtfly  R2,R3, 64

Racc4 = vadd Racc8,Racc9    { activate adder with 32 bits precision }

Racc8 = vnbtfly R0, R3, 83

Racc9 = vnbtfly  R1,R2, 36

Racc5 = vadd Racc8,Racc9   { activate adder with 32 bits precision }

Racc8 = vnbtfly R0, R1, 64

Racc9 = vnbtfly  R3,R2, 36

Racc6 = vadd Racc8,Racc9

Racc8 = vnbtfly R0, R3, 36

Racc9 = vnbtfly  R2,R1, 83

Racc7 = vadd Racc8,Racc9  { activate adder with 32 bits precision }

{ shift operations, can be combined with 'vadd' in more advanced SIMD design}

R4 = vshift Racc4, ShftVal

R5 = vshift Racc5, ShftVal

R6 = vshift Racc6, ShftVal

R7 = vshift Racc7, ShftVal

# Benefits of JCTVC-F251 proposal over JCTVC-F446

Upon analyzing of both proposals the author concludes that the proposal JCTVC-F251 is crucially beneficial for VLSI design than the alternative JCTVC-F446. The reason is full-factorization of data flow in both forward and inverse transforms.

This enables low-cost implementation of dedicated Transform Processor and IC-unit.

For illustration let’s consider the signal flowgraph in Fig 1. of JCTVC-F251, the flowgraph reveals the following architecture solution for 32x1 row or column transform:

Notice that Both Top and Bottom Processing units can be divided into individual serially dependent butterfly stages, which leads to an efficient, pipelined hardware implementation.  This is not true of F446, which leads to considerable inefficiencies, and is the main reason that F446 is not preferred.

# References

[1] – JCTVC-E243

[2] – JCTVC-F251

[3] – JCTVC-F446

[4]  - "Discrete Cosine Transform", K.R. Rao, P. Yip

# Patent rights declaration(s)

CSR plc group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).