| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G862 WG11 Doc: m22439 |
| --- | --- |

| Title: | Advanced Resampling Filters for HEVC Applications |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Dai, Madhu Krihnan, Pankaj Topiwala 5840 Banneker Rd., #230 Columbia, MD 21044 USA | Tel: Email: | +1-410-730-6922 pankajtva@gmail.com |
| Source: | FastVDO LLC |  |  |

_____________________________

# Abstract

Resampling filters are commonly used in a variety of image processing tasks.  They are intimately related to two- and multi-channel filter banks, as well as to the well known Laplacian Pyramid. They arise in video coding applications in several instances: (a) spatial scalability, (b) resolution adaptation, and (c) color sampling, as part of 4:4:4 coding. All three application spaces are currently being explored in the HEVC project. We propose a single, general design methodology, and specific sampling filter instances, which can reportedly meet all of these demanding applications simultaneously. This proposal builds on previous work proposed at the Torino meeting [9].

# Introduction

Spatial resampling filters are in wide use in image and video processing applications, as there is often a mismatch between the native resolution of coded video data and that of target displays. Spatial resampling is even used within the context of video coding itself, such as in spatial scalability, where a mixed representation requiring multiple resolutions is an explicit part of the coding architecture.  In SVC for example, resampling filters are standardized (more specifically, the upsampling filter, needed in the decoder, is standardized). Note that no matter how well designed, the use of resampling filters nearly always involves a serious loss of information (when applied in contexts other than perfect reconstruction filter banks), which one naturally wishes to minimize. Resampling filters are also regularly used with little notice for color space representations, as most video codecs take YUV 4:2:0 as input, whereas the native representation of acquired video is typically RGB (4:4:4). Similarly to spatial resampling, there is information lost in the color space resampling to YUV 4:2:0 which is forever gone, so that that conversion needs to be of high quality. In fact, for certain applications, YUV 4:2:0 may be entirely inadequate, and working in fully 4:4:4 is required. In the development of HEVC, 4:4:4 coding is already part of the mandate, although an explicit timetable for work in this area has yet to be formulated. This disconnect between 4:4:4 and 4:2:0 has recently emerged as a possible issue on the reflector for the coding of screen content video in HEVC. Likewise, scalability is also of current interest in the HEVC development. In this proposal, we develop and discuss advanced spatial filters which can potentially provide improved quality for both spatial resampling, as well as chroma sampling.

In spatial resampling, one is often faced with having to select filters that perform well in generic situations, and applied in contexts where other processing elements may be unknown.  In particular, if downsampling and upsampling filters are independently developed, and in practice, unknown to each other, they must be both 'generic' low-pass filters.  While each may be optimized in this generic sense, the combination of such generically optimal filters may indeed be quite suboptimal. In this proposal, we present some FIR “nearly low-pass” filter designs for dyadic down-sampling and up-sampling filters, which, when properly co-designed, significantly outperform the generic low-pass filters. The proposed filters have their roots from the wavelet and spline interpolation theories, which has long been established to have excellent interpolation characteristics. Specifically, we propose an 11-tap downsampling filter that is designed to minimize aliasing artifacts in the sub-sampled images. The impulse response of this symmetric filter actually only contains 7 non-zero dyadic coefficients, so its computational complexity is quite modest. In contrast, the high-performance but generic downsampler we compare to is the 13-tap filter used in the SVC design.

On the upsampling side, we propose a dyadic symmetric designs with length 7, which is effectively only a 4-tap filter. It come from a family of interpolative splines called O-MOMS (optimal maximal-order minimal-support splines, designed for best least-square performances [2]). All of the proposed filters have integer coefficients; some with very low dynamic range, and hence, very hardware-friendly. This proposal also illustrates that coding efficiency does not necessarily have to be sacrificed by employing short low-complexity integer-coefficient filters.

# Spatial Sampling Considerations

2.1. Downsampling Filter Design

Instead of optimizing the low-pass filter such that its frequency response has steep transition characteristics to match the ideal low-pass box filter, we opt for a smoother, slower-decaying frequency response. In general, we observe that filters which allow a little aliasing (to capture a bit more image information) seem to outperform filters with good anti-aliasing characteristics and good wavelet filters tend to perform well here. In line with a previous contribution in SVC [1], we proposed the following filters for dyadic subsampling:

- h5 = [ -1  2  6  2  -1 ] / 8: 5-tap dyadic-coefficient filter, the exact filter used by JPEG2000 for lossless coding.

- h9 = [ 1  -1  -3  9  20  9  -3  -1  1 ] / 32: 9-tap dyadic-coefficient filter, a close approximation of the default 9-tap irrational-coefficient Daubechies wavelet filter in JPEG2000;

- h4 = [ -1  3  3  -1 ] / 4: 4-tap dyadic-coefficient filter;

- h8 = [ 3  -9  -7  45  45  -7  -9  3 ] / 64: 8-tap dyadic-coefficient filter.

- h11 = [1  0  -3  0  10  16  10  0  -3  0  1] / 32.

- svc13 = [ 2 0 -4 -3 5 19 26 19 5 -3 -4 0 2] / 64

The h11 filter is an 11-tap dyadic-coefficient half-band filter designed to minimize aliasing effects in subsampled images. Although its support is 11, since it is half-band, the number of non-zero weights is only seven. Furthermore, with its symmetry, it can be computed with only four multiplications. And in fact, because of dyadic rational coefficients, these convert to just shifts and adds. In fact, its operational complexity count in terms of number of binary shifts and additions required in the implementation is even lower than that of the 8-tap and 9-tap filters.

The frequency responses of all of the proposed filters, even-length as well as odd-length, are depicted in Figure 1 along with the default 13-tap low-pass filter in SVC (in blue). All of the filters have linear phase, a critical requirement for imaging applications and fast implementation. It is clear that our last solution h11 (in red) has much improved stop-band attenuation, critical in mitigating aliasing artifacts.  We will focus on the h11 downsampler.  Contrast this to the 13-tap filter used in the SVC reference software, svc13. This design has more taps, fewer zeros, and larger coefficients, making it substantially more complex than the h11 design.  As expected, the svc13 design has a nice, low-pass filter characteristic, with a steep dropoff in the frequency response.  The h11 design is reasonably close in that respect, though less clearly a half-band filter.

Figure 1. Frequency responses of proposed downsampling low-pass filters.

2.2. Upsampling Filter Design

Generally, we found that filters with good anti-aliasing characteristics with smooth frequency responses (maxflat characteristic) tend to perform well in upsampling. An 11-tap filter studied during the SVC development, derived from an order-5 Lanczos interpolation family, has both of these properties:

- l11 = [1     0    -5     0    20    32    20     0    -5     0     1] / 32.

In our previous contribution [1], two short up-sampling filters were proposed based on the concept of biorthogonal partners [3]. In other words, these are designed from approximated spectral factorization of various max-flat half-band filters from wavelet theory. Finally, sharp frequency transition was always sacrificed for a higher degree of smoothness/regularity.

- f4 = [ 1  3  3  1 ] / 4; designed to partner with the 8-tap down-sampling filter h8 from the previous section.

- f7 = [ -1  0  9  16  9  0  -1 ] / 16; designed to partner with the 9-tap down-sampling filter h9 from the previous section. This filter turns out to be equivalent to the order-3 Catmull-Rom interpolation spline and it is the same dyadic solution proposed in [5].

Note that a well-designed dyadic interpolation filter such as this one, which is designed to be applied after the signal is upsampled by adding zeros in between samples, in fact reproduces the original signal samples, and replaces the zeros with values filtered from the original samples. In that context, this filter really acts as a 4-tap filter [-1 9 9 -1] / 16. That is, the 7-tap up is really a 4-tap.

In this proposal, we also introduce the following two  new filters for dyadic upsampling. Both filters are derived from the family of interpolative splines called O-MOMS (optimal maximal-order minimal-support) designed for best least-square performances [2]. We refer the reader to JVT-V030 [4] for the design details of these interpolating filters.

- f7 = [-1     0     5     8     5     0    -1] / 8: 7-tap dyadic-coefficient filter; the equivalence of our length-4 interpolation based on order-3 O-MOMS spline. Again, it is a 4-tap, [-1 5 5 -1] / 8.

- f11 = [1     0    -3     0    10    16    10     0    -3     0     1] / 16: 11-tap dyadic-coefficient interpolation filter; the equivalence of our length-6 interpolation based order-5 O-MOMS spline. This is actually 6-tap filter, [1 -3 10 10 -3 1] / 16.

The frequency responses of all aforementioned low-pass filters for upsampling are depicted in Figure 2 below.  The new 11-tap filter f11 from MOMS spline comes closest to the box filter response of the ideal sinc interpolation function. If complexity is a concern, the new 7-tap solution f7 provides a good trade-off. Its frequency characteristic is very similar to that of the 11-tap MOMS filter f11 or the 11-tap Lanczos filter l11.

The accompanying Excel files and the reconstructed sequences demonstrate that the combination of our proposed down- and up-sampling filters offers improved performances both in the minimum mean-squared error sense as well as in visual reconstruction sense, especially when only the base layer is received at the decoder.

Figure 2. Frequency responses of upsampling low-pass filters.

These filters may be compared to the 4-tap filter standardized in the SVC design.

Svc4 = [ -3 19 19 -3] / 32.

In summary, we will compare the SVC 13 down and 4 up (SVC 13d 4u) with the system FV 11 down and 4 up (FV 11d 4u) in experiments.

2.3. Spatial Scalability Experiments

We will consider only the special dyadic case of two spatial resolutions that are factor of 2 related (such as CIF and QCIF). In scalability applications, one often encodes using a mixed combination of codes drawn from both resolutions, and wherein the lower resolution video is used to interpolate the higher resolution one.  Although scalable coding is a complicated application, we will zero-in on the element of downsampling to create a lower resolution video, and upsampling it back to reconstruct it.  We formulate this issue as two very specific problems.

Problem 1. In the first instance, we  greatly simplify the problem to consider only this bare bones issue: for a given video A at resolution X, what video B at resolution X/2 can one construct such that, when it is interpolated back to A' at resolution X, then the Error = A – A' has minimum mean-square.

Problem 2. In the second instance, if the lower resolution video B is encoded and decoded to B', then the reconstructed video A' is such that again the Error = A – A' has minimum mean-square.  For convenience, we focus on just Intra coding for experiments, to minimize the impact of other tools not in test.

Results of our preliminary investigations in this domain, first presented in the context of the SVC development, are reproduced here in accompanying excel files (from JVT-V031).  Example figures are presented below.  This addresses problem 1.  A consistent gain of approximately 1 dB has been seen on a variety of video content, in the absence of compression. This is documented in the spreadsheet SpatialFilterStudy-v3.

To address problem 2, one can anticipate that since there are significant gains in the absence of compression, they should persist for modest levels of compression, and gradually diminish with higher compression levels.  This is precisely what is observed.  As examples of results one can obtain with the new HEVC codec (Intra only), we see that even under compression, the gain obtained by using the more advanced proposed filters continue to hold, but diminish the higher the quantization (which is expected). A spreadsheet is provided for details (SpatFilterStudy-v3).  Compression with configurations other than Intra have not been tested, as it is anticipated that the effectiveness of these sampling methods cannot be adequately tested in configurations where the underlying video contest is highly estimated/compensated.

# Chroma Sampling Considerations

It turns out, these ideas in spatial sampling can also be applied for chroma sampling directly.  While in principle one could specialize the filters for the different contexts to obtain optimal results, just to prove our point, we use the very same low complexity filters for our chroma sampling work as for spatial.

3.1. RGB 4:4:4 to YUV 4:2:0

Numerous ITU recommendations and ISO/IEC standards begin with discussing chroma sampling, as most profiles in video standards use YUV 4:2:0 as input, while several more advanced profiles may work with 4:2:2 or 4:4:4. Such is the case with ITU/H.264 | ISO/IEC MPEG-4 Part 10 (“AVC”), and will likely be the case in the standardization of HEVC.  The current effort to develop “Part 1” is focused on the case of YUV 4:2:0 chroma sampling as input. However, the process of producing the various chroma samplings from RGB 4:4:4 has not been standardized to date (to the authors' knowledge). In fact, this process generally involves two steps: a color transform (say from RGB to some version of YUV), followed by a sub-sampling in the chroma components.  The example of 4:2:0 sampling is given below.

3.2. Chroma Sampling Experiments (e.g., for Screen Content Coding)

A. Color Transform and Sampling Filters.

1. We want to test 3 color transforms:

a. YCbCr,

b. YCoCg (MS), and

c. YFbFr (FastVDO). There is also a variant, YFbFrv2.

2. We also want to test 2 sets of down/upsampling filters.

a. Some standard filters based on half-band filtering (SVC 13d 4u)

b. Our advanced filters (FV 11d 4u)

We refer to JVT-I015 [7] for a description of these color spaces.

B. Experiments.

1. No compression.

a. RGB444 -> ColorSpace444 -> 420 -> ColorSpace444 -> RGB444. Measure loss (psnr).

2. With compression.

a. RGB444 -> ColorSpace444 -> 420 ->Encode/Decode-> ColorSpace444 -> RGB444.

Measure R&D performance.

Test results.

Results are in an enclosed spreadsheet.  These results  indicate that the proposed 11-4 filters consistently outperform the 13-4 sampling filters in chroma sampling applications, with an approximately 1dB gain, in the absence of compression.  Again, one would anticipate that these gains would persist for low levels of compression, and diminish with greater compression.  Again, this is what is observed.

To conduct this test, we add the additional variable of color space.  We test 4 color spaces, the standard YcbCr, and three purely integer (reversible) color transforms; YcoCg, YFbFr, and YFbFrv2.

A spreadsheet is provided with these results, ColorSpFilterStudy-v2.  For tests with compression (Intra only), we obtain similar results to the spatial domain, where we maintain gains at low qp, which diminish slowly at higher qp. We will provide those in a spreadsheet. We especially want to apply these techniques to screen content coding, as we anticipate good results.

# Adaptive Resolution Coding (ARC) Considerations

Adaptive Resolution Coding is the topic of AHG18, and specifically in proposal G264 [8], to which FastVDO is a co-proposer.  These same filters are shown to be very effective for this application, where under conditions where channel capacity varies significantly (as in many network streaming environments), it could become necessary to adapt the native resolution of the streamed video to optimize system performance.  In this scenario, our advanced sampling filters can supply strong performance to deliver the highest quality available under the circumstances.

# Conclusions

Our preliminary tests indicate that for both spatial sampling and chroma sampling, our advanced filter design seems to provide a consistent advantage over generic half-band filters.  The use of these filters can have wide application, from spatial scalability to the coding of specialized content such as screen content coding (SCC).  In particular, SCC can be potentially enhanced by working with the 4:4:4 data as input, applying color transforms (several choices available), and explicitly doing chroma sampling prior to applying a YUV 4:2:0 codec.  On reconstruction, we specify a method of reconstructing back to RGB based on the color transform and resampling filters used.  The intelligent co-design of the sampling filters (for down and upsampling) is what permits these advantages.

References

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “High-performance low-complexity re-sampling filters for SVC”, JVT-T041, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 20th meeting, Klagenfurt, AT, Jul. 2006.

- Thierry Blu, Philippe Thevenaz, and Michael Unser, “MOMS: Maximal-order interpolation of minimal support,” IEEE Trans. On Image Processing, vol. 10, No. 7, pp. 1069-1080, Jul. 2001.

- P. P. Vaidyanathan and Bojan Vrcelj, “Biorthogonal partners and applications”, IEEE Trans. on Signal Processing, vol. 49, No. 5, pp. 1013-1028, May 2001.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Extended Spatial Re-sampling Filters For SVC”, JVT-V030, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- Trac D. Tran, Lijie Liu, and Pankaj Topiwala, “Advanced Dyadic Spatial Re-sampling Filters For SVC”, JVT-V031, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 22nd meeting, Marrakech, Morocco, Jan. 2007.

- Gary Sullivan, “Resampling filters for SVC Upsampling”, JVT-R066, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG 18th meeting, Bangkok, Jan. 2006.

- Pankaj Topiwala and Chengjie Tu, “New Invertible Integer Color Transforms Based on Lifting Steps and Coding of 4:4:4 Video”, JVT-I015r7, San Diego, Sept., 2003.

- Thomas Davies, Pankaj Topiwala, “AHG18: Adaptive Resolution Coding (ARC),” JCTVC-G264, Geneva, CH, 21-30 November, 2011.

- Wei Dai, Madhu Krishnan, Pankaj Topiwala, “Advanced Resampling Filters for Spatial Scalability and Chroma Sampling Applications in HEVC”, JCTVC-F618, Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

FastVDO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).