| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G863 WG11 Number: m22440 |
| --- | --- |

| Title: | CE10: Crosscheck of FastVDO/Samsung proposal on core transforms (JCTVC-G266) for high and low QPs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rajan Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Email: | rajanj@qualcomm.com Tel. +1-858-658-4511 |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of FastVDO/Samsung’s proposal on core transforms (JCTVC-G266) for high and low QPs. JCTVC-G266 proposes 4, 8 and 16-point core transforms. For 32-point transform, the 32-point transform proposed in JCTVC-G737 is reused. The proposed 4, 8 and 16-point transforms are reported to provide the capability for lossless coding. The BD-rate results matched those reported by FastVDO/Samsung. The average luma BD-rate difference averaged over AI-HE, RA-HE, LB-HE, AI-LC, RA-LC, and LB-HC configurations was -0.05% for low QP range and 0.0% for high QP range with respect to HM3.0.

# Introduction

This contribution provides a summary of a cross-check performed by Qualcomm of FastVDO/Samsung’s proposal on core transforms (JCTVC-G266) for high and low QPs. It proposes 4, 8 and 16-point core transforms. For 32-point transform, the 32-point transform proposed in JCTVC-G737 is reused. The proposed 4, 8 and 16-point transforms are reported to provide the capability for lossless coding. This is accomplished by decomposing a butterfly operation into 3 lifting steps. The 4 and 8-point transforms are embedded in the 16-point transform. But the 32-point transform does not embed the proposed 4, 8 and 16-point transforms.

# Results

Results for AI-HE, RA-HE, LB-HE, AI-LC, RA-LC, and LB-HC configurations were crosschecked for low and high QP ranges. Table 1 shows a summary of the BD-rate results for low QPs. Table 2 shows a summary of the BD-rate results for high QPs. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster (64 bit) by specifying only one type of CPU.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 0.3% | 0.3% | -0.7% | -0.3% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% | -0.2% | -0.1% | -0.1% |
| Class C | 0.0% | 0.0% | 0.0% | -0.4% | -0.2% | -0.2% |
| Class D | 0.0% | 0.0% | 0.0% | -0.3% | -0.1% | -0.1% |
| Class E | 0.0% | 0.0% | 0.0% | -0.2% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | -0.4% | -0.1% | -0.1% |
|  | 0.0% | 0.1% | 0.1% | -0.4% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 101% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.3% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | -0.1% | 0.0% | 0.1% |
| Class E |  |  |  |  |  |  |
| Overall | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 102% |  |  | #NUM! |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 105% |  |  |
| Dec Time[%] | 103% |  |  | 102% |  |  |

Table 1: BD-rate results for lowQP range

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.3% | 0.0% | -0.3% | 0.0% |
| Class B | 0.0% | -0.2% | -0.4% | 0.0% | -0.1% | 0.1% |
| Class C | -0.1% | -0.3% | -0.1% | 0.0% | 0.0% | 0.1% |
| Class D | 0.0% | 1.0% | 0.1% | 0.0% | -0.4% | -0.4% |
| Class E | 0.0% | -0.3% | 0.5% | -0.1% | 0.3% | -0.1% |
| Overall | 0.0% | 0.0% | -0.1% | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -1.4% | 0.3% | 0.0% | 0.0% | -0.1% |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | 0.8% | -1.0% | 0.0% | 0.7% | -0.1% |
| Class B | 0.1% | -1.2% | 0.4% | -0.2% | -0.5% | -0.1% |
| Class C | -0.2% | -1.8% | 1.0% | 0.0% | -2.1% | -0.1% |
| Class D | -0.1% | 2.9% | 0.0% | -0.3% | 0.6% | -0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 0.0% | 0.1% | 0.1% | -0.1% | -0.3% | -0.2% |
|  | 0.0% | 0.2% | 0.1% | -0.1% | -1.0% | -0.4% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.1% | 1.2% | 0.4% | 0.2% | 0.2% | 0.0% |
| Class C | -0.1% | -0.2% | 0.5% | 0.2% | -1.5% | -0.3% |
| Class D | 0.2% | -12.1% | -1.4% | -0.2% | 1.2% | 1.5% |
| Class E | -0.2% | 1.6% | -3.5% | 0.2% | 3.0% | 1.6% |
| Overall | 0.0% | -2.4% | -0.7% | 0.1% | 0.5% | 0.6% |
|  | 0.0% | 0.7% | -1.2% | 0.0% | 0.5% | 0.6% |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Table 2: BD-rate results for the high QP range

# Conclusions

We have verified results reported in JCTVC-G266 for high and low QPs. JCTVC proposes 4, 8 and 16-point core transforms. For 32-point transform, the 32-point transform proposed in JCTVC-G737 is reused. The proposed 4, 8 and 16-point transforms reportedly provide the capability for lossless coding.

The average luma BD-rate difference averaged over AI-HE, RA-HE, LB-HE, AI-LC, RA-LC, and LB-HC configurations was -0.05% for low QP range and 0.0% for high QP range with respect to HM3.0.