| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G864 WG11 Number: m22441 |
| --- | --- |

| Title: | CE8 Subset b.1: Cross check of Cisco's ALF with low latency and reduced complexity (JCTVC-G498) by Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | I. S. Chong and M. Karczewicz 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: 1-858-845-5477 Email: ichong@qualcomm.com |  |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports verification results of JCTVC-G498 for CE8 Subset b.1.  Coding efficiency results are confirmed.  No mismatch between encoder and decoder is observed.  Relative encoding and decoding time have some degree of deviation.  Considering the difference of operating the environments, results are confirmed.

# Introduction

This contribution is a report of verification results of JCTVC-G498, which is a contribution to CE8 Subset b.1. Verification is performed under the CE8 test conditions.  In addition, source code is reviewed.

# Experimental Results

Our encoding environment is using 64bit linux cluster and decoding was done on sinlge 64 bit windows PC. There was no mismatch between encoder and decoder. There is no mismatch of overall BD-rate between the verification results and the proponent’s results.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.7% | 1.9% | 1.4% | -2.3% | 0.4% | 0.4% |
| Class B | 0.4% | 2.7% | 2.2% | -1.4% | 0.5% | 0.5% |
| Class C | 0.5% | 3.8% | 4.0% | -0.9% | 0.6% | 0.6% |
| Class D | 0.2% | 3.1% | 3.5% | -0.4% | 0.3% | 0.3% |
| Class E | 0.7% | 4.5% | 6.1% | -1.9% | 0.9% | 0.9% |
| Overall | 0.5% | 3.1% | 3.2% | -1.4% | 0.5% | 0.5% |
|  | 0.5% | 3.1% | 3.2% | -1.3% | 0.5% | 0.5% |
| Enc Time[%] | 77% |  |  | 108% |  |  |
| Dec Time[%] | 91% |  |  | 110% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 1.5% | 7.5% | 6.5% | -3.6% | 1.4% | 0.7% |
| Class B | 1.0% | 4.1% | 2.6% | -2.4% | 0.5% | 0.5% |
| Class C | 0.9% | 2.6% | 3.0% | -1.5% | 0.9% | 0.9% |
| Class D | 0.9% | 2.2% | 2.5% | -1.6% | 0.5% | 0.4% |
| Class E |  |  |  |  |  |  |
| Overall | 1.1% | 4.1% | 3.6% | -2.3% | 0.8% | 0.6% |
|  | 1.1% | 4.0% | 3.5% | -2.3% | 0.8% | 0.6% |
| Enc Time[%] | 95% |  |  | 101% |  |  |
| Dec Time[%] | 90% |  |  | 109% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.1% | 3.6% | 2.5% | -2.2% | 0.6% | 0.7% |
| Class C | 1.0% | 3.2% | 3.5% | -2.0% | 1.0% | 0.8% |
| Class D | 0.7% | 3.5% | 3.7% | -1.1% | 0.9% | 1.0% |
| Class E | 1.3% | 3.1% | 3.4% | -3.1% | 1.3% | 0.9% |
| Overall | 1.0% | 3.4% | 3.2% | -2.0% | 0.9% | 0.8% |
|  | 1.0% | 3.4% | 3.2% | -2.0% | 0.9% | 0.8% |
| Enc Time[%] | 97% |  |  | 102% |  |  |
| Dec Time[%] | 93% |  |  | 108% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.2% | 3.9% | 2.6% | -5.1% | 0.2% | -0.3% |
| Class C | 0.4% | 2.4% | 2.7% | -2.5% | 0.6% | 0.7% |
| Class D | 0.1% | 2.1% | 2.1% | -0.9% | 0.3% | 0.2% |
| Class E | 1.3% | 4.3% | 3.8% | -7.0% | -0.2% | 0.4% |
| Overall | 0.7% | 3.2% | 2.7% | -3.7% | 0.3% | 0.2% |
|  | 0.7% | 3.2% | 2.7% | -3.7% | 0.3% | 0.2% |
| Enc Time[%] | 94% |  |  | 107% |  |  |
| Dec Time[%] | 90% |  |  | 108% |  |  |

Table 1. Cisco’s proposal on CE8 Tool b.1

# Source code review

Cisco’s code follows the description of CE8 Tool b.1. This proposal tries to reduce the complexity of ALF by using 64x64 or 128x128 ALF units.