| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G868 WG11 Number: m22445 |
| --- | --- |

| Title: | CE6: Test results of DCIM |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ehsan Maani, Ali Tabatabai 1730 N 1st Street San Jose, CA 95112 Tomoyuki Yamamoto, 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +1-408-352-4099 ehsan.maani@am.sony.com ali.tabatabai@am.sony.com +81-43-299-8526 yamamoto.tomoyuki@sharp.co.jp |
| Source: | Sony Electronics Inc. and Sharp Corp. |  |  |

_____________________________

# Abstract

This document presents the description and results of Differential Coding of Intra Modes (DCIM) [1]. On average, using this technique, 0.2% and 0.3% gain is achieved compared to HM 4.0 anchors for High Efficiency (HE) and Low Complexity (LC) settings, respectively.

# Algorithm description

DCIM uses neighborhood edge estimation to predict the Intra prediction direction and differentially encodes the selected direction with respect to the predicted direction (i.e., given by the edge). This enables a higher accuracy in the Intra prediction directions (modes) without substantially increasing the mode signaling overhead. An additional flag bit is transmitted per Prediction Unit (PU) to signal to the decoder whether DCIM is used. The decoder needs to perform edge detection for the PUs for which the DCIM flag is on. Details of various parts of the proposed technique are described below. The DCIM technique is presented in details in [2]. This implementation of DCIM in HM4.0 however, does not include bi-prediction.

# Results

The proposed algorithm was implemented into HM-4.0 reference software. Based on the description in CE6.c, the following results were obtained.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.09% | -0.71% | -0.17% | -0.07% | 0.07% | 0.25% |
| Class B | -0.16% | -0.91% | -0.72% | -0.20% | 0.23% | 0.17% |
| Class C | -0.58% | -0.70% | -1.02% | -0.80% | 0.01% | -0.38% |
| Class D | -0.18% | -0.29% | -0.26% | -0.25% | 0.03% | -0.02% |
| Class E | -0.25% | -1.20% | -1.49% | -0.44% | -0.57% | 0.05% |
| Overall | -0.21% | -0.75% | -0.69% | -0.34% | -0.01% | 0.02% |
|  | -0.21% | -0.72% | -0.68% | -0.34% | 0.00% | 0.03% |
| Enc Time[%] | 116% |  |  | 120% |  |  |
| Dec Time[%] | 102% |  |  | 100% |  |  |

# Patent rights declaration(s)

Sony Electronics and SHARP may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- E. Maani, W. Liu, “Differential Coding of Intra Modes (DCIM)”, Doc. JCTVC-B109, Geneva, Switzerland, Jul 2010

- E. Maani, T. Yamamato, A. Tanizawa, T. Shiodera , V. Drugeon, “CE6.c: Differential Coding of Intra Modes (DCIM)”, Doc. JCTVC-F566, Torino, Italy, Jul 2011.