| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G873 |
| --- | --- |

| Title: | Cross-check report of JCTVC-G704 on last position coding for CABAC from Qualcomm |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 2 99 87 68 83 edouard.francois@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

The purpose of this document is to report cross-checking of proposal JCTVC-G704 on last position coding for CABAC from Qualcomm. The verification task has been done successfully and the results match those presented in JCTVC-G704. The software implementation has also been checked and the consistency with the proposal description has been verified. The proposal results in a simplification of the HEVC design, by using less bins for last position coding and by reducing the number of bins coded with context. A slight BDR gain is in addition observed.

- Code verification

The source code has been analyzed. It has been verified that this implementation was in line with the proposal JCTVC-G704 description.

The proposal consists in using reduced number of bins and shorter truncated unary codes to code the last significant coefficient position. In HM4.0, coding of last position value V is based on the maximum possible value W. If V<W/2, truncated unary code is used. Otherwise, the codeword is a concatenation of a truncated unary coded and of a FLC code. FLC code bins are bypass coded.

In JCTVC-G704, the part of the last position value coded by FLC is increased, while the length of truncated unary codes is decreased. More position values are then using concatenation of truncated unary and FL codes. This proposal results in more bins coded by bypass coding than in HM4.0.

The software implementation has been verified and is confirmed to be aligned with the proposal description. The corresponding code is clean.

The source code was compiled successfully using windows32, VC2008 and using Linux 2.6.18-194.el5,  gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

# Results

Tests were done according to the tests description provided in [1]. The xls files are attached. The results perfectly match those generated by Qualcomm.

# Conclusion

The results presented by Qualcomm and MediaTek in JCTVC-G203 are confirmed.

The proposal results in a simplification of the HEVC design, by using less bins for last position coding and by reducing the number of bins coded with context. A slight BDR gain is in addition observed.

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-F900, 6th Meeting: Torino, Italy, July, 2011.