| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G884 |
| --- | --- |

| Title: | Verification of JCTVC-G131 on interpolation filter order |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Frank Bossen 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +1 650 496 4742 bossen@docomoinnovations.com |
| Source: | DOCOMO Innovations, Inc. |  |  |

_____________________________

# Abstract

At the 6th JCT-VC meeting the motion compensation process was modified based on JCTVC-F537 such that intermediate data buffers require only 16 bits per sample, regardless of the bit depth of the video being processed. One side effect is that the order in which horizontal and vertical interpolation is applied becomes normative for input bit depths > 8. In JCTVC-G131 it is proposed to condition the order of horizontal and vertical interpolation on the shape of a PU to reduce the number of multiply-accumulate (MAC) operations in the interpolation process. The proposed change has no substantial impact on R-D performance (0.0% luma averages) but seems to slow down the HM software a bit (up to 2%) where a speed-up may have been expected.

# Description

At the 6th JCT-VC meeting the motion compensation process was modified based on JCTVC-F537 such that intermediate data buffers require only 16 bits per sample, regardless of the bit depth of the video being processed. One side effect is that the order in which horizontal and vertical interpolation is applied becomes normative for input bit depths > 8. In JCTVC-G131 it is proposed to condition the order of horizontal and vertical interpolation on the shape of a PU to reduce the number of multiply-accumulate (MAC) operations in the interpolation process.

Similarly to the data presented in G131, the table below summarizes the number of operations required for “fat” PUs (width > height). The columns “H first” represent the case where horizontal interpolation is applied before vertical interpolation. The “H” and “V” sub-columns represent the number of samples interpolated by horizontal and vertical interpolation. The “total” sub-column represents the average number of multiply-accumulate (MAC) operations per interpolated sample (assuming an 8-tap filter and one MAC operation per tap).

|  | H first |  |  | V first |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PU size WxH | H | V | Total | V | H | Total | Diff |
| 8x4 | 8x11 | 8x4 | 30 | 15x4 | 8x4 | 23 | -23% |
| 16x4 | 16x11 | 16x4 | 30 | 23x4 | 16x4 | 19.5 | -35% |
| 16x8 | 16x15 | 16x8 | 23 | 23x8 | 16x8 | 19.5 | -15% |
| 32x8 | 32x15 | 32x8 | 23 | 39x8 | 32x8 | 17.75 | -23% |
| 32x16 | 32x23 | 32x16 | 19.5 | 39x16 | 32x16 | 17.75 | -9% |

Similar numbers are derived for the chroma case:

|  | H first |  |  | V first |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PU size WxH | H | V | Total | V | H | Total | Diff |
| 4x2 | 4x5 | 4x2 | 14 | 7x2 | 4x2 | 10 | -29% |
| 8x2 | 8x5 | 8x2 | 14 | 11x2 | 8x2 | 9.5 | -32% |
| 8x4 | 8x7 | 8x4 | 11 | 11x4 | 8x4 | 9.5 | -14% |
| 16x4 | 16x7 | 16x4 | 11 | 19x4 | 16x4 | 8.75 | -20% |
| 16x8 | 16x11 | 16x8 | 9.5 | 19x8 | 16x8 | 8.75 | -8% |

## Notes

- In the provided implementation the interpolation order is kept as horizontal first in the motion estimation stage, regardless of PU size.

# Results

The table below summarizes the results obtained with HM 4.0. Note that for the LC configurations, there is no change in R-D performance, as would have been expected.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.1% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class D | 0.1% | 0.3% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.3% | 0.5% | 0.0% | 0.0% | 0.0% |
| Class F | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.1% | 0.1% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.1% | 0.2% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.2% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class E | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class F | 0.0% | 0.3% | -0.3% | 0.0% | 0.0% | 0.0% |
| Overall | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.2% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

Average recorded run times are either at 100% or slightly above with respect to the HM4 anchor. There are several possible reasons for the increase in runtime:

- The size of the buffers TComPrediction::m_filteredBlockTmp are increased by 20 samples in each dimension. The reason for this increase is unclear as it seems unnecessary. Furthermore this increase modifies the byte alignment of each row in the array such that it is not aligned on 16-byte boundaries anymore (such alignment is useful for SIMD code, which the compiler is able to produce from the source code).

- The width of the array interpolated after vertical filtering (when done first) is not a multiple of 8 anymore (for luma). This may also have adverse effects on SIMD performance.

- Doing interpolation along the vertical direction may be slower as it requires more registers for addressing data. While the proposal reduces the number of filtering operations, it does increase the number along the vertical direction.

# Conclusion

The R-D performance of the G131 proposal was verified. While a theoretical analysis indicates a reduction in the number of operation to be performed, the reduction does not translate in a reduction of runtime of the HM software.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).