| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G886 |
| --- | --- |

| Title: | Cross-verification of ETRI's Inconsistency of intra LM mode between HM and WD (JCTVC-G419) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tammy Lee Jianle Chen Jeonghoon Park | Email: | tammy.lee@samsung.com jianle.chen@samsung.com jeonghoon@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports cross-checking results of ETRI's Inconsistency of intra LM mode between HM and WD in JCTVC-G419. The results confirm the BD-rates and encoding/decoding time from the proponents. The proponents’ code has also been analyzed and found to match to the context in the proposal.

# Test results

In this cross-checking report, solution2 in JCTVC-G449 is verified, where 1st left column is used instead of 2nd left column in LM mode. Software was compiled on 64-bit Linux and executed on Linux cluster.

Table 1 shows test results of solution2. The results are consistent to those from the proponent. More details can be found in the accompanying Excel sheets.

Table 1. Solution2 using 1st left column in LM mode

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1% | 2.2% | 1.6% | 0.1% | 2.2% | 1.7% |
| Class B | 0.1% | 0.4% | 0.2% | 0.1% | 0.3% | 0.1% |
| Class C | 0.2% | 0.9% | 1.0% | 0.2% | 0.5% | 0.8% |
| Class D | 0.1% | 0.3% | 0.3% | 0.1% | 0.3% | 0.3% |
| Class E | 0.0% | 0.1% | 0.2% | 0.0% | 0.1% | 0.1% |
| Overall | 0.1% | 0.8% | 0.7% | 0.1% | 0.7% | 0.6% |
|  | 0.1% | 0.8% | 0.6% | 0.1% | 0.7% | 0.6% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Solution1 of specification change in JCTVC-G449 had to be updated into WD according to adoption of JCTVC-F760, while HM4.0 was updated properly.

# References

[1]	J. Lee, S.-C. Lim, H. Y. Kim, J. S. Choi, “ETRI's Inconsistency of intra LM mode between HM and WD”, Document of Joint Collaborative Team on Video Coding, JCTVC-G419, November 2011.