| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G887 |
| --- | --- |

| Title: | CE 10: hardware test of inverse transform proposals |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sumit Johar STMicroelectronics Plot No. 1, Knowledge Park 3 Greater Noida, 201308, UP, India Daniele Alfonso STMicroelectronics Via Tolomeo 1 Cornaredo, 20010 MI, Italy | Tel: +91 120 4003909 Email: sumit.johar@st.com Tel: +39.02.93519755 Email: daniele.alfonso@st.com |  |
| Source: | STMicroelectronics |  |  |

_____________________________

# Abstract

This report presents the results of HW synthesis using an automatic RTL generation tool for the 32-point transform proposals submitted by Samsung, FastVDO and Qualcomm. The authors assert that F251 proposal gives better gate count for the same frequency and latency performance when compared to HM and F352 transforms.

- Introduction

To understand the impact of 32-point spatial transform proposals submitted in the last HEVC meeting on the hardware performance and area, it was decided to do actual HW synthesis. There are 2 main approaches to the HW synthesis, one is to do hand-written RTL and another is to use the automatic RTL generation tools. At STMicroelectronics we used Synphony C Compiler tool [1] for automatic RTL generation for 3 different 32-point transform proposals viz. HM, F251, and F352. The RTL generated was synthesized for a range of frequencies with Synopsys Design Compiler (DC) and the gate-count of the obtained net-list was reported.

- Description of the tool

Like other automatic RTL tool generators, Synopsys’s Synphony C Compiler takes C-code as input and generates RTL as output. There are essential design parameters that needs to be specified like frequency, technology library etc. which effect the output RTL’s performance and area. Apart from the input parameters the tool provides c-coding techniques like pragmas which can be used to achieve specific memory or system design. Depending upon the user’s requirements these parameters can be tuned. Using the test-vectors of the C-code the tool can generate test-vectors for hardware generation, which are used to verification with RTL simulations. The tool has its own design compiler which synthesizes the generated RTL to give final net-list but as a generally followed practice the output RTL is synthesized using Synopsys DC to get better area and performance. Synthesis using Synopsys DC also acts as a verification of the generated RTL.

This contribution is related to the activites of Core Experiment 10 on HEVC core transform design [2].

# Experimental Set-up

The c-code provided for the submitted proposals, performed 1-D inverse transform for 32 coefficients for the worst case scenario (maximum coefficient values possible) with the understanding that the 2-dimensional 32-point inverse transform will have the 64 (32 rows + 32 columns) times the performance of this system. To understand the HW implementation complexity and performance of the given transform proposals, it was agreed upon by all the members of CE10 to synthesize the proposals at different latencies for a certain set of frequencies with a throughput of 1. The latency means maximum cycles between the first input and first output of the system and throughput of 1 imply there is an output every cycle. For a pipelined-architecture with a throughput of 1, the latency implies the number of pipeline stages. The values of latencies agreed upon are 2,3,4,5,6,7 whereas the agreed range for frequencies is 150 MHz – 500 MHz, taking into account the generally acceptable implementations.

At STMicroelectronics we used an automatic tool generator Synphony CC (referred as SCC in the rest of the document) for synthesis and used the c-code provided by participant members as input.

- To do a fair comparison there was no optimization method implemented for any of the proposals, except for the appropriate bit-sizing of the variables and inputs used in the c-code.

- All the c-codes had all inlined code i.e. no function calls, in order to enable SCC to interpret data dependencies and hence exploit maximum hardware sharing.

Based on the C-code and input parameters SCC gives RTL as output. As input parameters we specify the frequency and throughput. Frequency is a very important input parameter to SCC as this dictates the available cycle width and hence responsible for the number of pipeline stages in the design. Throughput is also an important parameter as this is the performance requirement of the design. Based on the code and the given input parameters, SCC creates an RTL with a given latency. Hence, latency is the output of the synthesis process and whereas frequency and throughput are inputs, so we have direct control over the frequency values but latency is decided by SCC and that is why it was not possible to achieve all latency values for all the proposals.

Different implementations were created in SCC for different frequencies and all with a target throughput 1. The RTL generated for above implementations had certain values of latencies. The RTL generated for the desired latency values was chosen and a net-list is synthesized using SynopsysDC for the given range of frequencies. CMOS 45nm library was used for the purpose of synthesis. The gate count for the synthesized net-list for the given proposals at different latencies and frequencies have been reported in the tables below.

Note: Figures reported in Red indicate negative slack, as reported by SynopsysDC.

| HM |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency(MHz) /Latency | L=2 | L=3 | L=4 | L=5 | L=6 | L= 7 |
| 150 | 108364.00 | 134553.00 | 147701.00 |  |  | 182063.33 |
| 200 | 110269.00 | 134615.00 | 147711.00 |  |  | 182016.00 |
| 250 | 123155.00 | 143120.00 | 151092.00 |  |  | 182387.33 |
| 300 | 126322.00 | 143793.00 | 159014.00 |  |  | 204536.33 |
| 350 | 129150.00 | 153378.00 | 168162.00 |  |  | 204752.33 |
| 400 |  | 165196.00 | 178534.00 |  |  | 216074.66 |
| 450 |  |  |  |  |  |  |
| 500 |  |  |  |  |  |  |

| F251 Samsung+FastVDO |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency(MHz) /Latency | L=2 | L=3 | L=4 | L=5 | L=6 | L=7 |
| 150 | 49647.00 |  | 57656.67 | 67446.33 | 77805.00 | 81256.00 |
| 200 | 57869.66 |  | 70142.00 | 68928.67 | 78830.33 | 82065.00 |
| 250 | 71706.66 |  | 74467.00 | 83119.67 | 84958.33 | 92561.00 |
| 300 | 77493.33 |  | 81463.00 | 84758.33 | 96415.67 | 98555.67 |
| 350 | 86402.00 |  | 87763.00 | 89052.67 | 100879.33 | 100296.67 |
| 400 | 103183.33 |  | 99331.33 | 90469.67 | 101509.33 | 103581.33 |
| 450 |  |  |  |  |  |  |
| 500 |  |  |  |  |  |  |

| F352 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frequency(MHz) /Latency | L=2 | L=3 | L=4 | L=5 | L=6 | L=7 | L=8 | L=9 |
| 150 | 116031.33 | 94362.00 | 78452.00 | 75705.00 | 75390.00 | 79697.33 | 82231.33 | 86422.00 |
| 200 | 167160.00 | 129802.33 | 93772.67 | 95208.67 | 97408.33 | 95931.00 | 94642.33 | 96029.00 |
| 250 |  | 174509.00 | 118496.33 | 105109.00 | 101987.00 | 106014.33 | 103304.67 | 105961.33 |
| 300 |  |  | 137616.00 | 120813.33 | 111168.00 | 108840.67 | 109733.67 | 108550.33 |
| 350 |  |  |  | 147816.67 | 135385.67 | 123382.67 | 115746.00 | 119313.33 |
| 400 |  |  |  |  | 154668.33 | 150333.00 | 129616.67 | 127963.33 |
| 450 |  |  |  |  |  |  | 152017.00 | 146060.67 |
| 500 |  |  |  |  |  |  |  |  |

# Observations

The F251 proposal was consistently found to deliver better gate count for the same frequency/latency performance when compared to HM proposal for all the latencies and frequency values. The maximum gain was up to 61% and minimum gain 33% with an average gain of 47%. Whereas F352 was found to take larger area at lower latencies but performed better at higher latencies when compared with HM. At latency of 7, the maximum gain is reported at 46%. The reason for F352 to take higher area at lower latencies can be attributed to the extensive usage of shifters, because at lower latency for a given throughput sharing of hardware is compromised.

# Conclusions

The authors assert that F251 proposal gives better area for the same frequency and latency performance when compared to HM and F352, as demonstrated by the reported simulation results, obtained without doing any hardware optimizations using SynphonyCC automatic RTL generator tool.

# References

- http://www.synopsys.com/Systems/BlockDesign/HLS/Pages/SynphonyC-Compiler.aspx

- P. Topiwala, M. Budagavi, R. Joshi, E. Alshina, “CE10: Summary Report of Core Experiment on Core Spatial Transforms”, JCTVC-G040, Geneva, November 2011.

- E. Alshina, A. Alshin, I.-K. Kim, P. Topiwala, “CE10: Full-factorized core transform test by Samsung/FastVDO”, JCTVC-F251, Torino , July 2011.

- R. Joshi, Y. Reznik, J. Sole, M. Karczewicz, “CE10: Scaled orthogonal integer transforms supporting recursive factorization structure”, JCTVC-F352, Torino, July 2011.

# Patent rights declaration

STMicroelectronics does not have any current or pending patent rights relating to the technology described in this contribution