| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G889 |
| --- | --- |

| Title: | Cross-check of JCTVC-G271 on Sign Data Hiding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Marta Mrak, Andrea Gabriellini BBC R&D, Centre House, 56 Wood Lane W12 7SB London, UK |  | marta.mrak@bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution reports the verification results of JCTVC-G271 for Orange Labs’ proposal on Sign Data Hiding. Coding efficiency results are confirmed for class A to E. Class F results are slightly different and it is thought that a known bug, ticket n. 203 (http://hevc.kw.bbc.co.uk/trac/ticket/203), may be responsible for it. Running times are very close to the proponents’ and whitin the range of variability showed by the cluster used for the simulations. JCTVC-G271 results are thus confirmed, with the qualifications expressed above. The proposal presents a technique that allows to omit the sending of explicit information about the sign of quantised coefficients.

# Introduction

This contribution is a report of verification results of JCTVC-G271, which is a proposal for Sign Data Hiding, a technique enabling to omit the sending of explicit information about the sign of quantised coefficients. Four test points from common test conditions (JCTVC-F900) are used, all high efficiency: intra, random access, low delay and low delay P. Additionally, the following parameters were provided in input to the encoder:

--IFrameTSIGIntra=5

--TSIGIntra=5

--TSIGInter=3

--IFrameTSIGIntraChroma=5

--TSIGIntraChroma=5

--TSIGInterChroma=5

These parameters represent the thresholds defined in JCTVC-G271.

The source code provided by Orange Labs has also been reviewed.

# Experimental results

The simulation was run on a cluster of 64-bit linux machines. There was no mismatch between coded and decoded sequences. The simulation yielded the same BD-rate results as the proponent’s for sequences in classes A to E. Results for sequences in class F were slightly different and it is believed that a known bug in HM 4.0, ticket n. 203 (http://hevc.kw.bbc.co.uk/trac/ticket/203), may be responsible for it. The running times are very close to the proponents’ and within the range of variability showed by the cluster used for the simulation.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.5% | -0.8% | -0.7% |
| Class B | -0.6% | -0.8% | -0.7% |
| Class C | -0.7% | -0.9% | -0.9% |
| Class D | -0.7% | -1.1% | -1.0% |
| Class E | -0.4% | -1.0% | -0.9% |
| Class F | -0.8% | -1.0% | -0.9% |
| Overall | -0.6% | -0.9% | -0.9% |
|  | -0.6% | -0.9% | -0.9% |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.5% | -0.5% | -0.7% |
| Class B | -0.6% | -0.3% | -0.4% |
| Class C | -0.6% | -0.5% | -0.5% |
| Class D | -0.6% | -0.6% | -0.6% |
| Class E |  |  |  |
| Class F | -0.7% | -0.4% | -0.6% |
| Overall | -0.6% | -0.5% | -0.5% |
|  | -0.6% | -0.5% | -0.5% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Low delay B HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.8% | -0.4% | -0.8% |
| Class C | -0.8% | -0.6% | -0.9% |
| Class D | -0.8% | -0.2% | -0.8% |
| Class E | -0.2% | -0.5% | -0.9% |
| Class F | -0.9% | -0.8% | -0.7% |
| Overall | -0.7% | -0.5% | -0.8% |
|  | -0.7% | -0.5% | -0.8% |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 101% |  |  |

|  | Low delay P HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.7% | -0.4% | -0.7% |
| Class C | -0.8% | -0.4% | -0.7% |
| Class D | -0.7% | -0.3% | -0.4% |
| Class E | -0.2% | -0.7% | -1.4% |
| Class F | -0.8% | -0.6% | -0.9% |
| Overall | -0.7% | -0.5% | -0.8% |
|  | -0.7% | -0.4% | -0.8% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 100% |  |  |

Table 1 - Results For Orange Lab's Sign Data Hiding Proposal

# Source code review

The analysis of the code confirms the description of the algorithm provided in JCTVC-G271.