| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G890 |
| --- | --- |

| Title: | CE6.c: Cross-check of JCTVC-G322 on Harmonization of HE residual coding with non-square block transforms |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Marta Mrak, Andrea Gabriellini BBC R&D, Centre House, 56 Wood Lane W12 7SB London, UK |  | marta.mrak@bbc.co.uk andrea.gabriellini@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This contribution reports the verification results of JCTVC-G322 on Harmonization of HE residual coding with non-square block transforms. The proposed solution addresses coding of non-square blocks in HM-4.0, and in this report its handling of motion compensated blocks is analyzed. Coding efficiency results are confirmed in all test points. Running times are very close to the anchors (HM-4.0) and the analysis of the code shows that the changes from this proposal should not affect the run times. The proposed method simplifies handling of non-square blocks in HM-4.0 and harmonizes usage of diagonal scans on square and non-square blocks.

# Solution analysis

In HM-4.0, CABAC coefficients coding is not directly handling non-square transform blocks. With the introduction of NSQT, the existing implementation needed to be amended in order to provide support for non-square blocks. In HM-4.0 this is done by remapping non-square blocks into square blocks. Such solution is not desirable because it adds a number of unnecessary coding steps, as well as a number of problems in other functions that are dependent on the order of the coefficients.

JCTVC-G322 proposes a solution that removes the remapping requirement, thus making the code simpler. Moreover, it harmonizes diagonal scan usage between square and non-square blocks. Proposed changes are implemented in all points of the codec, including the search functions and RDOQ.

The verification test was performed on a cluster of 64-bit linux machines. The results are summarized in Table 1. The running times are very close to the proponents’ and within the range of variability showed by the cluster used for the simulation.

Table 1: Cross-check results for G322

|  | Random Access HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0% | -0.2% | -0.3% |
| Class B | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | -0.1% | 0.0% |
| Class D | 0.0% | 0.0% | 0.1% |
| Class E |  |  |  |
| Class F | 0.0% | -0.1% | -0.2% |
| Overall | 0.0% | -0.1% | -0.1% |
|  | 0.0% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.1% | -0.1% | -0.2% |
| Class C | 0.0% | -0.1% | -0.1% |
| Class D | 0.0% | 0.1% | -0.2% |
| Class E | -0.1% | 0.4% | 0.5% |
| Class F | -0.3% | -0.5% | -0.1% |
| Overall | -0.1% | -0.1% | 0.0% |
|  | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% |
| Class C | -0.1% | 0.0% | -0.2% |
| Class D | 0.0% | 0.2% | -0.1% |
| Class E | 0.0% | -0.2% | 0.1% |
| Class F | -0.2% | 0.3% | -0.1% |
| Overall | -0.1% | 0.1% | -0.1% |
|  | -0.1% | 0.1% | -0.1% |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |