| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 19-30 November, 2011 | Document: JCTVC-G891 M22469 |
| --- | --- |

| Title: | Non-CE9: cross-verification of Simplification on AMVP/Merge (JCTVC-G542 from Panasonic) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: : | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc; |  |  |

_____________________________

# Abstract

This contribution reports cross-check results for simplification 2 and 3 of Panasonic’s proposal JCTVC-G542 entitled simplification on AMVP/Merge. Simplification 2 is about reducing the complexity for the scale sharing of the AMVP scaled spatial MVP derivation process, and simplification 3 is about reducing the second pruning process of the merge/skip list construction by appending zero MVPs directly to the list and skipping the pruning process. No loss was observed in Simplification 2, small gain (i.e. 0.1%) was found in LB configurations and no loss was observed in RA configurations of Simplification 3.  The BD-rate results match the ones reported by proponents. The source code has been carefully checked to verify its consistency with the proposal description.

.

# Introduction

This contribution reports cross-check results for simplifications 2 and 3 of Panasonic’s proposal JCTVC-G542 entitled simplification on AMVP/Merge.

- In simplification 2 the spatial MVP scaling is marked as “used” as long as there is at least one neighboring PU on the left side which is available and not coded with intra-mode. In HM4.0 the spatial MVP scaling is  marked as “used” if both the conditions are true: 1) there is at least one neighboring PU on the left side which is available and not coded with intra-mode, and 2) MVPs from the same reference frame of different list are also unavailable.   This simplification can lead to that the spatial MVP scaling does not take place on the upper side even if the scaling actually is not used on the left side in certain cases. But the proposed design is definitely simpler and can reduce (MUX) complexity for the spatial MVP scaling sharing from upper and left side during the AMVP list derivation process.

- In simplification 3 the zero MVPs are not pruned. Instead, they are directly appended to the merge/skip MVP list. This reduces the complexity of the second pruning process of the merge/skip list construction because the incoming number of MVP candidates for pruning is 2 less.

# Test Settings and Conditions

The simulations of this document have used HM4.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental results

The experimental results are summarized in Table 1and Test 2.  No loss was observed in Simplification 2, small gain (i.e. 0.1%) was found in LB configurations and no loss was observed in RA configurations of Simplification 3. The results match the ones reported by proponents. Please be advised runtime here may not be accurate.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.1% | -0.2% | 0.0% | 0.1% | 0.2% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.1% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | 0.0% | -0.1% | 0.0% | 0.0% | -0.1% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 99% |  |  | 103% |  |  |
| Dec Time[%] | 95% |  |  | 111% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.2% |
| Class C | 0.0% | -0.1% | -0.1% | 0.0% | 0.2% | -0.1% |
| Class D | 0.0% | 0.3% | 0.1% | 0.0% | 0.2% | -0.3% |
| Class E | 0.0% | -0.1% | 0.8% | -0.1% | 0.2% | 0.7% |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | 0.0% | 0.2% | 0.0% | 0.2% | 0.1% |
|  | 0.0% | 0.0% | 0.1% | 0.0% | 0.1% | 0.1% |
| Enc Time[%] | 104% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |

Table 1. Experimental results for Simplification 2

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0% | -0.2% | -0.2% | 0.0% | 0.0% | 0.0% |
| Class B | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class C | 0.0% | 0.0% | 0.1% | 0.0% | 0.0% | 0.0% |
| Class D | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Class E |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Overall | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
|  | 0.0% | -0.1% | 0.0% | 0.0% | 0.0% | 0.0% |
| Enc Time[%] | 96% |  |  | 104% |  |  |
| Dec Time[%] | 93% |  |  | 106% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1% | -0.4% | -0.3% | -0.1% | -0.1% | 0.1% |
| Class C | -0.1% | -0.1% | 0.0% | -0.1% | 0.2% | -0.1% |
| Class D | 0.0% | 0.6% | -0.1% | 0.0% | 0.2% | 0.1% |
| Class E | -0.3% | -0.3% | -0.5% | -0.4% | -0.3% | -0.4% |
| Class F |  |  |  |  |  |  |
| Overall | -0.1% | -0.1% | -0.2% | -0.1% | 0.0% | 0.0% |
|  | -0.1% | -0.1% | -0.2% | -0.1% | -0.1% | -0.1% |
| Enc Time[%] | 104% |  |  | 100% |  |  |
| Dec Time[%] | 102% |  |  | 98% |  |  |

Table 2. Experimental results for Simplification 3

# Comments

# The proposed simplifications 2 and 3 of JCTVC-G542 are helpful for reducing the complexity of merge/skip and AMVP list derivation process. It is recommended to consider this proposal together with other contributions in this category.

# References

[1]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-F900, Torino, IT, July 2011.

[2]	B. Bross, W.-J. Han,  J.-R. Ohm, G. J. Sullivan, T. Wiegand  “WD4: Working Draft 4 of High-Efficiency Video Coding,” JCT-VC Document, JCTVC-F803, Torino, IT, July 2011

[4] B. Bross, W. Chien, I. Kim, M. Zhou, “Core Experiment 9: MV Coding and Skip/Merge Operations”, JCT-VC Document, JCTVC-F909, Torino, IT, July 2011.