| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G893 |
| --- | --- |

| Title: | A Unified LCU, sub-LCU and sub-CU (TU) Delta QP Signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xue Fang Ajay Luthra Krit Panusopone Jae Hoon Kim Limin Wang | Tel: Email: | +1-858-404-2723 X.Fang@motorola.com |
| Source: | Motorola Mobility |  |  |

_____________________________

# Abstract

This document describes a Motorola Mobility’s proposal of combination of delta quantization parameter (dQP) signaling at LCU level, sub-LCU level [1] and sub-CU level [2]. The unified dQP signaling can be achieved with one new added control parameter. The new parameter, tu_delta_qp_depth, allows a leaf TU to send its own dQP when its transform depth is below the tu_delta_qp_depth threshold. The existing parameter, max_cu_qp_delta_depth, controls if a leaf CU (has at least one nonzero coefficient) can signal its own dQP or share a dQP with others [3]. The max_cu_qp_delta_depth, is extended to control TU dQP signaling also.

# Introduction

Sub-LCU level adaptive Quantization Parameter (QP) control allows delta quantization parameter dQP signaling at CUs smaller than the LCU size for finer granularity and visual quality control. The max_cu_qp_delta_depth parameter is defined as the maximum CU depth that can signal dQP. When a CU depth is less than the max_cu_qp_delta_depth, a dQP is signaled before coefficient at that CU if there is at least one nonzero coefficient [4].  If depths of the leaf CUs are larger than or equal to max_cu_qp_delta_depth, only single dQP will be signaled at the first leaf CU with nonzero coefficient. The rest of those leaf CUs will share the same QP.

In order to improve finer granularity and visual quality control further, sub-CU adaptive Quantization Parameter (QP) signaling is introduced [2]. The same max_cu_qp_delta_depth parameter is used for both sub-LCU and sub-CU signaling. When depths of leaf CUs are larger than or equal to max_cu_qp_delta_depth, only single dQP will be signaled the same way as sub-LCU control. If depth of a leaf CU is below the max_cu_qp_delta_depth, a TU can signal its own dQP if the full depth (CU depth + transform depth) of that TU is less than the given threshold. Same as sub-LCU, leaf TUs will share a single TU dQP when they are over the max_cu_qp_delta_depth. Table 1 shows the dQP signaling with various settings.

Table 1: Sub-CU based dQP Signaling With Various Configurations

| max_cu_qp_delta_depth | CU | TU |
| --- | --- | --- |
| 0 | LCU dQP only |  |
| A | if ( depth >= A ) shared sub-LCU dQP | if ((cudepth + trdepth) < A) Own sub-CU dQP if ((cudepth + trdepth) >= A ) Shared sub-CU dQP |

Due the shared max_cu_qp_delta_depth for both TU and CU dQP signaling control, the overhead of the sub-CU (TU) adaptive Quantization Parameter (QP) control is bound to this CU based threshold. For example, when sub-CU based dQP control is enabled along with max_cu_qp_delta_depth set to 3, the maximum number of dQP signaling in a 64x64 CU can be as many as 64 in worst case when all the leaf TUs are signaled.

Another problem is lack of flexibility if want to achieve both sub-CU and sub-LCU due to the syntaxes limitation.

- Proposed Implementation

A solution for the possible issues above is proposed by introducing the second depth control parameter tu_delta_qp_depth for sub-CU (TU) dQP signaling. This tu_delta_qp_depth is a relative depth starting from the root of CU.  The overhead of the sub-CU (TU) adaptive Quantization Parameter (QP) can be controlled by setting the tu_delta_qp_depth independent of max_cu_qp_delta_depth.  Sub-LCU or sub-CU adaptive Quantization Parameter (QP) can also be achieved by setting max_cu_qp_delta_depth and tu_delta_qp_depth the same or in different.

Following are the possible rules with the values of max_cu_tu_qp_delta_depth and tu_delta_qp_depth:

- If max_cu_qp_delta_depth set to 0, tu_delta_qp_depth will not necessary be signaled and will be inferred to 0. If  max_cu_qp_delta_depth is set to 0, only one dQP will be signaled under entire LCU;

- If max_cu_qp_delta_depth is a no-zero value, the tu_delta_qp_depth will be signaled.  The max_cu_qp_delta_depth must be less than the total depth of CU quadtree.  The tu_delta_qp_depth  is bounded by the minimum value of max_cu_qp_delta_depth and the total depth of TU quadtree;

- When max_cu_qp_delta_depth is a no-zero value,  set tu_delta_qp_depth to 0 will enable sub-LCU-level adaptive Quantization Parameter (QP) control;

With the combination of setting these two parameters, dQP signaling can be achieved at various levels in Table 2.

Table 2: dQP Signaling Level With Various Configurations

| max_cu_qp_delta_depth | tu_delta_qp_depth | CU | TU |
| --- | --- | --- | --- |
| 0 | x | LCU dQP only |  |
| A (A>0) | 0 | sub-LCU dQPs |  |
| A | B (B<=A) | if ( depth >= A ) shared sub-LCU dQP | if ((cudepth + trdepth) < A && trdepth <B) Own sub-CU dQP if ((cudepth + trdepth) >= A ¦¦ trdepth >=B) Shared sub-CU dQP |

# Simulation Results

Since the proposal does not change algorithm of either sub-LCU or sub-CU.  The simulation results of both sub-LCU and sub-CU adaptive Quantization Parameter control can be found in JCTVC-G721 (anchor and sub-CU).

# Conclusions

In this report, a unified dQP signaling can be achieved with added sub-CU depth control parameter to provide more flexible method for both sub-LCU and sub-CU adaptive Quantization Parameter control.

# References

- Madhukar Budagavi, Minhua Zhou, “Delta QP signaling at sub-LCU level” JCTVC-D038

- K. Panusopone, A. Luthra, X. Fang, L. Wang, “QP adaptation at sub_CU level,” JCTVC-F577

- Tzu-Der Chuang , Ching-Yeh Chen , Yu-Lin Chang, Yu-Wen Huang, Shawmin Lei, “AhG Quantization: Sub-LCU Delta QP” JCTVC-E051

- L. Dong, W. Liu, K. Sato, “CU Adaptive Quantization Syntax Change for Better Decoder pipelining” JCTVC-D258

# Patent rights declaration(s)

Motorola Mobility may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).