| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G895-r2 |
| --- | --- |

| Title: | CE6: Cross-verification of Combination of SDIP and Its Improvements (JCTVC-G558) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Keiichi Chono Hirofumi Aoki 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +81 50 3757 5439 chono@ct.jp.nec.com |
| Source: | [Information and Media Processing Labs, NEC Corporation] |  |  |

_____________________________

# Abstract

This contribution presents cross-verification results of combination of SDIP and its improvements (JCTVC-G558). It was confirmed that the test results of SDIP with all improvements in section 4 of JCTVC-G558 are accurate.

- Simulation results

Software provided by JCTVC-G558 proponents was successfully complied on Windows7 64bit computer. The summaries of simulation results are shown in Table1 and Table2. It is confirmed that our results match with those provided by JCTVC-G558 proponents.

Table1. Summary of JCTVC-G558 results w/o JCTVC-G354

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.72% | -2.86% | -2.84% | -1.39% | -3.88% | -4.00% |
| Class B | -1.46% | -4.16% | -4.17% | -2.47% | -4.85% | -5.30% |
| Class C | -1.62% | -2.55% | -2.87% | -2.38% | -3.38% | -3.90% |
| Class D | -1.51% | -2.60% | -2.52% | -2.17% | -3.24% | -3.08% |
| Class E | -2.34% | -7.39% | -6.99% | -3.97% | -9.35% | -7.72% |
| Class F | -6.08% | -6.20% | -6.39% | -6.95% | -6.85% | -6.84% |
| Overall | -2.25% | -4.16% | -4.18% | -3.16% | -5.07% | -5.04% |
|  | -2.25% | -4.18% | -4.19% | -3.16% | -5.07% | -5.04% |
| Enc Time[%] | 127% |  |  | 133% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table2. Summary of JCTVC-G558 results w/ JCTVC-G354

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1.05% | -3.14% | -3.13% | -1.51% | -3.89% | -4.09% |
| Class B | -1.64% | -4.23% | -4.30% | -2.57% | -4.85% | -5.30% |
| Class C | -1.74% | -2.68% | -3.06% | -2.44% | -3.38% | -3.89% |
| Class D | -1.64% | -2.68% | -2.65% | -2.29% | -3.24% | -3.16% |
| Class E | -2.48% | -7.56% | -7.21% | -4.08% | -9.30% | -7.63% |
| Class F | -6.14% | -6.16% | -6.63% | -6.88% | -6.65% | -6.78% |
| Overall | -2.42% | -4.27% | -4.38% | -3.23% | -5.03% | -5.04% |
|  | -2.41% | -4.29% | -4.38% | -3.23% | -5.04% | -5.05% |
| Enc Time[%] | 128% |  |  | 134% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Reference

- X. Cao et al., "Report on Combination of SDIP and Its Improvements," JCTVC-G558, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011.