| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G896 |
| --- | --- |

| Title: | Cross verification of improved directional intra prediction smoothing (JCTVC-G443) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jani Lainema Kemal Ugur | Email: | jani.lainema@nokia.com kemal.ugur@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents Nokia’s cross verification of Mitsubishi’s proposal JCTVC-G443. Contribution verifies that the BD rate savings provided by the proposal are -0.02 % and -0.01 % for AI_HE and AI_LC configurations, respectively. It is further verified that the proposal has negligible impact on computational complexity.

# Analysis of the algorithm

The algorithm proposed in JCTVC-G443 is a modification to the “harmonized intra smoothing” proposal of JCTVC-G280. In JCTVC-G280 all directions with abs(intraPredAngle) > 9, excluding the case of intraPredAngle equal to 32, are treated identically and smoothing is performed in +/-45 degree angle. JCTVC-G443 proposes to modify smoothing process for the predicted samples associated with intraPredAngle equal to +/-13 and +/-17. In these cases the proposed approach would filter a border pixel in the PU with a reconstructed pixel two rows below / two columns to the right of the predicted pixel, resulting in a filtering process closer to the inverse of the selected prediction angle.

The benefit of the scheme is the ability to model the inverse prediction angle more accurately than done in JCTVC-G280 leading into -0.02 % and -0.01 % bitrate reductions for AI_HE and AI_LC configurations, respectively. The disadvantage is that it will create a need to support an additional process for obtaining the smoothed samples.

The simplified version of the proposed scheme disables the border smoothing for the negative angles. That modification seems to have negligible effect on the results in the context of this proposal (+0.02 % and +0.01 % effect for AI_HE and AI_LC configurations, respectively).

# Objective results

Table 1 below summarizes results for the modifications proposed in JCTVC-Gxxx and Table 2 the simplified method proposed in the same document. Considering the proposal is modifying the harmonized intra smoothing method of CE6.d test 13 (providing gains of -0.72 % for AI_HE and -0.70 % for AI_LC), the gains provided by the proposed method are -0.02 % for AI_HE and -0.01 % for AI_LC. Similarly, the simplified proposed approach provides gains of 0.00 % for both AI_HE and AI_LC configurations. The decoder was run once to measure the decoder runtimes the deviation from the anchor runtime is within normal variations.

Table 1:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.91% | -0.78% | -0.87% | -0.81% | -0.93% | -1.08% |
| Class B | -0.66% | -0.62% | -0.60% | -0.60% | -0.78% | -0.78% |
| Class C | -0.66% | -0.65% | -0.64% | -0.71% | -0.79% | -0.72% |
| Class D | -0.68% | -0.62% | -0.62% | -0.71% | -0.64% | -0.73% |
| Class E | -0.83% | -0.75% | -0.83% | -0.76% | -0.82% | -0.91% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.74% | -0.68% | -0.70% | -0.71% | -0.79% | -0.84% |
|  | -0.74% | -0.68% | -0.70% | -0.71% | -0.79% | -0.84% |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Table 2:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.92% | -0.78% | -0.81% | -0.82% | -0.96% | -1.06% |
| Class B | -0.65% | -0.59% | -0.60% | -0.59% | -0.77% | -0.79% |
| Class C | -0.64% | -0.61% | -0.64% | -0.68% | -0.75% | -0.70% |
| Class D | -0.64% | -0.61% | -0.63% | -0.68% | -0.69% | -0.71% |
| Class E | -0.82% | -0.69% | -0.83% | -0.75% | -0.90% | -0.89% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Overall | -0.72% | -0.65% | -0.69% | -0.70% | -0.80% | -0.82% |
|  | -0.73% | -0.66% | -0.70% | -0.70% | -0.81% | -0.82% |
| Enc Time[%] | 101% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).