| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G898 |
| --- | --- |

| Title: | Non-CE3: Cross check for Toshiba's proposal (JCTVC-G770) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Sony City Osaki, 2-10-1 Osaki Shinagawa-ku, Tokyo Japan | Tel: Email: | +81-50-3750-2744 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | SONY |  |  |

_____________________________

# Abstract

This contribution reports cross-verification results of non-CE3 described in JCTVC-G770 [1]. Cross-verification task has been successfully completed, and it is confirmed that the BD-rate results match those provided by proponent.

# Software analysis

The following modifications are confirmed through the analysis of software provided by Toshiba:

- PU 8x4 and 4x8 for bi-prediction is restricted when both motion vectors for L0/L1 are not zero to both luma and chroma:

  - This is performed in encoder only. So this modification is non-normative change.

  - In motion estimation, if the motion vector is all zero for horizontally, vertically, L0 and L1, 
the RD cost is set extreme high.

    - Then, the PU is not chosen because of largest cost.

  - The disabling of above PU 8x4 and 4x8 is checked in motion compensation function.

  - In personal opinion, I expect that it may avoid slight coding loss by modifying motion vector to zero. (It does not set high cost.) However, it will bring more modifications of implementation unfortunately.

# Visual quality

For visual quality, BQMall and PartyScene for P-only without SAO are checked. I felt that the proposal is the same with anchor for visual quality. Those still images are included in this contribution.

# Experiment

## Test condition and machine platform

Experiment was carried out based on HM software version 4.0 as follow CE3 mandate. And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 3.2GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output.

## Coding condition

High coding efficiency anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Low complexity anchor:

Random access configuration

Low delay configuration

“P only” (GPB off case)

Common test condition is specified in [2].

# Conclusion

It is confirmed that the results reported by proponent are accurate.

# References

[1] “Non-CE3: A restriction of motion vector for small block size”, JCTVC-G770, Nov. 2011

- [2] F. Bossen, “Common HM test conditions and software reference configurations,” JCTVC-F900, 6th. JCT-VC Meeting, Torino, IT, 14-22 July, 2011.

# Patent rights declaration(s)

SONY corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).