| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, Suisse, 21-30 November, 2011 | Document: JCTVC-G899 |
| --- | --- |

| Title: | Cross-check of CU-based Merge Candidate List Construction JCTVC-G416 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Simon Oudin Benjamin Bross Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | +49 30 31002 167 simon.oudin@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results of JCTVC-G416, a study on the construction of a CU-based Merge list. The technique has been studied, proposed code has been checked, compiled and the results reported by ETRI and Kyung Hee University can be confirmed.

# CU-based Merge Candidate List Construction

For the tested proposal JCTVC-G416 [1], one software package has been release and it has been integrated into HM4.0 [2]. One macro has been introduced to integrate this proposal:

- ETRI_KHU_CU_BASED_MERGE_CANDLIST

In HM4.0 [2], the creation of merge candidates list is proccessed for each partition of a CU partioning (PU). The proposal of ETRI and Kyung Hee University changes the process of merge candidate list construction in order to have only one list per CU. It means that all PUs of the same CU will have the same merge candidate list. This merge candidate list is based on the list created for the CU partioning named 2Nx2N where the PU has the same size than the CU as shown in Figure 1. The 2 PUs (PU0, PU1) are the 2 partitions of the same CU as shown in Figure 1. For this proposal, they are sharing the same merge candidate list composed of the neighbors: A0, A1, B2, B1 and B2 and also some extra merge candidates as the temporal merge candidate obtained from a position co-located in a previously coded picture or the combined and zero motion candidates.

When the macro ETRI_KHU_CU_BASED_MERGE_CANDLIST is active the software based on HM4.0 is using only the merge candidate list described above for each CU. When this macro is deactivated the software is working exactly as HM4.0 [2].

CU

Figure 1: The neighboring PU positions used for all the PUs in a CU.

While checking the code, nothing special was observed.

The main changes are located in the predInterSearch function for encoding process where the merge candidate list was created for each CU partitioning originally and now only for the first PU of the CU. For the decoding process of a CU, the main changes are visible in the function decodePUWise where it is now tested if a PU has already been decoded with a merge candidate which means that the CU based merge candidate list has already been created for this current CU.

# Simulation Environment and Results

## Software

The binaries were compiled on a 64 bit Ubuntu 10.10 Linux using the included make files and gcc 4.4.5. The coding conditions used for the simulations are specified in [3]. The HM4.0 default settings, both for configuration file and compile-time, are used in this test as anchor.

## Hardware

For all tests configurations one computer type has been used to run the simulations running a 64 bit Ubuntu 10.10 Linux operating system. It is equipped with Intel(R) Xeon(R) CPU X5570 @ 2.93GHz quadcore CPUs and 32 GB of memory.

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-G899.xls. The encoding and decoding times shown shouldn’t be considered for this cross-check. The results corresponding for cross-check are visible in Table 2-1.

It should be noted that the anchor results have been generated using a different type of computer and thus the runtimes are not reliable.

Table 2-1  cross-check results

# References

- H. Y. Kim, K. Y. Kim, S. M. Kim, G- H. Park, S.-C. Lim, J. Lee and J. S. Choi, “CU-based Merge Candidate List Construction”, JCTVC-G416, Geneva, November, 2011.

- CT-VC, “HEVC Test Model Software 4.0”, https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-4.0

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Turin, July, 2011.