| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G902 WG11 number: m22487 |
| --- | --- |

| Title: | CE4 Subtest 2.2.b: Cross-verification of JCTVC-G382 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mingyuan Yang | Tel: Email: | +44 60614080 yangmingyuan@huawei.com |
| Source: | Huawei Technology |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by Huawei Technology for RIM’ CE4 proposal (JCTVC-G382) on Subtest 2.2.b. The cross-check rate-distortion performance results match those provided by the proponent. The cross-checker uses windows 64bits server.

# Simulation Results

We have verified the results provided by RIM. The test conditions as specified in CE4 Subtest 2 JCTVC-F904 are followed. Tests were conducted on Random Access and Low Delay configurations for both high efficiency and low-complexity settings. And in order to meet the rate constraint, lambda is tuned respectively.

The results are shown below.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.6% | -5.1% | -4.7% | -2.1% | -6.9% | -6.8% |
| Class B | -1.0% | -4.0% | -3.4% | -1.6% | -5.1% | -5.1% |
| Class C | -0.8% | -3.3% | -3.2% | -1.2% | -4.2% | -4.1% |
| Class D | -0.5% | -2.9% | -3.0% | -0.9% | -3.8% | -4.1% |
| Class E |  |  |  |  |  |  |
| Overall | -0.7% | -3.8% | -3.6% | -1.5% | -5.0% | -5.0% |
|  | -0.7% | -3.8% | -3.5% | -1.2% | -5.0% | -4.9% |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1.8% | -5.9% | -5.5% | -2.4% | -7.6% | -8.3% |
| Class C | -1.8% | -5.1% | -4.8% | -2.3% | -7.0% | -7.2% |
| Class D | -1.2% | -3.7% | -3.9% | -1.8% | -5.9% | -6.2% |
| Class E | -0.3% | -1.2% | -0.5% | -0.8% | -3.2% | -2.9% |
| Overall | -1.4% | -4.3% | -4.0% | -2.0% | -6.2% | -6.5% |
|  | -1.4% | -4.2% | -3.9% | -1.9% | -6.1% | -6.2% |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

# Conclusions

The obtained BD-Rate results perfectly match the results provided by RIM. Summary of the results are shown in the table above and more detailed results can be found in the Excel sheet in the compressed file.

# Comments on source code

Source code was roughly reviewed and confirmed that it was consistent with the description in JCTVC- F382.