| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 7th Meeting: Geneva, CH, 21-30 November, 2011 | Document: JCTVC-G905 |
| --- | --- |

| Title: | Crosscheck report of ZTE’s joint coding of splitting flag and inter modes (JCTVC-G355) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xianglin Wang 5775 Morehouse Dr San Diego, CA 92121 USA | Tel: Email: | 1-858-651-5135 xianglin@qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document reports results on cross-checking the implementation and simulation results of proposal JCTVC-G355 from ZTE on joint coding of CU split flag and inter modes in CAVLC.

# Proposed method

According to this proposal, in jointly coding CU split flag and inter prediction modes, the initial codeword assignment in a slice is dependent on prediction modes of neighboring blocks. More specifically, it is dependent on prediction modes of the left block and the upper block neighboring to a current block. Four different classes or “coding types” are defined depending on neighboring block prediction modes and CU depth. For each class or “coding type”, a different initial codeword assignment may be signaled at slice header.

# Simulation results

Simulations were performed according to common test conditions[1], but only with the three low complexity configurations where CAVLC is enabled. Macro CODEWORD_LEN is defined as 31. Simulation results are as follows. For the three test cases, PSNR results match those provided in JCTVC-G355.

# Conclusion

This document confirms that cross-checking results match those provided in JCTVC-G355.

The software quality is fine. Related changes to source code are not minor.

# Reference

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-F900, Torino, July, 2011.

- W. Zhang, P.Wu, “Joint coding of splitting flag and inter modes in CAVLC”, JCTVC-G355, Geneva, Nov. 2011.